3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207741

# Electronic Meeting, May 09 – May 20, 2022

Agenda item:	9.23.2

Source:	Qualcomm Inc.

Title:	On NR SL relay RRM Requirement

Document for:	Discussion

# Introduction

In this contribution, we present our view on SL relay performance requirement scope.

# Discussion

## SL Relay Test Scope

RAN4 introduced two sets of requirements for R17 SL relay: discovery configuration interruption and measurement delay for relay selection/reselection. Since discovery configuration is identical to general RRC reconfiguration, we don’t need an additional test to verify it. We can use LTE ProSe A.7.5.12 test as a reference and design measurement delay test for R17 SL relay.

Proposal 1: Introduce one test for relay selection/reselection measurement delay requirement.

Note that discover resources are configured and granted by eNB (emulated by TE) in LTE ProSe test A.7.5.12. However, since remote UE is likely to operate in out of coverage scenario, considering mode 2 SL makes more sense. Therefore, we propose to test remote UE with mode 2 SL resource allocation mode and skip the discover resource configuration and allocation steps.

Proposal 2: Configuration mode 2 SL resource allocation for UE under test in relay selection/reselection test and skip the discover resource configuration and allocation steps in LTE test.

# Conclusions

Proposal 1: Introduce one test for relay selection/reselection measurement delay requirement.

Proposal 2: Configuration mode 2 SL resource allocation for UE under test in relay selection/reselection test and skip the discover resource configuration and allocation steps in LTE test.