3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207793

# Electronic Meeting, May 09 - May 20, 2022

Agenda item:	5.3.5.1

Source:	Apple

Title:	Discussion on incorrect PMI reporting

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#102-e and RAN#95-e introducing requirements for PMI to address false PMI reporting were discussed. RAN4 is tasked to discuss impact of false PMI reporting in interference conditions as captured in [1].

RAN4 is tasked to discuss and conclude on the following topics in Q2 to enable RAN#96 to make necessary decisions. - Existence and impact of the incorrect PMI reporting - Usefulness of a demod requirement with no impact to other WGs. · Discuss if it is needed to define a new UE capability, e.g., in relation to a UE reference receiver. The following point can be discussed in RAN4 if the workload allows. Otherwise, it can be discussed in RAN#96. - Work scope and the number of RAN4 meetings needed to develop a requirement

In this contribution we present our views on PMI reporting requirements in ICI.

# 2. Discussion

In RAN4#102-e in there was no consensus whether PMI reporting requirement sin ICI should be introduced:

- FFS to define PMI reporting requirement with inter-cell interference in Rel-17 FeMIMO WI

- FFS on where to handle the PMI reporting requirement with inter-cell interference

  - Option 1: Rel-17 TEI

  - Option 2: Rel-18 timeframe

  - Option 3: Rel-17 FeMIMO WI

The main motivation for introducing PMI reporting requirements in ICI was motivated by false PMI issue observed in OTA testing. First, RAN4 should further evaluate if there is an issue with PMI reporting before deciding to define requirements for PMI reporting for inter-cell scenario.

In [2] one option to further evaluate the existence of false PMI was captured:

Issue 5-1-2: PMI reporting with inter-cell interference evaluation assumption

- Option 1: For further evaluation of PMI reporting in ICI use the following simulation assumptions as baseline

  - Antenna config: 8x2 XP High

  - Prop. Channel model: TDLA30-5; ensure that channel from target and interference cell are statistically independent and have different beam direction (to ensure PMI are different)

  - NZP CSI-RS for interference:

    - Overlapping with serving cell

    - Non-overlapping with serving cell

  - CSI-IM for interference: non overlapping with CSI-IM for serving cell

  - Loading for interference cell: PDSCH transmission is enabled in all slots for interference cell

  - Evaluate performance based on TP ratio with and with ICI for (1) overlapping NZP CSI-RS (2) non-overlapping NZP CSI-RS

- Other options are not precluded

Using the simulation parameters above along with the simulation assumptions for 8TX PMI reporting requirements, we evaluate the performance of PMI reporting in ICI with overlapping and non-overlapping NZP CSI-RS. Detailed simulation assumptions are provided in Table 1.

Table 1: Detailed Simulation Assumptions for PMI reporting in ICI study

| Parameter |  |  | Unit | Serving cell | Interferer cell |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 | 10 |
| Duplex Mode |  |  |  | FDD | FDD |
| Subcarrier spacing |  |  | kHz | 15 | 15 |
| Propagation channel |  |  |  | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  |  | High XP 8 x 2 (N1,N2) = (4,1) | High XP 8 x 2 (N1,N2) = (4,1) |
| SINR |  |  | dB | [-6: 2 :10] | - |
| INR |  |  | dB | - | 10.04 |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  |  | 8 | 8 |
|  | CDM Type |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 8, (4,6) | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 5 | Overlapping: 5 Non-overlapping: 7 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  |  | Aperiodic | Aperiodic |
|  | CSI-IM RE pattern |  |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) | (6,9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | Same as serving cell |
| ReportConfigType |  |  |  | Aperiodic | Not configured |
| CQI-table |  |  |  | Table 1 | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | Not configured |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband |
| Sub-band Size |  |  | RB | 8 |  |
| Csi-ReportingBand |  |  |  | 1111111 | Not configured |
| CSI-Report periodicity and offset |  |  | slot | NA | Not configured |
| aperiodicTriggeringOffset |  |  |  | 5 | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) | (4,1) |
|  |  | CodebookSubsetRestriction |  | 0x FFFF | Not configured |
|  |  | RI Restriction |  | 00000001 | Not configured |
| Physical channel for CSI report |  |  |  | PUSCH | Not configured |
| CQI/RI/PMI delay |  |  | ms | 8 | Not configured |
| Maximum number of HARQ transmission |  |  |  | 1 | Not configured |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-2 |  |

We evaluate the performance for 2 modes:

- Follow CQI

- Fixed MCS (MCS 13 64QAM Table)

Performance with Follow CQI

We evaluate the performance by comparing the TP ratio between with interference and without interference with overlapping and non-overlapping NZP CSI-RS in follow CQI mode.

Figure 1: Performance with PMI reporting in ICI with follow CQI

We observe that the performance is very comparable between overlapping and non-overlapping NZP-CSI-RS with follow CQI.

Observation #1: With follow CQI the performance of overlapping and non-overlapping NZP CSI-RS is very similar in ICI conditions.

Performance with fixed MCS

We evaluate the performance by comparing the TP ratio between with interference and without interference with overlapping and non-overlapping NZP CSI-RS in fixed MCS mode. MCS 13 from 64QAM Table is used.

Figure 2: Performance with PMI reporting in ICI with fixed MCS

We observe that the performance is very comparable between overlapping and non-overlapping NZP-CSI-RS with fixed MCS as well.

Observation #2: With fixed MCS the performance of overlapping and non-overlapping NZP CSI-RS is very similar in ICI conditions.

Based on the performance above, we see that there is no severe performance impact with overlapping NZP CSI-RS for PMI reporting in ICI conditions.

Observation #3: No severe performance impact is observed with overlapping NZP CSI-RS for PMI reporting in ICI conditions.

Hence, we don’t see the necessity to introduce PMI reporting requirements in ICI conditions.

Proposal #1: Do not introduce PMI reporting in ICI conditions as no False PMI reporting is observed with overlapping NZP CSI-RS.

# 3. Conclusion

In this paper, we provide our views on PMI reporting requirements in ICI. Our observations and proposals are captured below:

Observation #1: With follow CQI the performance of overlapping and non-overlapping NZP CSI-RS is very similar in ICI conditions.

Observation #2: With fixed MCS the performance of overlapping and non-overlapping NZP CSI-RS is very similar in ICI conditions.

Observation #3: No severe performance impact is observed with overlapping NZP CSI-RS for PMI reporting in ICI conditions.

Proposal #1: Do not introduce PMI reporting in ICI conditions as no False PMI reporting is observed with overlapping NZP CSI-RS.

# Reference

- RP-220948, “Moderator's summary for discussion [95e-28-R17-feMIMO]”, Samsung

- R4-2207209, “WF on CSI requirement for Rel-17 FeMIMO”, Samsung.