3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207794

# Electronic Meeting, May 09 - May 20, 2022

Agenda item:	9.6.4.1

Source:	Apple Inc.

Title:	Simulation Results for PDSCH demod requirements with 1KQAM

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#102-e PDSCH demod requirements with 1024QAM in NR DL were discussed and way forward [1] was agreed.

- Interested companies are encouraged to provide the PDSCH simulation results according to Table 1 in the next meeting to decide the SNR test points. - Add 1dB additional margin on top of the average of companies’ impairment results for PDSCH 1024QAM.

In this contribution we provide simulation results for PDSCH demod requirements with 1024QAM.

# 2. Discussion

The agreed simulation assumptions for PDSCH demod requirements are captured in Table 1.

Table 1: Simulation parameters for PDSCH demod with 1024QAM

| Parameters | FDD | TDD |
| --- | --- | --- |
| SCS/CBW | 15kHz and 10MHz | 30kHz and 40MHz |
| TDD UL/DL configuration | N/A | 7D1S2U, with S=6:4:4 Schedule PDSCH in the special slots |
| MCS (MCS index table 4) | MCS23 |  |
| Rank | 1 |  |
| Antenna configuration | 2x2 with ULA Low 2x4 with ULA Low |  |
| Propagation channel | TDLD30-5 |  |
| PDSCH configuration | Type A mapping, Start symbol 2, Duration 12 |  |
| PDSCH DMRS configuration | Type 1, Single symbol, additional DMRS: pos1 |  |
| Tx EVM assumed for simulation | 2.5% |  |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |  |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 10 ms |  |
| Number of HARQ processes | 4 for FDD 15kHz, 8 for TDD 30kHz |  |
| Transform precoding | CP-OFDM |  |
| Allocated RBs | Full BWP |  |
| PRB bundling | 2 |  |
| Precoding model | Random Precoding, per slot and per RB bundling size granularity (codebook configuration: Single panel Type 1) |  |
| Receiver type | MMSE-IRC |  |
| Test metric | 70% of maximum throughput |  |

For the simulation assumptions in Table 1 we provide alignment and impairment results for PDSCH demod.

Table 2: Demod performance results with 1024QAM

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS | Propagation condition | Correlation matrix and antenna configuration | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| FDD | 10MHz/15KHz | 23 | TDLD30-5 | 2x2, ULA Low | 26.7 | 29.1 |
| FDD | 10MHz/15KHz | 23 | TDLD30-5 | 2x4, ULA Low | 23.3 | 25.6 |
| TDD | 40MHz/30KHz | 23 | TDLD30-5 | 2x2, ULA Low | 26.5 | 28.8 |
| TDD | 40MHz/30KHz | 23 | TDLD30-5 | 2x2, ULA Low | 23.0 | 25.4 |

# 3. Conclusion

In this paper, we provide simulation results for PDSCH demod requirements with 1024QAM.

# Reference

- R4-2203044, “Way forward for NR DL1024QAM demodulation and CQI reporting requirements”, Ericsson.