3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207813

# Electronic Meeting, May 09 - May 20, 2022

Agenda item:	9.19.5.3.2

Source:	Apple Inc.

Title:	On PMI Reporting Requirements for Reduced Capability Devices in NR

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#102-e RedCap UE demodulation and CSI reporting requirements was discussed and way forward [1] was agreed. In this contribution we present initial simulation results and test metrics for PMI and RI reporting requirements for RedCap UE.

# 2. Discussion

## PMI Reporting Requirements

In [1] the scope and simulation assumptions for PMI reporting requirements for RedCap UE were agreed.

Whether to define PMI reporting requirements for RedCap UEs - Define PMI reporting requirements with 4Tx configuration for FR1 for both 1Rx and 2Rx UEs - Define PMI reporting requirements with 2Tx configuration for FR2 for 2Rx UE.

For the agreed simulation assumptions, we provide initial simulation results for PMI reporting requirements. Since FR2 requirements for RedCap UE will be reused from existing Rel-15 requirements, we only provide results for FR1.

FDD FR1

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx | Table 6.3.2.1.1-1 Test 1 | Single PMI, Type I, rank 1 | 10 / 15 | 4 | 4 x 1 ULA high | New |
| 2Rx | Table 6.3.2.1.1-1 Test 1 | Single PMI, Type I, rank 1 | 10 / 15 | 4 | 4 x 2 XP high | Reuse |

TDD FR1

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx | Table 6.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 20 / 30 | 4 | 4 x 1 ULA high | New |
| 2Rx | Table 6.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 20 / 30 | 4 | 4 x 2 XP high | New |

Based on the results above, we think it would be feasible to use the same TP gain metric for PMI reporting requirements for RedCap UE as existing requirements with 4x2 in Rel-15.

Observation #1: The same TP gain metric can be re-used for RedCap UE requirements for 2RX and 1RX for FDD and TDD.

Proposal #1: Reuse the TP gain metric of 1.3 for RedCap requirements with 2RX and 1RX.

## RI Reporting Requirements

The scope of RI reporting requirements for RedCap UE are not agreed yet.

Whether to define RIreporting test for RedCap 2Rx UE - Option 1: Define RI reporting tests for RedCap 2Rx UE. - Option 2: Not to define RI reporting tests for RedCap 2Rx UE.

RI reporting is not needed for RedCap UE to reduce the UE implementation complexity and testing burden. Moreover, RI reporting can only be for 2RX and 2 layers which is subject to UE capability. Hence, we don’t see the necessity to introduce RI reporting requirements for 2RX for RedCap.

Proposal #2: Do not define RI reporting requirements for RedCap UE.

# 3. Conclusion

In this paper, we provide initial simulation results and test metrics for PMI and RI reporting requirements for RedCap UE. Our observations and proposals are captured below:

Observation #1: The same TP gain metric can be re-used for RedCap UE requirements for 2RX and 1RX for FDD and TDD.

Proposal #1: Reuse the TP gain metric of 1.3 for RedCap requirements with 2RX and 1RX.

Proposal #2: Do not define RI reporting requirements for RedCap UE.

# Reference

- R4-2207206, “WF on RedCap UE demodulation and CQI reporting requirements”, Ericsson.