3GPP TSG-RAN WG4 Meeting #103-e	R4- 2207820

Electronic Meeting, May. 09 – May. 20, 2022

Agenda Item:	9.8.2.5

Source:	Apple

Title:	Discussion on the measurement requirement in FR2 HST RRM

Document for:	Discussion

# 1	Introduction

In Rel-17 NR support for high-speed train scenario in FR2 [1], potential RRM enhancement is proposed:

  - Study and identify RRM requirements impact and possible enhancement for

    - Idle/inactive mode cell reselection requirements enhancement

    - Connected mode requirements

      - Handover delay requirement

      - Measurement requirements including both L1 and SSB based L3 measurement

      - Beam management requirements including beam failure detection, candidate beam detection performance requirements

      - Other requirements if identified

[2,3] captures RRM related discussion in RAN4 102-e.

In this paper, we discuss the remaining issue of measurement requirement in HST FR2.

# 2	Discussion

In [2], the remaining open issue is the scaling factor for time period for PSS/SSS detection and mreasurement period for intra-frequency measurement.

GtW Agreement: - Agreements - Scaling factors (Mpss/sss_synch_w/o_gaps and Mmeas_period_w/o_gaps) equal to 6 for Set 1 and [18] for Set 2

Current intra-frequency PSS/SSS detection define Mpss/sss_sync_w/o_gaps is 24 in FR2 for UE power class 2,3,4. For measurement period requirement Mmeas_period_w/o_gaps is also 24 in FR2 for UE power class 2,3,4. Those values are significantly larger than FR1 case due to UE Rx beam sweeping. In current FR2, N=8, and 24 samples can be written as 3xN. Following number of Rx beam discussion, the intra-frequency PSS/SSS detection and measurement period can be enhancement for scenario A and scenario B separately.

Proposal 1: Define Mpss/sss_sync_w/o_gaps = 18 for set 2, and Mmeas_preriod_w/o_gaps = 18 for set 2.

# 3	Summary

In this paper, we provide our view on mobility aspect of RRM enhancement for HST FR2.

Proposal 1: Define Mpss/sss_sync_w/o_gaps = 18 for set 2, and Mmeas_preriod_w/o_gaps = 18 for set 2.

# References

- RP-202118, New WID on NR support for high-speed train scenario in FR2, Samsung, Nokia, Sep 2020

- R4-2206848, Way forward for FR2 HST RRM (part 1), Nokia, Feb 2022

- R4-2202767, Way forward for uplink timing for FR2 HST, Samsung, Jan 2022