3GPP TSG-RAN WG4 Meeting # 103-e						R4-2207936

Electronic Meeting, May 09 – 20, 2022

Source:		Verizon, Samsung

Title:			TP for TR 38.841: CA_n5-n77

Agenda item:		8.24.2

Document for:		Approval

# 1. Introduction

This is a text proposal to include MSD due to cross band isolation for the approved PC2 CA_n5-n77 combo in [1].

# 2. Reference

- 3GPP TR 38.841

- TR 38.716-02-00

# 3. Text Proposal

<Start of Text Proposal>

## 5.6 	CA_n5-n77

### 5.6.1	Configurations

Table 5.6.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n77A | n778,9 CA_n5A-n77A8 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n77C | n778,9 CA_n5A-n77A8 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.6.2	Maximum output power

| Uplink CA configuration | Power class 2 cases for CA_n5A-n77A | CA power class | Carrier n5 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n5A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

### 5.6.3	REFSENS requirements

According to the PC3 CA_n5A-n77A study, the 4th and 5th IMD products from dual uplink of band 5 and n77 may fall into band 5 Rx frequency range. Thus additional MSD should be considered to mitigate the impact of the interference for PC2 CA_n5A-n77A combination.

#### 5.6.3.1	Power class 2 Case A

The MSD due to receiver harmonic mixing for CA_5A-n77 was studied in Rel-16 [2]. And, the same MSD values are reused in table 5.6.3.1-1 for the configuration of CA_n5A-n77A.

Table 5.6.3.1-1:  MSD due to receiver harmonic mixing for CA in NR FR1

|  | MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band |  | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 100 MHz |
|  |  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n77 |  | n5 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |

The additional MSD due to intermodulation for PC2 CA_n5A-n77A are defined in table 5.6.3.1-2.

Table 5.6.3.1-2: MSD test points for PCell due to dual uplink operation for PC2 NR CA in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD for PC2 (dB) | Duplex mode | Source of IMD |
| CA_n5A-n77A | n5 | 844 | 5 | 25 | 889 | 18.6 | FDD | IMD44 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 5.6.3.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B CA_n5A-n77A are same as the Case A defined in table 5.6.3.1-1.

The additional MSD due to receiver harmonic mixing for Case B are defined in table 5.6.3.2-1.

Table 5.6.3.2-1:  MSD due to receiver harmonic mixing for CA in NR FR1

|  | MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band |  | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 100 MHz |
|  |  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n77 |  | n5 | 8.1 | 6.0 | 4.8 | 4.3 |  |  |  |  |  |  |  |

#### 5.6.3.3	Single uplink power Class 1.5

The additional MSD due to receiver harmonic mixing for Case B are defined in table 5.5.3.3-1.

Table 5.5.3.3-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for NR CA  in NR FR1

| NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n5 | 10.7 | 8.4 | 7.0 | 6.4 |  |  |  |  |  |  |  |  |

#### 5.6.3.3	OOB blocking exception requirements

Since band n5 is a low band and n77 is a wide band, the OOBB exception is needed.

Table 5.6.3.3-1: NR DC band combination with exceptions allowed

CA band combination

CA_n5-n77

### 5.6.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n5A-n77A requirements are applied for PC2 CA_n5A-n77A.

<End of Text Proposal>