3GPP TSG-RAN4 Meeting #103-e	R4-2207980

# electronic meeting,  9th  - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR | 0001 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.106: Conducted requirements corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_repeaters-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The TX IM requirement refers to the operating band instead of the passband, and so does not consider multi-passband, which is in the scope of the specification. Also the interfering signal definition is based on passband bandwidth, which is not aligned with BS TX IM. Further details in R4-2207979. The TDD switching requirement description refers to transmission bandwidth configuration, which is not a property of a repeater and not declared. The TDD switching bandwidth is propsoed to be set to passband bandwidth instead. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The power reference to TX IM is updated to align to the BS definition. The interfering signal bandwidth is set to a nominal channel bandwidth. The notes on signal positions are updated to align to the BS TX IM requirement. TDD switching requirement updated to be based on passband bandwidth. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TX IM requirmeent that does not allow for multiple passbands and that does not align to the BS TX IM requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 6.8.2, 6.10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Antenna connector: connector at the conducted interface of the repeater type 1-C

Beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array

Beam centre direction: direction equal to the geometric centre of the half-power contour of the beam

Beam direction pair: data set consisting of the beam centre direction and the related beam peak direction

Beam peak direction: direction where the maximum EIRP is found

Beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

Equivalent isotropic radiated power: equivalent power radiated from an isotropic directivity device producing the same field intensity at a point of observation as the field intensity radiated in the direction of the same point of observation by the discussed device

Fractional bandwidth: fractional bandwidth FBW is defined as FBW=200∙FFBWhigh-FFBWlowFFBWhigh+FFBWlow%

Maximum output power: mean power level measured within the passband at the antenna connector, during the transmitter ON period in a specified reference condition

Maximum TRP output power: mean power level measured per RIB during the transmitter ON period in a specified reference condition and corresponding to the declared rated carrier TRP output power (Prated,TRP)

Measurement bandwidth: RF bandwidth in which an emission level is specified

Multi-band repeater: Repeater Type 1-C whose antenna connector is associated with a transmitter and/or receiver that is characterized by the ability to process two or more passband(s) in common active RF components simultaneously, where at least one passband is configured at a different operating band than the other passband(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band

Operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements

Passband: [The frequency range in which the repeater operates in with operational configuration, this frequency range can correspond to one or several consecutive nominal channels, if they are not consecutive each subset of channels shall be considered as an individual passband, a repeater can have one or several passbands, all channels within the passband(s) shall belong to a single operator or collaborating operators.]

Radiated interface boundary: operating band specific radiated requirements reference where the radiated requirements apply

Radio Bandwidth: frequency difference between the upper edge of the highest used passband and the lower edge of the lowest used passband

Rated beam EIRP: For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the NR repeater is declared to radiate at the associated beam peak direction during the transmitter ON period

Rated output power: mean power level associated with a passband the manufacturer has declared to be available at the antenna connector

rated total output power: mean power level associated with a particular operating band the manufacturer has declared to be available at the indicated interface, during the transmitter ON period in a specified reference condition

Rated TRP output power: mean power level declared by the manufacturer, that the manufacturer has declared to be available at the RIB

Reference beam direction pair: Beam direction pair in the reference direction declared by the manufacturer.

Repeater type 1-C:	NR repeater operating at FR1 with a requirement set consisting only of conducted requirements defined at individual antenna connector.

Repeater type 2-O: NR repeater operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB

Requirement set: one of the NR requirements set as defined for NR repeater

Sub-band: A sub-band of an operating band contains a part of the uplink and downlink frequency range of the operating band.

Superseding-band: A superseding-band of an operating band includes the whole of the uplink and downlink frequency range of the operating band.

Total radiated power: is the total power radiated by the antenna

NOTE:	The total radiated power is the power radiating in all direction for two orthogonal polarizations. Total radiated power is defined in both the near-field region and the far-field region

Transmission bandwidth: RF Bandwidth of an instantaneous transmission from an NR repeater, measured in resource block units

Transmitter OFF state: Time period during which the repeater downlink or uplink is not allowed to transmit in the corresponding direction

Transmitter ON state: Time period during which the repeater downlink or uplink is transmitting in the corresponding direction

Transmitter transient period: Time period during which the transmitter is switching from the OFF period to the ON period or vice versa

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

FFBWhigh	Highest supported frequency within supported passband, for which fractional bandwidth support was declared

FFBWlow	Lowest supported frequency within supported passband, for which fractional bandwidth support was declared

Prated,out	Maximum rated output power conducted

Prated,out_AC	Maximum rated output power at the antenna connector

Prated,t,out_AC	Rated total output power at the antenna connector

Prated,out,EIRP	Maximum rated output power EIRP

Prated,out,TRP	Maximum rated output power TRP

Prated,in	Input power intended to produce the maximum rated output power

Prated,in_AC	Input power intended to produce the maximum rated output power (Prated,out) at the antenna connector

Pmax,out	Maximum carrier output power measured per antenna connector

Pmax,out_AC	Maximum output power measured per antenna connector at the antenna connector

Pmax,out,,EIRP	The maximum EIRP measured at the RIB(s), and corresponding to the declared rated TRP output power (Prated,c,TRP)

Pmax,out,TRP	Maximum TRP output power measured at the RIB(s), and corresponding to the declared rated TRP output power (Prated,c,TRP)

### 6.8.2	Minimum requirements for repeater type 1-C

#### 6.8.2.1	Minimum requirements

The output intermodulation level is the power of the intermodulation products when an interfering signal is injected into the output port. The wanted signal passbands shall be the maximum bandwidth(s) supported by the repeater.

For repeater type 1-C, the wanted signal and interfering signal centre frequency is specified in table 6.8.2.1-1, where interfering signal level is Rated total output power (Prated,t,out) at antenna connector in the operating band– 30 dB.

The unwanted emission with output intermodulation applied shall not exceed the corresponding uplink and downlink unwanted emission limits in clause 6.5 in the presence of an interfering signal according to table 6.8.2.1-1. The measurement may be limited to frequencies on which third and fifth order intermodulation products appear, considering the width of these products.

Table 6.8.2.1-1: Interfering and wanted signals for the output intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal type | NR signal, filling all supported passbands in the operating band and with sufficient carriers to fill each passband. Minimum defined SCS for the operating band |
| Interfering signal type | 10MHz NR signal, with the minimum SCS defined in the operating band |
| Interfering signal level | Rated total output power (Prated,t,out) in the operating band– 30 dB |
| Interfering signal centre frequency offset from the lower/upper edge of the wanted signal or edge of sub-block inside a sub-block gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the repeater are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, TS 38.115-1 [7] provides further guidance regarding appropriate test requirements. NOTE 2: In Japan, NOTE 1 is not applied in Band n77, n78, n79. |  |

### 6.10.1	Transmitter OFF power

#### 6.10.1.1	General

Transmit OFF power requirements apply only to TDD operation of the repeater. The requirement applies to both downlink and uplink of the repeater.

Transmitter OFF power is defined as the mean power measured over 70/N us filtered with a square filter of bandwidth equal to the passband bandwidth of the repeater (BWpassband) centred on the assigned channel frequency during the transmitter OFF period. N = SCS/15, where SCS is Sub Carrier Spacing in kHz of the input signal.

For multi-band connectors and for single band connectors supporting transmission in multiple operating bands, the requirement is only applicable during the transmitter OFF period in all supported operating bands.