3GPP TSG-RAN WG4 Meeting #103-e 		R4-2208011

Electronic meeting, 9th – 20th May, 2022

Source:	Ericsson

Title:	Simulation results for PUSCH demodulation requirements for NR coverage enhancement

Agenda item:	9.17.2.1

Document for:	Information

# Introduction

In the last RAN4 meeting, companies have discussions on the NR coverage enhancement demodulation requirements. In this contribution, simulation results for PUSCH demodulation requirements based on the latest WF [1] are delivered for further discussion.

# Discussion

  - PUSCH with JCE

Table 2.1-1 Simulation assumptions for PUSCH with JCE

| Parameter |  | Value |
| --- | --- | --- |
| Test metric |  | SNR@70% throughput and SNR@BLER=1% |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern |  | 15 kHz SCS: FDD |
| Antena Configuration |  | 1Tx 2/4/8Rx |
| Propagation conditions and correlation matrix |  | TDLA30_10 Low |
| Code rate |  | MCS4 and MCS2 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | [0, 2, 3, 1] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Pos0 and pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH repetition type | back-to-back PUSCH, Type A |
|  | PUSCH repetition factor | 2/4/8/16 |
|  | AvailableSlotCounting | Enabled |
| JCE configuration | PUSCH-TimeDomainWindowLength (i.e. cTDW) | 2/4/8/16 |
|  | Actural window length (aTDW) | 2/4/8/16 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disable |
| Code block group based PUSCH transmission |  | Disabled |
| Phase error model |  | NA |
| Power error model |  | disabled |

Table 2.1-2 Simulation results for PUSCH with JCE, 15kHz SCS 20MHz BW, precoding disabled, MCS2

| Tx | Rx | channel | MCS | repetition | DM-RS | JCE | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 70% Thp | 1% BLER |
| 1 | 2 | TDLA30-10 | 2 | 2 | 1+0 | enable | -7.7 | -2.5 |
|  |  |  |  |  | 1+1 | enable | -7.8 | -2.7 |
|  |  |  |  | 4 | 1+0 | enable | -10.6 | -5.6 |
|  |  |  |  |  | 1+1 | enable | -10.6 | -5.7 |
|  |  |  |  | 8 | 1+0 | enable | -13.4 | -8.2 |
|  |  |  |  |  | 1+1 | enable | -13.5 | -8.6 |
|  |  |  |  | 16 | 1+0 | enable | -15.9 | -10.5 |
|  |  |  |  |  | 1+1 | enable | -15.9 | -10.7 |
| 1 | 4 | TDLA30-10 | 2 | 2 | 1+0 | enable | -11.7 | -7.4 |
|  |  |  |  |  | 1+1 | enable | -11.8 | -7.7 |
|  |  |  |  | 4 | 1+0 | enable | -14.5 | -10.2 |
|  |  |  |  |  | 1+1 | enable | -14.6 | -10.5 |
|  |  |  |  | 8 | 1+0 | enable | -16.8 | -12.1 |
|  |  |  |  |  | 1+1 | enable | -17 | -12.7 |
|  |  |  |  | 16 | 1+0 | enable | -18.5 | -13.7 |
|  |  |  |  |  | 1+1 | enable | -18.6 | -13.7 |
| 1 | 8 | TDLA30-10 | 2 | 2 | 1+0 | enable | -14.1 | -11.5 |
|  |  |  |  |  | 1+1 | enable | -14.4 | -11.7 |
|  |  |  |  | 4 | 1+0 | enable | -16.5 | -13.6 |
|  |  |  |  |  | 1+1 | enable | -16.8 | -14 |
|  |  |  |  | 8 | 1+0 | enable | -18.2 | -14.9 |
|  |  |  |  |  | 1+1 | enable | -18.5 | -15.6 |
|  |  |  |  | 16 | 1+0 | enable | -19.5 | -16.1 |
|  |  |  |  |  | 1+1 | enable | -19.8 | -16.5 |

Table 2.1-3 Simulation results for PUSCH with JCE, 15kHz SCS 20MHz BW, precoding disabled, MCS4

| Tx | Rx | channel | MCS | repetition | DM-RS | JCE | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 70% Thp | 1% BLER |
| 1 | 2 | TDLA30-10 | 4 | 2 | 1+0 | enable | -5.4 | -0.1 |
|  |  |  |  |  | 1+1 | enable | -5.5 | -0.3 |
|  |  |  |  | 4 | 1+0 | enable | -8.4 | -3.2 |
|  |  |  |  |  | 1+1 | enable | -8.5 | -3.3 |
|  |  |  |  | 8 | 1+0 | enable | -11.3 | -6 |
|  |  |  |  |  | 1+1 | enable | -11.4 | -6 |
|  |  |  |  | 16 | 1+0 | enable | -14 | -8.5 |
|  |  |  |  |  | 1+1 | enable | -14 | -8.8 |
| 1 | 4 | TDLA30-10 | 4 | 2 | 1+0 | enable | -9.5 | -5.1 |
|  |  |  |  |  | 1+1 | enable | -9.6 | -5.5 |
|  |  |  |  | 4 | 1+0 | enable | -12.5 | -8.7 |
|  |  |  |  |  | 1+1 | enable | -12.5 | -9 |
|  |  |  |  | 8 | 1+0 | enable | -15.1 | -10.7 |
|  |  |  |  |  | 1+1 | enable | -15.2 | -11.1 |
|  |  |  |  | 16 | 1+0 | enable | -17.2 | -12.9 |
|  |  |  |  |  | 1+1 | enable | -17.4 | -13.3 |
| 1 | 8 | TDLA30-10 | 2 | 2 | 1+0 | enable | -12.2 | -9.5 |
|  |  |  |  |  | 1+1 | enable | -12.4 | -9.7 |
|  |  |  |  | 4 | 1+0 | enable | -14.9 | -12.2 |
|  |  |  |  |  | 1+1 | enable | -15.1 | -12.3 |
|  |  |  |  | 8 | 1+0 | enable | -17.1 | -14.3 |
|  |  |  |  |  | 1+1 | enable | -17.3 | -14.4 |
|  |  |  |  | 16 | 1+0 | enable | -18.6 | -15.7 |
|  |  |  |  |  | 1+1 | enable | -19 | -16.1 |

  - PUSCH with TBoMS

Table 2.2-1 Simulation assumptions for FR1 PUSCH TBoMS

| Parameter |  | Value |
| --- | --- | --- |
| Test metric |  | SNR@70% throughput, SNR@BLER=2% |
| Transform precoding |  | Disabled and enabled |
| Default TDD UL-DL pattern |  | 15 kHz SCS FDD |
| Antenna Configuration |  | 1Tx2RX/4Rx and 8Rx |
| Propagation conditions and correlation matrix |  | TDLB100-400 Low |
| MCS |  | MCS4 and MCS2 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | Slot assignment | 2/4 slots |
|  | Repetition | Disabled |
| Frequency domain resource assignment | RB assignment | 15 kHz SCS: 5 PRBs |
|  | Frequency hopping | Disabled |

Table 2.2-2 Simulation results for PUSCH with TBoMS, 15kHz SCS 20MHz BW

| Tx | Rx | channel | MCS | repetition | DM-RS | precoding | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 70% Thp | 1% iBLER |
| 1 | 2 | TDLB100-400 | 2 | 2 | 1+1 | disable | -3.6 | 2 |
|  |  |  |  |  | 1+1 | enable | -3.6 | 2 |
|  |  |  |  | 4 | 1+1 | disable | -4.3 | 0 |
|  |  |  |  |  | 1+1 | enable | -4.3 | 0 |
|  |  |  | 4 | 2 | 1+1 | disable | -1.5 | 4 |
|  |  |  |  |  | 1+1 | enable | -1.5 | 4.2 |
|  |  |  |  | 4 | 1+1 | disable | -2.2 | 2 |
|  |  |  |  |  | 1+1 | enable | -2.2 | 2.2 |
| 1 | 4 | TDLB100-400 | 2 | 2 | 1+1 | disable | -7.2 | -3 |
|  |  |  |  |  | 1+1 | enable | -7.2 | -3.1 |
|  |  |  |  | 4 | 1+1 | disable | -7.6 | -4.6 |
|  |  |  |  |  | 1+1 | enable | -7.6 | -4.6 |
|  |  |  | 4 | 2 | 1+1 | disable | -5.2 | -1 |
|  |  |  |  |  | 1+1 | enable | -5.2 | -1 |
|  |  |  |  | 4 | 1+1 | disable | -5.6 | -2.6 |
|  |  |  |  |  | 1+1 | enable | -5.6 | -2.6 |
| 1 | 8 | TDLB100-400 | 2 | 2 | 1+1 | disable | -10.2 | -7.1 |
|  |  |  |  |  | 1+1 | enable | -10.2 | -7.2 |
|  |  |  |  | 4 | 1+1 | disable | -10.6 | -8.3 |
|  |  |  |  |  | 1+1 | enable | -10.6 | -8.3 |
|  |  |  | 4 | 2 | 1+1 | disable | -8.4 | -5.4 |
|  |  |  |  |  | 1+1 | enable | -8.4 | -5.5 |
|  |  |  |  | 4 | 1+1 | disable | -8.6 | -6.5 |
|  |  |  |  |  | 1+1 | enable | -8.6 | -6.5 |

# Conclusions

The simulation results for PUSCH with JCE and TBoMS based on the WF are delivered.

# References

[1]	R4-2207210, WF for PUSCH coverage enhancement, China Telecom