3GPP TSG-RAN WG4 Meeting # 103-e 	                                            R4-2208067

Electronic Meeting, May 09 – May 20

Agenda Item:	9.10.2.1.

Source:	Intel Corporation

Title:	Discussion on NR Pre-configured MG test cases configuration and list

Document for:		Discussion

# Introduction

In the last RAN4 meeting, the RRM core requirements related to the pre-configured MG (Pre-MG) were completed [1]. Accordingly, RAN4 is required to define the necessary test cases to verify the RRM requirements in TS38.133 [2].

In this paper we firstly provide the considerations on the basic test configurations for Pre-MG based measurement. And the list of the test cases to verify the corresponding core requirements for Pre-MG is proposed.

# Discussion

- Issue 1. NR standalone only

In Rel17, the pre-configured MG will be NOT used in MR-DC scenario but only NR SA. That is currently it is enough to define the test cases for NR standalone scenarios.

Proposal 1: It is necessary to define the test cases for NR standalone scenario only.

- Issue 2: Which types of measurement reference signal measured by Pre-MG?

In Rel17 the measurements with pre-MG can be used for not only SSB but also other reference signal (e.g. CSI-RS, PRS). However, since measurements on the CSI-RS and PRS needs the always being activated Pre-MG, it is not feasible to verify all new functionalities of Pre-MG (e.g. the activation/deactivation transition could not be tested) in a single test case. Thus we proposed that:

Proposal 2: Test cases for SSB measurement with Pre-MG shall be defined at least.

- Issue 3: Intra-frequency and inter-frequency measurement

As RAN4 defined the measurement delay requirements for Pre-MG based measurements without the status changed, the separated test cases for intra-frequency and inter-frequency measurement are needed. However, regarding to the inter-frequency measurement with Pre-MG which shall be always activated is completely same as that with the legacy measurement gap from UE behavior perspective, we can focus on the intra-frequency with gap firstly in order to save testing efforts.

Proposal 3: Test cases for the intra-frequency with Pre-MG gap can be defined with high priority.

- Issue 4: Pre-MG activation mechanism

In principle, both the UE autonomous pre-MG activation and network signalling Pre-MG activation mechanism shall be tested.

On the other hand, in comparison with single carrier cases, there is difference from UE behaviour perspective when the pre-MG activation mechanism is based on the network signalling. Therefore, we can propose that:

Proposal 4-1: When the pre-MG activation is based on the network signaling, the test cases under both single carrier and CA test cases shall be defined

Proposal 4-2: When the pre-MG activation is based on UE autonomous rules, the test cases under single carrier test cases needed only.

- Issue 5: For UE autonomous pre-MG activation, whether all trigger events shall be tested?

In Rel17, multiple trigger events below were agreed for pre-MG activation mechanism. However, from the requirements themselves, we believe only BWP-switching case is needed.

And we can also focus on the trigger event of BWP switching which is more important using cases.

Proposal 5: BWP switching trigger event can be tested firstly.

- Issue 6: DRX

As in WID, the measurement gap enhancement is only considered in RRC_CONNECT, we need not any test cases for DRX.

Proposal 6: Non-DRX cases will be tested only in Rel17.

Generally, the following test cases for NR Pre-MG are necessary:

- Activation delay test case for Pre-configured measurement gap

- Reporting delay test cases for NR standalone cells

  - Intra-frequency

In summary, the following test cases for Pre-MG core requirement were proposed.

Proposal 7: The following test cases for core requirement of Pre-MG are listed in Table 1 below.

Table 1 Test cases for Pre-MG core requirements

| No | Type of Test | Description | Test purpose | Notes |
| --- | --- | --- | --- | --- |
| 1-1 | Pre-configured measurement gap activation delay upon DCI/timer-based BWP switching for UE support the autonomous pre-MG activation | FDD/ TDD Pre-configured gap configuration ON, No network signaling to indicate pre-MG activation/deactivation status Gap#0 BWP switching trigger No DRX cycle AWGN | Core requirements in section 8.19.2 which is also rely on UE’s capability to be verified. UE completes the pre-configured MG activation/deactivation within the requirements |  |
| 1-2 | Pre-configured measurement gap activation delay upon DCI/timer-based BWP switching for UE support the signaling pre-MG activation | FDD /TDD Pre-configured gap configuration ON, Network signaling to indicate pre-MG activation/deactivation status Gap#0 BWP switching trigger No DRX cycle AWGN | Core requirements in section 8.19.2 which is also rely on UE’s capability to be verified. UE completes the pre-configured MG activation/deactivation within the requirements |  |
| 2-1 | Intra-freq measurement without gap reporting | TDD/FDD, FR1/FR2 SSB Pre-MG deactivated Gap#0 No DRX cycle Alignment b/w cells = synchronous 2 cells in total AWGN | Core requirements in section 9.9.2.4 which is also rely on UE’s processing capability to be verified. UE reports RSTD within required delay for certain number of cells |  |
| 2-2 | Intra-freq measurement with gap reporting | TDD/FDD, FR1/FR2 SSB Pre-MG activated Gap#0 No DRX cycle Alignment b/w cells = synchronous 2 cells in total AWGN | Core requirements in section 9.9.2.4 which is also rely on UE’s processing capability to be verified. UE reports RSTD within required delay for certain number of cells |  |

- Issue 7. Testing procedure

Nevertheless, in order to reduce the number of test cases as possible, if the testing procedure in in which three successive time periods being included, both the requirements of Pre-MG activation/deactivation and intra-frequency measurement with gap can be tested together.

In our view, the testing procedure for the enhanced measurement gap can be based on the test procedure for the legacy measurement gap in Rel16. The difference is the pre-MG activation/deactivation in the testing procedure is needed.

Figure 1. Testing procedure for measurement with activated Pre-MG

As illustrated in the figure above, the testing procedure for measurements with activated Pre-MG can consist of three successive time periods, with time durations of T1, T2 and T3 respectively.

- During the duration of T1, UE can be configured with Pre-MG but being deactivated.

- At the start of time duration T2, the serving gNB can trigger Pre-MG activation. And UE is expected to complete the Pre-MG activation within T2.

- At the start of time duration T3, the UE may not have any timing information of neighbor cell to be measured (e.g. cell 2).

Figure 2. Testing procedure for measurement with deactivated Pre-MG

The testing procedure for measurements with deactivated Pre-MG can consist of three successive time periods, with time durations of T1, T2 respectively.

- During the duration of T1, UE can be configured with Pre-MG but being deactivated.

- At the start of time duration T2, the UE may not have any timing information of neighbor cell to be measured (e.g. cell 2).

Proposal 8: RAN4 can combine the test cases for Pre-MG activation/deactivation delay and measurement reporting by the testing procedure in Figure 1.

the testing procedure for measurements with activated Pre-MG can consist of three successive time periods, with time durations of T1, T2 and T3 respectively. - During the duration of T1, UE can be configured with Pre-MG but being deactivated. - At the start of time duration T2, the serving gNB can trigger Pre-MG activation. And UE is expected to complete the Pre-MG activation within T2. - At the start of time duration T3, the UE may not have any timing information of neighbor cell to be measured (e.g. cell 2). And UE is expected to complete the measurements by the activated Pre-MG within T3.

# Summary

This paper proposes the main configurations parameters and test cases list to verify the RRM requirements related to NR Pre-MG.

Proposal 1: It is necessary to define the test cases for NR standalone scenario only.

Proposal 2: Test cases for SSB measurement with Pre-MG shall be defined at least.

Proposal 3: Test cases for the intra-frequency with Pre-MG gap can be defined with high priority.

Proposal 4-1: When the pre-MG activation is based on the network signaling, the test cases under both single carrier and CA test cases shall be defined

Proposal 4-2: When the pre-MG activation is based on UE autonomous rules, the test cases under single carrier test cases needed only.

Proposal 5: BWP switching trigger event can be tested firstly.

Proposal 6: Non-DRX cases will be tested only in Rel17.

Proposal 7: The following test cases for core requirement of Pre-MG are listed in Table 1 below.

Table 1 Test cases for Pre-MG core requirements

| No | Type of Test | Description | Test purpose | Notes |
| --- | --- | --- | --- | --- |
| 1-1 | Pre-configured measurement gap activation delay upon DCI/timer-based BWP switching for UE support the autonomous pre-MG activation | FDD/ TDD Pre-configured gap configuration ON, No network signaling to indicate pre-MG activation/deactivation status Gap#0 BWP switching trigger No DRX cycle AWGN | Core requirements in section 8.19.2 which is also rely on UE’s capability to be verified. UE completes the pre-configured MG activation/deactivation within the requirements |  |
| 1-2 | Pre-configured measurement gap activation delay upon DCI/timer-based BWP switching for UE support the signaling pre-MG activation | FDD /TDD Pre-configured gap configuration ON, Network signaling to indicate pre-MG activation/deactivation status Gap#0 BWP switching trigger No DRX cycle AWGN | Core requirements in section 8.19.2 which is also rely on UE’s capability to be verified. UE completes the pre-configured MG activation/deactivation within the requirements |  |
| 2-1 | Intra-freq measurement without gap reporting | TDD/FDD, FR1/FR2 SSB Pre-MG deactivated Gap#0 No DRX cycle Alignment b/w cells = synchronous 2 cells in total AWGN | Core requirements in section 9.9.2.4 which is also rely on UE’s processing capability to be verified. UE reports RSTD within required delay for certain number of cells |  |
| 2-2 | Intra-freq measurement with gap reporting | TDD/FDD, FR1/FR2 SSB Pre-MG activated Gap#0 No DRX cycle Alignment b/w cells = synchronous 2 cells in total AWGN | Core requirements in section 9.9.2.4 which is also rely on UE’s processing capability to be verified. UE reports RSTD within required delay for certain number of cells |  |

Proposal 8: RAN4 can combine the test cases for Pre-MG activation/deactivation delay and measurement reporting by the testing procedure in Figure 1.

the testing procedure for measurements with activated Pre-MG can consist of three successive time periods, with time durations of T1, T2 and T3 respectively. - During the duration of T1, UE can be configured with Pre-MG but being deactivated. - At the start of time duration T2, the serving gNB can trigger Pre-MG activation. And UE is expected to complete the Pre-MG activation within T2. - At the start of time duration T3, the UE may not have any timing information of neighbor cell to be measured (e.g. cell 2). And UE is expected to complete the measurements by the activated Pre-MG within T3.

# References

- R4-2206883, WF on R17 NR MG enhancements – Pre-configured MG

- 3GPP TS 38.133 v17.5.0