3GPP TSG-RAN WG4 Meeting # 103-e                                                            R4-2208143

Electronic Meeting, May 09 – May 20, 2022

Title: 	Draft reply LS on the minimum guard period between two SRS resources for antenna switching

Source: 	CATT

Agenda item:	9.15.1

Document for:	Approval

- Background

RAN1 send a LS [2] to RAN4 to ask RAN4 view on the minimum guard period between two SRS resources for antenna switching for 480 and 960 kHz SCS. The reply LS was discussed in last meeting and no agreements. This contribution provides the draft LS which is the same with the last version of the revised LS in last meeting.

- Discussion

In R15, the SRS antenna switching time was discussed and a reply LS [1] was sent to RAN1. The switching time is the same for FR1 and FR2-1. For FR2-2, there’s no new requirements/performance improvement related to the antenna switching time, such as transient period, LO retuning time, etc. So the evaluation results in R15 still apply to FR2-2. The reply LS [1] in R15 can still be referred by RAN1 for FR2-2 discussion.

Based on above understanding, the reply to RAN1 can be the following. The answer is the same with the last version of the revised LS in last meeting.

Question to RAN4: How many symbol(s) is/are needed to accommodate the required minimum guard time for SRS antenna switching for 480 and 960 kHz respectively, in FR2-2?

Answer from RAN4: The absolute switching time for FR2-2 is the same as the capability evaluated in R15, i.e. the antenna switching time is 15 usec. The detail evaluation results for every possible switching scenario are included in the R15 reply LS R4-1710048 [1]. The decision on the symbol(s) needed to accommodate the required minimum guard time for SRS antenna switching for 480 and 960 kHz SCS is up to RAN1 discussion based on the absolute switching time in R4-1710048.

- Summary

This contribution provides the analysis and draft reply LS for the minimum guard period between two SRS resources for antenna switching. A draft LS is attached in the annex for discussion. The content of the draft LS is the same with the last version of the revised LS in last meeting.

Reference

[1] R4-1710048, “LS Reply to LS related to SRS hopping”, RAN4#NR#3, 18 - 21 Septmeber, 2017

[2] R1-2200796, “LS on a minimum guard period between two SRS resources for antenna switching”, RAN1#107b-e

[3] R4-2206433, “Email discussion summary for [102-e][133] NR_ext_to_71GHz_Part_1”, RAN4#102e.

Draft reply LS to RAN1

3GPP TSG-RAN WG4 Meeting # 103-e                                                            R4-22xxxxx

Electronic Meeting, May 09 – May 20, 2022

Title	Reply LS for the minimum guard period between two SRS resources for antenna switching

Release:	Release 17

Work Item:	NR_ext_to_71GHz-Core

Source:	RAN4

To:	RAN1

Cc:	-

Contact person:

- Name:		Huiping Shan

- E-mail Address:	shanhuiping [at] catt [dot] cn

Attachments:

[1] R4-1710048, “LS Reply to LS related to SRS hopping”, RAN4#NR#3, 18 - 21 September, 2017

1. Overall description

RAN4 would like to thank RAN1 for the LS to RAN4 on a minimum guard period between two SRS resources for antenna switching (R1-2200796). RAN4 discussed the issue and have the following conclusions.

The evaluation results in the R15 reply LS [1] still apply to FR2-2, i.e. the absolute time for the antenna switching is the same as R15 capability.

The answer from RAN4 can be a reference for RAN1 further discussion.

Question to RAN4: How many symbol(s) is/are needed to accommodate the required minimum guard time for SRS antenna switching for 480 and 960 kHz respectively, in FR2-2?

Answer from RAN4: The absolute switching time for FR2-2 is the same as the capability evaluated in R15, i.e. the antenna switching time is 15 usec. The detail evaluation results for every possible switching scenario are included in the R15 reply LS R4-1710048 [1]. The decision on the symbol(s) needed to accommodate the required minimum guard time for SRS antenna switching for 480 and 960 kHz SCS is up to RAN1 discussion based on the absolute switching time in R4-1710048.

2. Actions

To RAN1 group:

ACTION: RAN4 respectfully asks RAN1 to take the above information into account.

3. Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #104-e	22th Aug – 26th Aug 2022 	Toulouse, FR

TSG RAN WG4 Meeting #104-b-e	10th Oct – 19th Oct 2022 	Online