3GPP TSG-RAN4 Meeting #103-e	R4-2208249

Electronic Meeting, May 09 – May 20, 2022

Title: 	TP for 38.863: clause 7.3.2 Conducted transmission characteristics

Source: 	CATT

Agenda item:	8.1.4

Document for:	Approval

- Introduction

This contribution provides a text proposal for 38.863: clause 7.3.2 conducted transmission characteristics [1].

- Reference

[1] TR 38.863, 0.3.0

[2] R4-2207369, TP for 38.863: clause 7.3.2 Conducted transmission characteristics, CATT

- Text proposal

---------------------------------------------------Start of Text proposal---------------------------------------------------------

7.3.2	Transmission characteristics

7.3.2.1	General

Radiated characteristics are defined over the air (OTA), where the radiated interface is referred to as the Radiated Interface Boundary (RIB) as shown in figure 7.3.2.1-1.

Conducted characteristics are defined at individual or groups of TAB connectors at the transceiver array boundary, which is the conducted interface between the transceiver unit array and the composite antenna. The TAB connector is shown in figure 7.3.2.1-1.

Figure 7.3.2.1-1 Reference point for SAN type 1-H

7.3.2.2	Conducted transmitter requirements

7.3.2.2.1	SAN output power

The power limitation for SAN will be specified on manufacture declaration basis, no limitation in RAN4 specification.

Some background information from regulatory can be included based on input.

7.3.2.2.2	Output power dynamics

7.3.2.2.2.1	RE power dynamic range

The RE power control dynamic range is the difference between the power of an RE and the average RE power for a BS at maximum output power for a specified reference condition.

RE power control dynamic range requirement is define in Table 7.3.2.2.2.1-1.

Table 7.3.2.2.2.1-1: RE power control dynamic range

| Modulation scheme used | RE power control dynamic range (dB) |  |
| --- | --- | --- |
| on the RE | (down) | (up) |
| QPSK (PDCCH) | -6 | +4 |
| QPSK (PDSCH) | -6 | +3 |
| 16QAM (PDSCH) | -3 | +3 |
| NOTE: The output power per carrier shall always be less or equal to the maximum output power of the satellite access node. |  |  |

7.3.2.2.2.2	Total power dynamic range

Total power dynamic range for SAN will reuse the requirement from TN BS for the same channel bandwidth.

The downlink (DL) total power dynamic range for each carrier shall be larger than or equal to the level in table 7.3.2.2.2.2-1.

Table 7.3.2.2.2.2-1: Total power dynamic range

| BS channel | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
| bandwidth (MHz) | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 5 | 13.9 | 10.4 | N/A |
| 10 | 17.1 | 13.8 | 10.4 |
| 15 | 18.9 | 15.7 | 12.5 |
| 20 | 20.2 | 17 | 13.8 |

7.3.2.2.3	Transmitted signal quality

7.3.2.2.3.1	Frequency error

For SAN type 1-H, the modulated carrier frequency of each carrier configured by the satellite access node shall be accurate to within 0.05ppm observed over 1 ms.

7.3.2.2.3.2	Modulation quality (EVM)

It is agreed to support QPSK and 16QAM for SAN. 64QAM will be optionally supported based on manufacture declaration.

For SAN type 1-H, the EVM levels of each carrier for different modulation schemes on PDSCH outlined in table 7.3.2.2.3.1-1 shall be met.

Table 7.3.2.2.3.1-1: EVM requirements for satellite access node type 1-H

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| NOTE: Support of 64QAM is based on manufacture declaration |  |