3GPP TSG-RAN WG4 Meeting #103-e	R4-2208324

Electronic Meeting, 9th – 20th May 2022

Agenda Item:	9.15.10.2.1

Source:	Ericsson

Title:	The remaining issues of the PDSCH requirements in 52.6 – 71 GHz band

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on PDSCH demodulation requirements for FR2-2. Furthermore, in RAN4 meeting #102-e, the channel model and the corresponding delay spread have been agreed in [1], such as

The channel models have been decided to be TDLA and TDLD, where both using the following delay spread RMS: - SCS = 120 KHz, 10 ns. - SCS = 480 KHz, 10 ns / 5 ns. - SCS = 960 KHz, 10 ns / 5 ns.

However, several general parameters necessary to the PDSCH requirements have not been agreed yet, such as

Issue 1-3-3: Max Doppler frequency Consider 3 km/h UE speed (200 Hz). FFS on higher UE speed. - Option 1: 10 km/h (650 Hz) - Option 2: 30 km/h (2000 Hz) Issue 1-4-1: Study on phase noise impact for requirements derivation Companies deliver two sets of ideal simulation results for requirement discussion. Result set#1 is without phase noise and set#2 is with phase noise. - No Tx phase noise is modelled - Rx Phase noise is modelled only to find feasible FRC configuration (i.e. achieve maximum throughput and loss in comparison to scenarios without Rx phase noise is less than 1 dB) - 70 GHz carrier frequency is assumed Issue 1-4-2: Phase noise model Consider PN model set 1 and 2 in TR 38.808 for initial simulation results alignment.

We follow-up on these issues in a separate paper [2].

# 2	Remaining issues for PDSCH requirements

Several issues need to be discussed in this meeting. Then, we will provide our proposals to address them.

Issue 3-1-1: SCS for DL requirements definition Consider the following SCS for DL requirements definition: - 120, 480 kHz - FFS 960 kHz Issue 3-1-2: CBW for DL requirements definition 120 kHz: - 100, 400 MHz 480 MHz: - 400 MHz - FFS on 1600 MHz 960 kHz: - 400 MHz - FFS on 2000 MHz Issue 3-3-7: MCS, modulation order for PDSCH requirements Define PDSCH requirements with MCS 4, MCS 13 and MCS [22] - Other MCS values are not precluded Issue 3-3-8: Rank Define PDSCH requirements with: - Rank 1 - FFS FD-OCC is not applied to all the antenna ports for DMRS - FFS Rank 2 Issue 3-3-9: PTRS configuration Consider PTRS configuration as K = 2, L = 1. Issue 3-3-12: Rx processing assumptions Define PDSCH performance requirements by using phase noise compensation - Companies are encouraged to bring results for CPE and CPE+ICI compensation methods

## 2.1	SCS/CBW combinations and general issues

For FR2-2, it is better that we focus on the SCS 120 KHz (as a common SCS with FR2-1) and an additional SCS such as 480 KHz. Furthermore, and to align our efforts with RAN1 progress, we define the performance requirements considering both min CBW and max CBW.

Proposal 1: RAN4 defines the PDSCH demodulation requirements with:

- FR2-2 TDD: SCS = 120 kHz with min CBW = 100 MHz and Max CBW = 400 MHz

- FR2-2 TDD: SCS = 480 kHz with min CBW = 400 MHz and Max CBW = 1600 MHz

It is worth mentioning here that we still deprioritize SCS 960 KHz for the following reasons detailed in the following observation

Observation 1: Using SCS 960 KHz, we increase max CBW by 25 % (from 1600 MHz to 2000 MHz), while the CP is halved (looses 50% of its duration).

Furthermore, we do not set any PRB numbers for the defined CBW and would prefer waiting for RF progress and agreements on that. In Proposal 2, we state some general aspects needed for PDSCH demodulation.

Proposal 2: RAN4 defines the UE demodulation and CSI reporting requirements with:

- Number of receive antennas: 2Rx

- Modulation order: Up to 64QAM

- Both single carrier (FR2-2) and NR-DC FR1 + FR2-2 scenarios

## 2.2	PDSCH performance under Phase noise effects

NR operations in 52.5 GHz – 71 GHz band are susceptible to phase noise (PN) effects that should be assessed and addressed. However, RAN4 was not able to agree on a single representative PN model during the study item and WF [1]. In TR 38.808, two PN models have been defined and referred to as Set 1 and Set 2. Furthermore, these PN models are different and for the sake of fairness, the companies are encouraged to consider both PN models for simulations. Moreover, the Release 15 has defined the PTRS patterns that remain sufficient in this band.

Observation 2: In Rel-17, NR operation in FR2-2 will only support Rel-15 patterns for CP-OFDM.

Proposal 3: Define PDSCH requirements using both PN models in TR 38.808, where

- Option 1: PN is considered at both sides (BS and UE).

- Option 2: PN is considered at the UE only.

Observation 3: When applying TR 38.808 Set 1, we set the design margin as 0 dB at the BS and 5 dB at the UE.

Proposal 4: Using Rel-15 PTRS patterns, define PDSCH requirements when using PN compensation as

- Option 1: CPE compensation only

- Option 2: CPE + ICI compensation.

The following proposal draws the interaction between the parameters needed for tests.

Proposal 5: Define PDSCH demodulation requirements for UE with the following test setup.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA and TDLD (5 ns, 10 ns, 30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h, 30 km/h |
| PA Model | None |
| gNB PN Model | TR38.808 PN model Set 1 and Set 2 |
| UE PN Model | TR38.808 PN model Set 1 and Set 2 |
| Pre-loaded Tx EVM | 0% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| CSI-RS / TRS | CSI-RS/TRS is assumed to be off (for RS overhead) |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 22 (64QAM). Note: It is assumed that NohPRB = 0 for MCS calculations. |

FR2-2 TDD, SCS 120 KHz

For the sake of compatibility with FR2-1, it is recommended to keep same TDD UL/DL pattern for PDSCH when using SCS 120 KHz in FR2-2. We do believe that the delay spread can be up to 30 ns while the velocity could be 30 Km/h for outdoor environment. In the sequel, we highlight the needed updates/modifications.

Observation 4: The proposed TDD UL/DL pattern for test (FR2.120-1) is given by DDDSU, S: 10D:2G:2U.

| Test number | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | SNR (dB) @ 70% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100MHz / 120kHz | QPSK 0.3 Rank 1 | FR2.120-1 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 1-2 | 100MHz / 120kHz | 16QAM 0.48 Rank 1 | FR2.120-1 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 1-3 | 100MHz / 120kHz | 64QAM 0.46 Rank 1 | FR2.120-1 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 1-4 | 100 MHz/ 120 KHz | QPSK 0.3 Rank 1 | FR2.120-1 | TDLA10-2000 TDLD10-2000 TDLA30-2000 TDLD30-2000 | 2x2 ULA Low | TBD | New |
| 1-5 | 100 MHz/ 120 KHz | 16QAM 0.48 Rank 1 | FR2.120-1 | TDLA10-2000 TDLD10-2000 TDLA30-2000 TDLD30-2000 | 2x2 ULA Low | TBD | New |
| 2-1 | 400MHz / 120kHz | QPSK 0.3 Rank 1 | FR2.120-1 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 2-2 | 400MHz / 120kHz | 16QAM 0.48 Rank 1 | FR2.120-1 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 2-3 | 400MHz / 120kHz | 64QAM 0.46 Rank 1 | FR2.120-1 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 2-4 | 400 MHz / 120kHz | QPSK 0.3 Rank 1 | FR2.120-1 | TDLA10-2000 TDLD10-2000 TDLA30-2000 TDLD30-2000 | 2x2 ULA Low | TBD | New |
| 2-5 | 400 MHz / 120kHz | 16QAM 0.48 Rank 1 | FR2.120-1 | TDLA10-2000 TDLD10-2000 TDLA30-2000 TDLD30-2000 | 2x2 ULA Low | TBD | New |

The detailed simulation parameters are provided in Appendix A.1.

FR2-2 TDD, SCS 480 KHz

The PDSCH configurations for SCS 480 KHz should be defined in FR2-2. Below is our proposed test setup for FR2-2 TDD for SCS 480 KHz.

| Test number | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | SNR (dB) @ 70% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 400MHz / 480kHz | QPSK 0.3 Rank 1 | TBD (Note 1) | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 1-2 | 400MHz / 480kHz | 16QAM 0.48 Rank 1 | TBD (Note 1) | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 1-3 | 400MHz / 480kHz | 64QAM 0.46 Rank 1 | TBD (Note 1) | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 1-4 | 400MHz/ 480kHz | QPSK 0.3 Rank 1 | TBD (Note 1) | TDLA10-2000 TDLD10-2000 TDLA30-2000 TDLD30-2000 | 2x2 ULA Low | TBD | New |
| 1-5 | 400MHz/ 480KHz | 16QAM 0.48 Rank 1 | TBD (Note 1) | TDLA10-2000 TDLD10-2000 TDLA30-2000 TDLD30-2000 | 2x2 ULA Low | TBD | New |
| 2-1 | 1600MHz / 480kHz | QPSK 0.3 Rank 1 | TBD (Note 1) | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 2-2 | 1600MHz / 480kHz | 16QAM 0.48 Rank 1 | TBD (Note 1) | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 2-3 | 1600MHz / 480kHz | 64QAM 0.46 Rank 1 | TBD (Note 1) | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 2-4 | 1600 MHz / 480kHz | QPSK 0.3 Rank 1 | TBD (Note 1) | TDLA10-2000 TDLD10-2000 TDLA30-2000 TDLD30-2000 | 2x2 ULA Low | TBD | New |
| 2-5 | 1600 MHz / 480kHz | 16QAM 0.48 Rank 1 | TBD (Note 1) | TDLA10-2000 TDLD10-2000 TDLA30-2000 TDLD30-2000 | 2x2 ULA Low | TBD | New |
| Note 1: As an extension to FR2.120-1, we proposed a TDD UL-DL pattern for SCS 480 KHz. |  |  |  |  |  |  |  |

Observation 5: For SCS 480 KHz, we defined a TDD UL-DL pattern in [2], given by 14D2S4U, S1 = 12D:2G:0U, S2 = 0D:6G:8U.

Observation 6: We proposed an exhaustive list of features to be examined, however, the companies are encouraged to discuss and limit the needed propagation environments, PN model(s), and MCS.

# 5	Specification structure

Table 1	Specification structure for UE Demodulation performance requirements in FR2-2 (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 7.2 | PDSCH demodulation requirements | Update Table 7.2-1 |
| 7.2.1 | 1RX requirements |  |
| 7.2.2 | 2RX requirements |  |
| 7.2.2.2 | TDD |  |
| 7.2.2.2.[4] | Minimum requirements for UEs in FR2-2 | New section |
| 7.2A | PDSCH demodulation requirements for CA |  |
| 7.2A.1 | 1RX requirements |  |
| 7.2A.2 | 2RX requirements | Updates on FR2-2 |
| ……. | …… |  |
| A.3.2 | Reference measurement channels for PDSCH performance requirements |  |
| A.3.2.2 | TDD | Updates on FR2-2 |

# 6	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Proposal 1: RAN4 defines the PDSCH demodulation requirements with:

- FR2-2 TDD: SCS = 120 kHz with min CBW = 100 MHz and Max CBW = 400 MHz

- FR2-2 TDD: SCS = 480 kHz with min CBW = 400 MHz and Max CBW = 1600 MHz

Observation 1: Using SCS 960 KHz, we increase max CBW by 25 % (from 1600 MHz to 2000 MHz), while the CP is halved (looses 50% of its duration).

Proposal 2: RAN4 defines the UE demodulation and CSI reporting requirements with:

- Number of receive antennas: 2Rx

- Modulation order: Up to 64QAM

- Both single carrier (FR2-2) and NR-DC FR1 + FR2-2 scenarios

Observation 2: In Rel-17, NR operation in FR2-2 will only support Rel-15 patterns for CP-OFDM.

Proposal 3: Define PDSCH requirements using both PN models in TR 38.808, where

- PN is considered at both sides (BS and UE).

- PN is considered at the UE only.

Observation 3: When applying TR 38.808 Set 1, we set the design margin as 0 dB at the BS and 5 dB at the UE.

Proposal 4: Using Rel-15 PTRS patterns, define PDSCH requirements when using PN compensation as

- CPE compensation only

- CPE + ICI compensation.

Proposal 5: Define PDSCH demodulation requirements for UE with the following test setup.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA and TDLD (5 ns, 10 ns, 30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h, 30 km/h |
| PA Model | None |
| gNB PN Model | TR38.808 PN model Set 1 and Set 2 |
| UE PN Model | TR38.808 PN model Set 1 and Set 2 |
| Pre-loaded Tx EVM | 0% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| CSI-RS / TRS | CSI-RS/TRS is assumed to be off (for RS overhead) |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 22 (64QAM). Note: It is assumed that NohPRB = 0 for MCS calculations. |

FR2-2 TDD, SCS 120 KHz

Observation 4: The proposed TDD UL/DL pattern for test (FR2.120-1) is given by DDDSU, S: 10D:2G:2U.

| Test number | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | SNR (dB) @ 70% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100MHz / 120kHz | QPSK 0.3 Rank 1 | FR2.120-1 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 1-2 | 100MHz / 120kHz | 16QAM 0.48 Rank 1 | FR2.120-1 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 1-3 | 100MHz / 120kHz | 64QAM 0.46 Rank 1 | FR2.120-1 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 1-4 | 100 MHz/ 120 KHz | QPSK 0.3 Rank 1 | FR2.120-1 | TDLA10-2000 TDLD10-2000 TDLA30-2000 TDLD30-2000 | 2x2 ULA Low | TBD | New |
| 1-5 | 100 MHz/ 120 KHz | 16QAM 0.48 Rank 1 | FR2.120-1 | TDLA10-2000 TDLD10-2000 TDLA30-2000 TDLD30-2000 | 2x2 ULA Low | TBD | New |
| 2-1 | 400MHz / 120kHz | QPSK 0.3 Rank 1 | FR2.120-1 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 2-2 | 400MHz / 120kHz | 16QAM 0.48 Rank 1 | FR2.120-1 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 2-3 | 400MHz / 120kHz | 64QAM 0.46 Rank 1 | FR2.120-1 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 2-4 | 400 MHz / 120kHz | QPSK 0.3 Rank 1 | FR2.120-1 | TDLA10-2000 TDLD10-2000 TDLA30-2000 TDLD30-2000 | 2x2 ULA Low | TBD | New |
| 2-5 | 400 MHz / 120kHz | 16QAM 0.48 Rank 1 | FR2.120-1 | TDLA10-2000 TDLD10-2000 TDLA30-2000 TDLD30-2000 | 2x2 ULA Low | TBD | New |

The detailed simulation parameters are provided in Appendix A.1.

FR2-2 TDD, SCS 480 KHz

| Test number | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | SNR (dB) @ 70% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 400MHz / 480kHz | QPSK 0.3 Rank 1 | TBD (Note 1) | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 1-2 | 400MHz / 480kHz | 16QAM 0.48 Rank 1 | TBD (Note 1) | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 1-3 | 400MHz / 480kHz | 64QAM 0.46 Rank 1 | TBD (Note 1) | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 1-4 | 400MHz/ 480kHz | QPSK 0.3 Rank 1 | TBD (Note 1) | TDLA10-2000 TDLD10-2000 TDLA30-2000 TDLD30-2000 | 2x2 ULA Low | TBD | New |
| 1-5 | 400MHz/ 480KHz | 16QAM 0.48 Rank 1 | TBD (Note 1) | TDLA10-2000 TDLD10-2000 TDLA30-2000 TDLD30-2000 | 2x2 ULA Low | TBD | New |
| 2-1 | 1600MHz / 480kHz | QPSK 0.3 Rank 1 | TBD (Note 1) | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 2-2 | 1600MHz / 480kHz | 16QAM 0.48 Rank 1 | TBD (Note 1) | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 2-3 | 1600MHz / 480kHz | 64QAM 0.46 Rank 1 | TBD (Note 1) | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | TBD | New |
| 2-4 | 1600 MHz / 480kHz | QPSK 0.3 Rank 1 | TBD (Note 1) | TDLA10-2000 TDLD10-2000 TDLA30-2000 TDLD30-2000 | 2x2 ULA Low | TBD | New |
| 2-5 | 1600 MHz / 480kHz | 16QAM 0.48 Rank 1 | TBD (Note 1) | TDLA10-2000 TDLD10-2000 TDLA30-2000 TDLD30-2000 | 2x2 ULA Low | TBD | New |
| Note 1: As an extension to FR2.120-1, we proposed a TDD UL-DL pattern for SCS 480 KHz. |  |  |  |  |  |  |  |

Observation 5: For SCS 480 KHz, we defined a TDD UL-DL pattern in [2], given by 14D2S4U, S1 = 12D:2G:0U, S2 = 0D:6G:8U.

Observation 6: We proposed an exhaustive list of features to be examined, however, the companies are encouraged to discuss and limit the needed propagation environments, PN model(s), and MCS.

# References

- R4-2207223, “WF on demodulation performance requirements definition for 52.6 - 71 GHz”, Intel.

- R4-2208018, “General view on demodulation requirements for NR extended to 71GHz”, Ericsson.

# Appendix	Simulation assumption

## A.1	PDSCH Mapping Type A

### FR2-1

Please refer to 38.101-4 Tables A.3.2.2.5

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| SCS |  | kHz | 120 |
| Active DL BWP index |  |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 160 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 for DL slots 9 for special slots |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Annex A.1.3 |