3GPP TSG-RAN WG4 Meeting # 103-e      		R4-2208414

Electronic Meeting, May 09 – May 20, 2022

Agenda Item:	9.11.2.1.1

Source: 	CMCC

Title: 	Simulation results for inter-cell interference suppressing

Document for:	Information

- Introduction

According to the simulation assumptions of test parameters and the interference model agreed in previous meetings [1-4], we provide our simulation results for inter-cell interference suppressing in this contribution.

- Simulation Results

Table 1. Simulation results

|  | Parameters | MMSE-MRC | MMSE-IRC |
| --- | --- | --- | --- |
| Case 1 | 2T2R ULA low FDD 15kHz 10MHz INR 7.77 and 2.29dB TDLC300-100 | SNR at 70% TP=13.8 | SNR at 70% TP=12.8 |
| Case 2 | 2T4R ULA low FDD 15kHz 10MHz INR 7.77 and 2.29dB TDLC300-100 | SNR at 70% TP=10.2 | SNR at 70% TP=7.9 |
| Case 3 | 2T2R ULA low FDD 15kHz 10MHz INR 7.58dB TDLA 30-10 | SNR at 70% TP=13 | SNR at 70% TP=10.6 |
| Case 4 | 2T4R ULA low FDD 15kHz 10MHz INR 7.58dB TDLA 30-10 | SNR at 70% TP=9.5 | SNR at 70% TP=5.7 |
| Case 5 | 2T2R ULA low TDD 30kHz 40MHz INR 7.77 and 2.29dB TDLC300-100 | SNR at 70% TP=13.8 | SNR at 70% TP=12.8 |
| Case 6 | 2T4R ULA low TDD 30kHz 40MHz INR 7.77 and 2.29dB TDLC300-100 | SNR at 70% TP=10.6 | SNR at 70% TP=7.9 |
| Case 7 | 2T2R ULA low TDD 30kHz 40MHz INR 7.58dB TDLA 30-10 | SNR at 70% TP=13.2 | SNR at 70% TP=10.8 |
| Case 8 | 2T4R ULA low TDD 30kHz 40MHz INR 7.58dB TDLA 30-10 | SNR at 70% TP=9.9 | SNR at 70% TP=6 |

Observation 1: Performance gain from MMSE-IRC can be observed for all test cases.

- Conclusion

In this contribution, we provide our simulation results for inter-cell interference suppressing. The observations are:

Observation 1: Performance gain from MMSE-IRC can be observed for all test cases.

- Reference

[1] R4-2115730, Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC, Intel Corporation.

[2] R4-2120707, Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC, Intel Corporation.

[3] R4-2203008, Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC, Intel Corporation.

[4] R4-2207242, Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC, Intel Corporation.

- Annex

  - Simulation Assumptions

Table 2. Simulation assumptions

| Network type | FDD/TDD sync |
| --- | --- |
| Antenna configuration | 2T2R and 2T4R ULA low |
| SCS and Channel bandwidth | FDD 15kHz/10MHz TDD 30kHz/40MHz |
| Physical cell ID | Physical cell ID of 0 for the serving cell, and cell ID 1 for interferer 1, cell ID 2 for interferer 2 |
| PDCCH and PDSCH allocation in time domain | Use symbols #0 and #1 of each slot for PDCCH PDSCH mapping type A, Start symbol 2, Duration 12 |
| PDSCH allocation in frequency domain | Full PRB |
| SSB configuration | For HomoNet: All SSBs (serving cell and interference cell(s)) are in the same time/frequency resources For HetNet: Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources |
| TRS/CSI-RS configuration | TRS/CSI-RS colliding with TRS/CSI-RS interference |
| DMRS configuration | For both serving and interfering cells, DMRS Type 1 with single symbol front loaded and 1 additional DMRS, with FDM applied between DMRS and data (number of CDM groups without data is equal to 1), i.e., overlapped DMRS between target and interferer |
| Propagation condition | For HomoNet: TDLC300-100 channel models For HetNet: TDLA 30-10 channel models |
| Transmission rank | rank 1 |
| MCS | MCS 13 (16QAM, CR=0.5) |
| Precoding model | Random precoder with Type I SP codebook |
| PRB bundle size | Set PRB bundle size as 2 for target PDSCH |
| HARQ process number | 4 for FDD 8 for TDD |
| Performance measurement point | SNR at 70% TP |
| INR values for Homogeneous deployment assumptions initial simulations | For HomoNet: INRs 7.77 and 2.29 dB For HetNet: INR 7.58dB |
| Transmission rank of interfering PDSCH | random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s) |
| Precoding of interfering PDSCH | Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2. |
| Modulation order of interfering PDSCH | Use 16QAM randomly modulated symbols for initial simulation assumption |
| Timing and frequency offset | For FDD: The serving cell is 3 us for interfering cell 1 and -1 us for interfering cell 2 For TDD: The serving cell is 1.5 us for interfering cell 1 and -0.5 us for interfering cell 2 The serving cell is 300 Hz for interfering cell 1 and -100 Hz for interfering cell 2 |
| Interference cell modeling | Interfering cell has 2Tx and random precoding with PRB granularity of 2. Interfering cell has same channel model as for serving cell and it is independently generated for each cell. Interfering cell has full PDSCH allocation on all slots where serving cell has PDSCH grant. |

  - Simulation Results

Homogeneous FDD

Heterogeneous FDD

Homogeneous TDD

Heterogeneous TDD