# 3GPP TSG-RAN WG4 Meeting #103-e	R4-2208619

# Electronic Meeting, 9th - 20th May, 2022

Source: 	vivo

Title: 	Discussion on remaining Tx RF requirements for FR2-2

Agenda Item:	9.15.3.1

Document for:	Approval

Introduction

In the last meeting, we made a progress on min peak EIRP and spherical coverage for PC1, PC2 and PC3 [1]. However, there are many UE Tx requirements left unfinished. In this contribution, we further discussed the UE Tx requirements for FR2-2.

Discussion

In RAN#95E, the following remaining issues in RAN4 are identified in the exception sheet for 52.6G~71GHz WI.

Core requirements including channel raster, sync raster, MPR, A-MPR, SEM, Spurious, Blocking, CA, ACS, Max input power, FRCs, Mandatory/optional UE channel bandwidths

In the following discussion, we focus on the remaining Tx RF requirements for FR2-2.

FR2-2 Power class

In FR2-2, we considered three power class, PC1 for FWA UE, PC2 for Vehicular UE, and PC3 for Handheld UE, each power class consists of four parts: Min peak EIRP, Max TRP, Max EIRP and Spherical Coverage. In the last meeting, we have reached consensus on minimum peak EIRP and spherical coverage. For Max TRP and Max EIRP, they depend on the regulatory requirements. Based on the earlier agreements, power class requirements for FR2-2 are proposed as follows:

Table 1: UE power class for FR2-2

| Operating band | Power Class (UE type) | Min peak EIRP (dBm) | Max TRP (dBm) | Max EIRP (dBm) | Spherical Coverage |  |
| --- | --- | --- | --- | --- | --- | --- |
| n263 | PC1 (FWA) | 30.6 | 27 | 43 | 19.1 dBm min EIRP at 85%-tile CDF | 11.5 dB Gain drop, 85%ile |
|  | PC2 （Vehicular） | 22.7 | 27 | 43 | 7.6 dBm min EIRP at 60%-tile CDF | 15.1 dB Gain drop, 60%ile |
|  | PC3 (Handheld) | 14.1 | 27 | 43 | 2.3 dBm min EIRP at 50%-tile CDF | 11.8 dB Gain drop, 50%ile |

FR2-2 Out of band emission

For Out of band emission requirements, ACLR and SEM are specified for this purpose. UE ACLR are obtained as 15 dB out of the results from co-existence study for 60/70 GHz. The UE ACLR requirements can be specified as follows:

ACLR

Table 2: General requirements for NRACLR

|  | Channel bandwidth / NRACLR / Measurement bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| NRACLR for band n263 | 15 dB | 15 dB | 15 dB | 15 dB | 15 dB |
| NR channel measurement bandwidth (MHz) | 95.16 | 380.28 | 760.32 | 1520.64 | 1797.12 |
| Adjacent channel centre frequency offset (MHz) | +100 / -100 | +400 / -400 | +800 / -800 | +1600 / -1600 | +2000 / -2000 |

For SEM requirements, FR2-2 SEM can be scaled according to FR2-1 SEM. When combining FR2-1 and FR2-2 SEM tables, the ΔfOOB should be coordinated according to the channel bandwidth for FR2-2. The SEM Table are proposed in Table 3.

SEM

Table 3: General NR spectrum emission mask for frequency range 2.

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz | Measurement bandwidth |
| 0-5 | -5 | -5 | -5 | -5 | -5 | -5 | -5 | 1 MHz |
| 5-10 | -13 | -5 | -5 | -5 | -5 | -5 | -5 | 1 MHz |
| 10-20 | -13 | -13 | -5 | -5 | -5 | -5 | -5 | 1 MHz |
| 20-40 | -13 | -13 | -13 | -5 | -5 | -5 | -5 | 1 MHz |
| 40-80 | -13 | -13 | -13 | -13 | -5 | -5 | -5 | 1 MHz |
| 80-100 | -13 | -13 | -13 | -13 | -13 | -5 | -5 | 1 MHz |
| 100-160 |  | -13 | -13 | -13 | -13 | -5 | -5 | 1 MHz |
| 160-200 |  | -13 | -13 | -13 | -13 | -13 | -5 | 1 MHz |
| 200-400 |  |  | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 400-800 |  |  |  | -13 | -13 | -13 | -13 | 1 MHz |
| 800-1600 |  |  |  |  | -13 | -13 | -13 | 1 MHz |
| 1600-3200 |  |  |  |  |  | -13 | -13 | 1 MHz |
| 3200-4000 |  |  |  |  |  |  | -13 | 1 MHz |

Conclusion

This contribution further discusses UE Tx RF requirements for FR2-2. The power class requirements and out of band emission requirements are proposed in this contribution.

- References

[1] R4-2206536, WF on 60 GHz UE RF, Qualcomm Incorporated, RAN4#102-e.