3GPP TSG-RAN WG4 Meeting #103-e	R4-2208663
Electronic Meeting, May 09 – May 20, 2022

Agenda item:	9.12.3.4

Source:	ZTE Corporation

Title:	TP to TS 38.108 on Conducted receiver characteristics

Document for:	Approval

1	Introduction

In TS38.108 [1], two SAN classes are currently defined which are GEO class and LEO class and corresponding reference measurement channel parameters are specified in table 7.2.2-1 and 7.2.2-2. In addition, there is not a table 7.2.2-3 included in clause 7.2.2.

Therefore, for minimum requirements for Satellite Access Node, table 7.2.2-3 should not be referenced and REFSENS should reference both table 7.2.2-1 for GEO class payload and table 7.2.2-2 for LEO class payload.

References

[1] TS 38108, v0.1.0, Satellite Access Node radio transmission and reception

Text Proposal

<TP for TS 38.108>

7	Conducted receiver characteristics

<< unchanged part >>

7.4.1.2	Minimum requirements for Satellite Access Node

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For SAN, the wanted and the interfering signal coupled to the SAN type 1-H TAB connector are specified in table 7.4.1.2-1 and the frequency offset between the wanted and interfering signal in table 7.4.1.2-2 for ACS. The reference measurement channel for the wanted signal is identified in table 7.2.2-1 and 7.2.2-2 for each SAN channel bandwidth in any operating band and further specified in annex A.1. The characteristics of the interfering signal is further specified in [annex D].

The ACS requirement is applicable outside the SAN RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the SAN RF Bandwidth edges or Radio Bandwidth edges.

Minimum conducted requirement is defined at the TAB connector for SAN type 1-H.

Table 7.4.1.2-1: Satellite Access Node ACS requirement

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20 (Note 1) | PREFSENS + 6 dB | GEO SAN class: -57 LEO SAN class: -60 |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the SAN for that bandwidth. NOTE 2: PREFSENS depends on the SAN channel bandwidth as specified in table 7.2.2-1and 7.2.2-2. |  |  |

Table 7.4.1.2-2: Satellite Access Node ACS interferer frequency offset values

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal center frequency offset from the lower/upper SAN RF Bandwidth edge (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±2.5025 |  |
| 10 | ±2.5075 | 5 MHz DFT-s-OFDM NR signal |
| 15 | ±2.5125 | 15 kHz SCS, 25 RBs |
| 20 | ±2.5025 |  |

<End of TP >