3GPP TSG-RAN WG4 Meeting # 103-e	R4-2208724

Electronic Meeting, May 09 – May 20, 2022

Source:	ZTE Corporation

Title:	Discussions on RRM measurement relaxations for RedCap UEs

Agenda item:	9.19.3.3

Document for:	Discussion

# 1	Introduction

In RAN4 102-e, there were discussions on the RRM relaxations due to reduced capability UEs. In the WF [1] some tentative agreement has been captured to this aspect.

Issue 2-2-1: Scaling factor value when Rel-17 single criteria (stationary) is satisfied - Proposals - Option 1: 8 (Apple) - Option 2: 4 (Ericsson Huawei CMCC) - Option 3: 6 (oppo Nokia vivo Apple xiaomi MTK Nokia QC) Agreement: option 3 Issue 2-2-2: The value of the one fixed long measurement period when both Rel-17 criteria are satisfied - Proposals - Option 1: 8 hours (Apple MTK xiaomi QC) - Option 2: 2 hours (Huawei CMCC Ericsson Nokia) - Option 3: 4 hours (vivo Apple xiaomi oppo) - Option 4: 3 hours (Nokia) Agreement: No consensus

- This paper discusses the issues listed above and provide our views.

2	Discussions

According to the previous RAN2 LS, a new threshold will be introduced, which is the background info of the following discussions.

Observation 1: A new threshold will be introduced in R17, named stationary threshold, and currently only applies to IDLE/INACTIVE Mode. The stationary threshold is based on RSRP and/or RSRQ, and R16 low mobility criterion is taken as a reference.

Issue 2-2-1: Scaling factor value when Rel-17 single criteria (stationary) is satisfied

When a single criteria is met then the scaling factor shall not be too large in this case. The idea behind this scaling factor is to enable power saving for RedCap UEs, which is desirable, but not so much at a cost of much larger delays for RRM measurements. A smaller value is needed to balance the RRM measurement delay and power saving gain.

Scaling factor value is 4 when Rel-17 single criteria (stationary) is satisfied.

Issue 2-2-2: The value of the one fixed long measurement period when both Rel-17 criteria are satisfied

When both of the relaxation criteria are met, the options from last meeting were all quite long enough in our view. Considering that RedCap is different from NB-IoT type of device which usually doesn’t move, mobility is expected (even low mobility) and thus, RRM measurement is also critical for RedCap UEs.

Based on this thinking, we don’t think a large value shall be adopted.

The value of the one fixed long measurement period when both Rel-17 criteria are satisfied is set to 2 hours.

3	Conclusion

Observation 1: A new threshold will be introduced in R17, named stationary threshold, and currently only applies to IDLE/INACTIVE Mode. The stationary threshold is based on RSRP and/or RSRQ, and R16 low mobility criterion is taken as a reference.

- Proposal 1: Scaling factor value is 4 when Rel-17 single criteria (stationary) is satisfied.

- Proposal 2: The value of the one fixed long measurement period when both Rel-17 criteria are satisfied is set to 2 hours.

References

- R4-2206974