3GPP TSG-RAN WG4 Meeting # 103-e	R4-2208814

# Electronic Meeting, May 09- May 20, 2022

Agenda item:	9.21.1.2

Source: 	vivo

Title: 	Remain issues on Efficient activation/de-activation mechanism for one SCG

Document for:	Discussion

- Introduction

In the RAN4 #102-e meeting, basically most of the aspects of activation/deactivation mechanism for SCG requirements were discussed and came to agreements. The agreements and open issues are captured in the WF [1].

In this paper we will provide our views on the last few issues left.

- Discussion

The following was captured in the WF [1] on activation/deactivation mechanism for SCG.

- RAN4 are to discuss the potential RRM impacts due to efficient activation/de-activation mechanism for one SCG - SCG Activation/deactivation delay (Issue 2-2) - UE processing time (Tprocessing) in PSCell activation delay (Issue 2-2-1) - known condition for PSCell activation (Issue 2-2-6) - Direct SCG activation for multiple cells (PSCell+SCell(s)) (Issue 2-2-8) - Interruption requirements (Issue 2-3) - Baseline for interruption due to PSCell activation/deactivation (Issue 2-3-1) - Whether to define interruption due to PSCell activation/deactivation in asynchronous deployment (Issue 2-3-2)

2.2 SCG Activation/deactivation delay

- UE processing time (Tprocessing) in PSCell activation delay (Issue 2-2-1) - When PSCell is activated from deactivated state - If any PSCell parameter is modified - Tprocessing = [20ms]. - Otherwise - Tprocessing = [5 or 10ms] - Note: further discuss if Tprocessing or a different term shall be used

In the last RAN4#102-e meeting, there has come to an initial agreement about the value of Tprocessing in PSCell activation delay. The processing is divided into two cases for the further discussion: whether there any PSCell parameter is modified or not. When there is no parameter modified, Tprocessing should consider the SW processing and RF warm up at least. Tprocessing = 5 or 10ms is highly up to the UE implementation. We prefer 10ms to cover all kinds of implementation. Considering if any PSCell parameter is modified, we understand 20ms is a reasonable value since it needs to spend extra time to reconfigure the modified parameters.

As to whether a new term is needed to replace Tprocessing, e.g. TRF_warmup, we do not observe any strong motivation to change it. And for simplifying workload, there is no need to limit the definition of Tprocessing to single time and discuss the influence of a certain part independently. The exact value of Tprocessing can vary depending on the different scenario.

Proposal 1:

- If PSCell parameters is not modified, Tprocessing could be 20 ms.

- If any PSCell parameter is modified, Tprocessing could be 10ms to cover all kinds of implementation

Proposal 2: To simplify workload, there is no need to replace or redefine Tprocessing with a new term.

- known condition for PSCell activation (Issue 2-2-6) - Option 1: add a condition on top of the conditions agreed in last meeting: - If configured to perform RLM on the deactivated PSCell, RLF has not been detected. - Option 2: don’t add the condition proposed in option1.

We understand the motivation of adding the option 1 to be the known condition for PSCel activation. But it will be more explicit to decouple the known conditions with RLM. In terms of the previous discussions, there are two scenarios which are the RACH and RACH-less based PSCell activation. For RACH-less based activation, it is only considered the case that RLM is configured and no failure is detected. There are no requirements if PSCell is unknown. However, for RACH based activation, it is not required to perform RLM so that we can’t judge the target cell is an unknown cell from this point of view. Therefore, we prefer to decouple the cell known condition with RLM and still reuse the agreement in RAN4#101bis-e meeting as the known condition for PSCell activation.

Proposal 3: For RACH based activation, it is not required to perform RLM so that we can’t judge the target cell is an unknown cell from this point of view. We should decouple known conditions with RLM and still reuse the current agreement to be the known condition for PSCell activation.

- Direct SCG activation for multiple cells (PSCell+SCell(s)) (Issue 2-2-8) - Further discussion on SCG activation for multiple cells (PSCell+SCell(s)) can take place in RAN4 #103-e. If no decision is made in RAN4 #103-e, then no requirements will be defined in Rel-17.

At first, we should clarify clearly the available scenario for multiple cells (PSCell + SCell(s)). Since the further discussion will be only allowed in RAN4#103-e meeting, our understanding is that we can just focus on the scenario for direct PSCell activation and direct multiple SCells activation simultaneously and define the corresponding requirements.

According to the current requirement in section 8.3.4, which considers that if the RRC reconfiguration message for direct SCell activation also configures PSCell addition or PSCell change, the direct SCell activation delay may be longer than the requirements on direct SCell activation at SCell addition. In terms of the case of multiple SCells at addition, it also needs to consider the influence on delay requirement from PSCell addition or PSCell change.

On the other hand, the following requirement on the direct PSCell activation alone came to an agreement in RAN4#101e-bis [2]:

The agreement in RAN4#101e-bis meeting - PSCell activation delay can reuse the legacy requirement of PSCell addition.

Combine these two aspects, we can infer that direct PSCell activation will have an effect on the direct multiple SCells activation when defining the whole delay of Direct SCG activation. We will clarify clearly the affect in the following description.

For the case in Fig 1, there is a main change that needs to be noticed on direct multiple SCells activation. Influenced by the processing time of direct PSCell activation. After received the RRC reconfiguration message and transmitted RRCReconfigurationComplete message, PSCell begin to do software processing and RF warm up. During the Tprocessing, multiple SCells cannot continue to execute the following procedures in parallel, which leads that the first complete SSB burst will not be indicated by the SMTC after slot n + TRRC_Process+T1NR slot length but after slot n + TRRC_Process+T1+TprocessingNR slot length at Point A.

Fig 1 Direct PSCell activation delay and direct SCell activation delay

Through the analysis of this case, the requirements on direct SCG activation (PSCell + SCell(s)) could be defined as:

- The requirement on Direct PSCell activation delay could be reuse the legacy requirement of PSCell addition.

- The requirement on Direct SCell(s) activation delay can use the current requirement in section 8.3.9 as the baseline. Specially,

The definition of TFirstSSB and TFirstSSB_MAX should be changed:

-	TFirstSSB: the time to the end of the first complete SSB burst indicated by the SMTC after slot n + TRRC_Process+T1+TprocessingNR slot length

-	TFirstSSB_MAX: the time to the end of the first complete SSB burst indicated by the SMTC after slot n + TRRC_Process+T1+TprocessingNR slot length

Proposal 4: The requirements on direct SCG activation (PSCell + SCell(s)) could be defined as:

- The requirement on Direct PSCell activation delay can reuse the legacy requirement of PSCell addition.

- The requirement on Direct SCell(s) activation delay can use the requirement in section 8.3.9 as the baseline.

Specially,

  - the definition of TFirstSSB and TFirstSSB_MAX should be changed:

-	TFirstSSB: the time to the end of the first complete SSB burst indicated by the SMTC after slot n + TRRC_Process+T1+TprocessingNR slot length

-	TFirstSSB_MAX: the time to the end of the first complete SSB burst indicated by the SMTC after slot n + TRRC_Process+T1+TprocessingNR slot length

2.3 Interruption requirements

- Baseline for interruption due to PSCell activation/deactivation (Issue 2-3-1) If PSCell is activated from a deactivated status - Option 1: Existing requirements for interruption due to SCell activation/deactivation can be used as a baseline. - Option 2: Existing requirements for interruption due to PSCell addition/release can be used as baseline, i.e., 1ms interruption length. - Whether to define interruption due to PSCell activation/deactivation in asynchronous deployment (Issue 2-3-2) - Option 1: - For SCG activation/deactivation in ENDC, - When SCG is activated (i.e., PSCell is activated), there are no active serving cells in the SCG. The interruption on LTE MCG can refer to clause 7.32.2.5 (Interruptions at SCell activation/deactivation) in TS 36.133. - For SCG activation/deactivation in NR-DC, the interruption requirements can refer to existing interruptions at activation/deactivation specified in clause 8.2.4.2.2 in TS38.133, where sync and async scenario has different interruption length. - Option 2: - For SCG activation/deactivation in ENDC, - When SCG is activated/deactivated, there are no active serving cells in the SCG. The interruption on LTE MCG can refer to clause 7.32.2.4 (Interruptions at Scell addition/release) in TS 36.133. For SCG activation/deactivation in NR-DC, the interruption requirements can refer to existing interruptions at PSCell addition/release specified in clause 8.2.4.2.1 in TS38.133.

Firstly, considering that if any RF parameters are modified, more 0.5 interruption length needs to be used to parameter loading. In terms of the worst case, we think it is reasonable to use the existing requirements for interruption due to PSCell addition/release can be used as baseline for insurance. Secondly, referring to the requirement for PSCell activation delay discussed in the previous meetings, it also inherited from the deley of PSCell addition delay to some extent.

From the above two points, we understand it is more reasonable to use Option 1 as the baseline for interruption due to PSCell activation/deactivation.

Proposal 5: Considering that if any RF parameters are modified, more 0.5 interruption length needs to be used, existing requirements for interruption due to PSCell addition/release can be used as a baseline when PSCell is activated from a deactivated status.

Same logic as the sync case, to align with the sync and async deployment, it also needs longer time for parameter loading for async deployment.

Proposal 6:

- For SCG activation/deactivation in EN-DC,

when SCG is activated/deactivated, there are no active serving cells in the SCG. The interruption on LTE MCG can refer to clause 7.32.2.4 (Interruptions at SCell addition/release) in TS 36.133.

- For SCG activation/deactivation in NR-DC,

the interruption requirements can refer to existing interruptions at PSCell addition/release specified in clause 8.2.4.2.1 in TS38.133.

- Conclusions

Proposal 1:

- If PSCell parameters is not modified, Tprocessing could be 20 ms.

- If any PSCell parameter is modified, Tprocessing could be 10ms to cover all kinds of implementation

Proposal 2: To simplify workload, there is no need to replace or redefine Tprocessing with a new term.

Proposal 3: For RACH based activation, it is no required to perform RLM so that we can’t judge the target cell is an unknown cell from this point of view. We should decouple known conditions with RLM and still reuse the current agreement to be the known condition for PSCell activation.

Proposal 4: The requirements on direct SCG activation (PSCell + SCell(s)) could be defined as:

- The requirement on Direct PSCell activation delay can reuse the legacy requirement of PSCell addition.

- The requirement on Direct SCell(s) activation delay can use the requirement in section 8.3.9 as the baseline.

Specially,

  - the definition of TFirstSSB and TFirstSSB_MAX should be changed:

-	TFirstSSB: the time to the end of the first complete SSB burst indicated by the SMTC after slot n + TRRC_Process+T1+TprocessingNR slot length

-	TFirstSSB_MAX: the time to the end of the first complete SSB burst indicated by the SMTC after slot n + TRRC_Process+T1+TprocessingNR slot length

Proposal 5: Considering that if any RF parameters are modified, more 0.5 interruption length needs to be used, existing requirements for interruption due to PSCell addition/release can be used as a baseline when PSCell is activated from a deactivated status.

Proposal 6:

- For SCG activation/deactivation in EN-DC,

when SCG is activated/deactivated, there are no active serving cells in the SCG. The interruption on LTE MCG can refer to clause 7.32.2.4 (Interruptions at SCell addition/release) in TS 36.133.

- For SCG activation/deactivation in NR-DC,

the interruption requirements can refer to existing interruptions at PSCell addition/release specified in clause 8.2.4.2.1 in TS38.133.

- Reference

[1] R4-2207006 WF on R17 further Multi-RAT Dual-Connectivity enhancements, Huawei

[2] R4- 2202688 WF on R17 further Multi-RAT Dual-Connectivity enhancements, Huawei