3GPP TSG-RAN4 Meeting #103-e		R4-2208822

# Electronic Meeting, May 9 - May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2338 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.133: Correction to measurement requirements for PDC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are TBDs in current requirements. Number of samples for RTT-based PDC with TRS is not finalized. Measurement reporting requirements for UE supporting gNB-SideRTT-BasedPDC-r17 are not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Replace TBD for UE Rx-Tx time difference measurement triggering with correct RRC command - Number of samples for TRS measurement period is specified as 1. - Specifed measurement reporting requirements for UE supporting gNB-SideRTT-BasedPDC-r17 - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurement core requirements for PDC are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12.4, 9.12.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

9.12	Measurement for Propagation Delay Compensation

9.12.1	Introduction

The requirements in this clause are applicable for UE capable of RTT based propagation delay compensation based on TRS and SRS, PRS and SRS, indicated by [TBD].

9.12.2	Requirements Applicability

The requirements in clause 9.12 apply for [periodic/semi-persistent/aperiodic] and triggered UE Rx-Tx time difference measurements, provided:

-	If UE Rx-Tx time difference measurement is based on PRS, the related side conditions given in clause [TBD] are met for a corresponding band.

-	If UE Rx-Tx time difference measurement is based on TRS, the related side conditions given in clause [TBD] are met for a corresponding band.

-	SRS is configured on at least one of the PCell.

9.12.3	Measurement Capability

If UE Rx-Tx time difference measurement is based on PRS, the capability is as indicated by the UE in [TBD].

If UE Rx-Tx time difference measurement is based on TRS, the capability is as indicated by the UE in [TBD].

9.12.4	Measurement period requirements

9.12.4.1 PRS Measurement Period

The UE shall be able to measure UE Rx-Tx time difference on PCell after receiving measObjectRxTxDiff with downlink reference signal of PRS within TUERx-Tx_PRS, where:

TUERxTx, PRS=NPRSslotN'Lavailable_PRSN*(Nsample-1)*Teffect+Tlast

Where:

Lavailable_PRS is the time duration of available PRS resources to be measured during TPRS, and is calculated in the same way as PRS duration K defined in 9.1 of TS 38.214 [26].

NPRSslot is the maximum number of DL PRS resources configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to [durationOfPRS-ProcessingSysmbols] in [TBD] processed every T ms corresponding to [durationOfPRS-ProcessingSymbolsInEveryTms] in [TBD] for a given maximum bandwidth supported by UE corresponding to [supportedBandwidthPRS ] in [TBD],

N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to [maxNumOfDL-PRS-ResProcessedPerSlot] as specified in [TBD],

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample= 4,

- Tlast is the measurement duration for the last UE Rx-Tx time difference measurement, including the sampling time and processing time, Tlast = T + TPRS ,

Teffect is periodicity of UE Rx-Tx time difference measurement:

Teffect= TTPRS*TPRS

Where:

T	corresponds to [durationOfPRS-ProcessingSymbolsInEveryTms] defined in [TBD],

TPRS is the PRS resource periodicity specific for PDC RTT UE Rx-Tx time difference measurement.

Note: the PRS measurement period TUERxTx, PRS can be revisited based on further agreement from RAN1/2 and further RAN4 discussion.

UE is only required to perform UE Rx-Tx time difference on PRS within the active DL BWP.

When UE is configured to perform UE Rx-Tx time difference measurement based on PRS, the requirements apply provided that the SCS of the PRS is same as that of the active BWP on PCell.

9.12.4.2 TRS Measurement Period

The UE shall be able to measure UE Rx-Tx time difference on PCell after receving measObjectRxTxDiff with downlink reference signal of CSI-RS for tracking within TUERx-Tx_TRS, where:

TUERxTx,TRS=Nsample *TTRS

Where:

-	Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample is 1,

-	TTRS is the TRS resource periodicity specific for PDC RTT UE Rx-Tx time difference measurement.

UE is only required to perform UE Rx-Tx time difference on TRS within the active DL BWP.

When UE is configured to perform UE Rx-Tx time difference measurement based on TRS, the requirements apply provided that the SCS of the TRS is same as that of the active BWP on PCell.

9.12.5	Measurement Reporting Requirements

The UE shall report UE Rx-Tx time difference measurement results if UE supports gNB-SideRTT-BasedPDC-r17.

The measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The UE Rx-Tx time difference measurement values contained in measurement report shall be based on the measurement report mapping requirements specified in clause [TBD].

The UE Rx-Tx time difference measurement accuracy shall be fulfilled according to the accuracy requirements specified in clause [TBD] and [TBD] for PRS and TRS, respectively.

# <End of change 1>