3GPP TSG RAN WG4 Meeting #103-e	R4-2208827

Electronic Meeting, May 9 - May 20, 2022

Agenda item:			9.19.3.4

Source:	vivo

Title:	Reply LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times

Document for:	Discussion

- Introduction

In the RAN2 LS [1], RAN4 is asked to provide feedback on feasibility/necessity of introducing offset between CD-SSB and NCD-SSB(s).

1. Overall Description: In RAN2#117-e, it was concluded that, from RAN2 signalling standpoint, CD-SSB and NCD-SSB(s) may be transmitted at different times by configuring an offset. RAN2 would like to ask RAN4 and RAN1 whether such offset is feasible/needed. 2. Actions: To RAN4 and RAN1 ACTION: RAN2 kindly asks RAN4 and RAN1 to take the information above into consideration and provide feedback.

In this contribution, we provide our views on NCD-SSB configuration in terms of timing offset to CD-SSB for RedCap UE.

- Discussion

NCD-SSB time offset configuration could impact RRM measurements. A RedCap UE could be configured with multiple frequency layers, including frequency layers on CD-SSB and NCD-SSB(s). For per-UE measurement gap, all the frequency layers should be covered by a single measurement gap.

Fig 1. Illustration of time offset configuration

As depicted in Fig 1, there could be restrictions on time offset for NCD-SSB. If CD-SSB and NCD-SSB is close enough, then both CD-SSB and NCD-SSB should be covered by the same gap, as Case 1 in Fig 1. Otherwise, time offset for NCD-SSB should guarantee that CD-SSB and NCD-SSB can be covered by different gaps, as Case 2 in Fig 2. The time offset configuration as Case 3 in Fig 1 is not expected as the NCD-SSB frequency layer can never be measured by measurement gap. In case 3, if NCD-SSB measurement is performed without gap, then the configuration should be allowed.

Therefore, it is in general feasible to configure time offset for NCD-SSB with some restrictions.

In our understanding, configuring time offset for NCD-SSB could provide flexibility for NW deployment. Thus, it may help NCD-SSB deployment for RedCap UE.

Observation 1: It is feasible to configure time offset for NCD-SSB so that NCD-SSB and CD-SSB can transmit at different times

Observation 2: Configuration of time offset should guarantee that NCD-SSB and CD-SSB could be covered by same or different gap(s) if measurements on both NCD-SSB and CD-SSB need gaps.

Observation 3: Configuring time offset for NCD-SSB may provide flexibility for NW deployment.

- Summary

In this contribution, we further provided our views on NCD-SSB configuration in terms of timing offset to CD-SSB for RedCap UE. Based on analysis following observations are present.

Observation 1: It is feasible to configure time offset for NCD-SSB so that NCD-SSB and CD-SSB can transmit at different times

Observation 2: Configuration of time offset should guarantee that NCD-SSB and CD-SSB could be covered by same or different gap(s) if measurements on both NCD-SSB and CD-SSB need gaps.

Observation 3: Configuring time offset for NCD-SSB may provide flexibility for NW deployment.

A draft Reply LS is provided in Annex.

- References

- R2-2204115	LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times, RAN2

- Appendix: Draft reply LS

3GPP TSG-RAN4 WG4 Meeting #103-e	R4-22xxxxx

Electronic meeting, May 9 – May 20, 2022

Title:	Reply LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times

Response to:	R2-2204115 (R4-2207619)

Release:	Rel-17

Work Item:	NR_redcap

Source:	RAN4

To:	RAN2, RAN1

Cc:

Contact Person:

Name:	Qian Yang

E-mail Address: qian9.yang@vivo.com

1. Overall Description:

RAN4 thanks RAN2 for the LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times.

RAN4 has discussed the time offset configuration for NCD-SSB and reached following conclusion.

It is feasible to configure time offset for NCD-SSB so that NCD-SSB and CD-SSB can transmit at different times. Configuration of time offset should guarantee that NCD-SSB and CD-SSB could be covered by same or different gap(s) if measurements on both NCD-SSB and CD-SSB need gaps.

Additionally, it is RAN4 understanding that configuring time offset for NCD-SSB may provide flexibility for NW deployment.

2. Actions:

To: RAN WG1

ACTION: 	RAN4 kindly asks RAN2 to take the above RAN4 agreements into account in the future work.

3. Reference:

- R2-2204115, LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #104-e		August 22 – August 26, 2022			Toulouse, FR