GPP TSG-RAN WG4 Meeting # 103-e	R4-2208926

# Electronic Meeting, May 09 – May 20, 2022

Agenda item:	4.1.5.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on FR2 inter-frequency relative RSRP accuracy TC

Document for:	Discussion

# Introduction

The discussion on FR2 inter-frequency relative RSRP accuracy TC has been going on for quite a long time. The current status are summarized in WF [1]. In this paper, we further provide our views on this issue and provide the solutions for moving forward.

# Discussion

Based on the discussion in previous meeting, there are multiple factors contributing to the uncertainty of relative RSRP results which may be not well considered in Rel-15. We take A.7.7.1.2 for example. In the test configuration, there are two cell configured, Cell1 (PCell) and Cell2 (neighbour cell) on a different frequency. The AoA setup is configured as follows: AoA 1 is aligned to the UE Rx beam peak direction as defined in TS 38.101-2, and AoA2 is aligned to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration | 1~2 |  | Setup 4b according to clause A.3.15.4.2 |  | Setup 4b according to clause A.3.15.4.2 |  |
|  |  |  | AoA1 Spherical coverage | AoA2 Rx Beam Peak | AoA1 Spherical coverage | AoA2 Rx Beam Peak |
| Assumption for UE beamsNote 7 | 1~2 |  | Rough |  | Rough |  |

The test requirements to verify the relative accuracy is shown as follows:

Table A.7.7.1.2.3-2: SS-RSRP relative accuracy test requirement

|  | Test requirement Notes1,2,3,4 |
| --- | --- |
| Cell 2 – Cell 1 | SSB_RP2 - SSB_RP1 -δ ≤ Reported RSRP(dB) ≤ SSB_RP2 - SSB_RP1 +δ –(X) |
| Note 1: SSB_RPn is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the cell n under consideration Note 2: δ is the RSRP relative accuracy requirement from Table 10.1.5.1.2-1 Note 3: Void Note 4: X is the Spherical coverage gain difference in dB, derived as (UE Refsens - UE Spherical coverage) from TS 38.101-2 [19] clauses 7.3.2 and 7.3.4, selected according to the UE power class and operating band. X is always a negative value. |  |

The motivation of the test requirements as described in Table A.7.7.1.2.3-2 is to make sure that the RSRP difference will be located within [SSB_RP2 - SSB_RP1 –δ, SSB_RP2 - SSB_RP1 +δ –(X)] to verify whether the relative RSRP accuracy requirements are fulfilled.

In the discussion in previous meetings, the margin in lower bound and upper bound are discussed separately. For the lower bound, there could be two factors contributing to the estimation error. One is the mis-alignment between fine beam and rough beam (D), and the margin due to different antenna gain on different frequency carrier (Ginter). Based on the discussion in last meeting, most companies agree with the above two observations and the only remaining issue is whether Ginter can also apply to intra-band case. From our understanding, there could also be gain difference for intra-band case when the separation between two intra-frequency carriers can be significant.

Proposal 1: Add Ginter in lower band and upper bound for both intra-band and inter-band, which is the margin due different antenna gain on different frequency carriers.

Regarding the upper bound,  as analysed in the last meeting, currently the assumed maximum reported RSRP difference is defined as SSB_RP2 - SSB_RP1 +δ –(X). X is the Spherical coverage gain difference in dB, derived as (UE Refsens - UE Spherical coverage) from TS 38.101-2 clauses 7.3.2 and 7.3.4, selected according to the UE power class and operating band. X is always a negative value.

It is assumed that –X is the largest gain difference from peak direction and spherical coverage direction using rough beam. However, -X is derived based on fine beam peak and spherical coverage direction, then the above assumption only valid based on following two conditions:

- The real gain difference between Fine peak and Fine coverage is smaller than –X.

- The difference between Fine beam peak and Rough beam peak is equal to the difference between Fine beam coverage and Rough beam coverage.

However, based on the discussion in last meeting, the above two conditions may not always true. For condition 1, as –X is derived by the minimum requirements of REFESENS and spherical coverage, it doesn't mean the real gain difference is smaller than –X. For instance, UE may have better performance at peak direction, then it is easily that the real gain difference extends –X. For condition 2, as elaborated in following Fig, if the gain at AoA1 rough beam coverage is exact Z dB (defined in Table B.2.1.3.1-1) lower than AoA1 fine beam coverage, and the gain at AoA2 rough beam peak is close to the gain of fine beam peak. Then the actual difference could be larger than –X. During the discussion in last meeting, company commented that the real difference may not be that severe. We think the comment is valid, but if the difference at peak is 2 dB and that at coverage direction is 4 dB, this will contribute another 2 dB error of the reported value. Companies commented that there is no need to consider the margin (E in the WF) but without much analysis on how the above unrealistic assumptions could work in the test cases.

Fig.1 Maximum gain difference between AoA2 and AoA1

Observation 1:

The upper bound of the current test requirements are based on following two unrealistic assumptions:

- The real gain difference between Fine peak and Fine coverage is smaller than –X.

- The difference between Fine beam peak and Rough beam peak is equal to the difference between Fine beam coverage and Rough beam coverage.

Based on the analysis and simulation results from [2], if n261 27.93 GHz is picked as the neighbour frequency, and n260 37.07 GHz is picked as serving frequency in a test, -67.5 dBm is the best RSRP in sphere coverage which is the “real” rough peak. The maximum difference could be 14.5 dB ± inter-frequency difference, which may easily extend –X (15.2 dB in [2]).

R4-2204374 Table 1. L3-RSRP measurement results based on test setting in A.5.7.1.2 Serving cell; Neighbor cell; Peak on NBRNote2 (SSB_RP3, dBm/SCS); 50% sphere coverage on Serv best Note3 (SSB_RP2, dBm/SCS); 50% sphere coverage on Serv worst Note4 (SSB_RP2, dBm/SCS). n258 24.3Ghz; n258 24.4Ghz; -71; -69.5; -76. n260 37.07Ghz; n260 37.17Ghz; -74; -72.5; -82. n261 27.93Ghz; n261 28.03Ghz; -71; -67.5; -76. Note 1: The frequency difference between two cells within one band is 100 MHz. Note 2: Peak on NBR: UE uses rough beam to measure the RSRP at beam peak Note 3: 50% sphere coverage on Serv best: UE uses rough beam to measure the RSRP for each AoAs within 50% sphere coverage and records the largest value of the RSRP. Note 4: 50% sphere coverage on Serv worst: UE uses rough beam to measure the RSRP for each AoAs within 50% sphere coverage and records the lowest value of the RSRP..

Companies also commented that to define new test procedure to avoid introducing addition margin. However, as commented during the meeting, we fails to see how to reflect the real gain difference in rough beam from two AoAs. The rough beam is very implementation specific which is only used for measurement. If the intention is to have the measurement results by scanning, then the relative accuracy error is already included in the results.

Observation 2: The rough beam is very implementation specific which is only used for measurement, there is not much analysis on how to obtain the reliable real gain difference between rough beam peak and rough beam coverage.

Additionally, the selection of AoA is from fine beam peak and spherical coverage derived from TS 38.101-2. If adopts new test procedure for reflect real gain difference in rough beam, the AoA setup and selection also need modify.

#### A.3.15.4.2 Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction There are 2 active probes in the test. The DL signals, and noise if applicable, are transmitted from the two active probes. One probe is aligned to the UE Rx beam peak direction as defined in TS 38.101-2 [19]. The second is aligned to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for each UE power class.

Based on the analysis above, we prefer not to have new test procedures as least for legacy requirements. Whether to have new test procedure can be discussed in future release which needs thorough evaluation on testability.

Proposal 2: Do not modify the test procedure. Whether to have new test procedure can be discussed in future release which may need thorough evaluation on testability.

Therefore, for the upper bound, we prefer to add additional margin to handle the issue above. In previous meetings, company commented that Y dB is too relaxed. We suggest to consider some compromise value. Provided that the gain difference for intra-band and inter-band is covered by Ginter in both lower bound and upper bound, according to the analysis based on the simulation results in [2] where the actual gain difference can approximate or even exceeds –X, thus 3dB margin is proposed as the results may not cover all possible cases.

Proposal 3: Add additional E + Ginter margin in upper bound, where E is 3 dB.

# Conclusions

Proposal 1: Add Ginter in lower band and upper bound for both intra-band and inter-band, which is the margin due different antenna gain on different frequency carriers.

Observation 1:

The upper bound of the current test requirements are based on following two unrealistic assumptions:

- The real gain difference between Fine peak and Fine coverage is smaller than –X.

- The difference between Fine beam peak and Rough beam peak is equal to the difference between Fine beam coverage and Rough beam coverage.

Observation 2: The rough beam is very implementation specific which is only used for measurement, there is not much analysis on how to obtain the reliable real gain difference between rough beam peak and rough beam coverage.

Proposal 2: Do not modify the test procedure. Whether to have new test procedure can be discussed in future release which may need thorough evaluation on testability.

Proposal 3: Add additional E + Ginter margin in upper bound, where E is 3 dB.

# References

[1] R4-2206791 WF on remaining issues in Rel-15 NR RRM, Huawei, HiSilicon

[2] R4-2204374 Discussion on FR2 inter-frequency relative RSRP accuracy, MediaTek Inc