3GPP TSG-RAN WG4 Meeting # 103-e	R4-2208939

# Electronic Meeting, May 9 - May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2351 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on PUCCH SCell activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to LS reply from RAN1 R1-2200871, the interruption is irrelevant to SCS configuration. Thus, the editor’s notes should be removed. - In the delay requirements, there is no additional uncertainty for PL-RS indication, but the measurement time (i.e. 5 samples) are needed for both FR1 and FR2. And according to previous agreement, it should be added that longer delay can be expected if the PL-RS is unknown. - For invalid TA case, the Tactivation_time should be the same as that for valid TA case with necessary updating compared with legacy requirements. - The conditions about the applicability shall apply to both valid TA and invalid TA case. In the current version, it seems only apply to invalid TA case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the editor’s notes in interruption requirements. - Clarify that X samples for PL-RS apply to both FR1 and FR2, and longer delay can be expected when the PL-RS is unknown. - Change the definition of Tactivation_time for invalid TA case. - Remove Tdelay_PUCCH_SCell in applicability rules. - Editorial changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2.20, 8.2.2.2.18, 8.2.3.2.19, and 8.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

##### 8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation

When one SCell in SCG configured with PUCCH is activated or deactivated,

The UE is allowed an interruption on active serving cell in SCG as defined in clause 8.2.1.2.4.

The starting time of interruption shall be within the delay as defined in clause 8.3.12.

### <End of Change 1>

### <Start of Change 2>

##### 8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation

When one SCell configured with PUCCH is activated or deactivated,

- The UE is allowed an interruption on active serving cell as defined in clause 8.2.2.2.2.

- The starting time of interruption shall be within the delay as defined in clause 8.3.12.

### <End of Change 2>

### <Start of Change 3>

##### 8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation

When one SCell in MCG configured with PUCCH is activated or deactivated,

The UE is allowed an interruption on active serving cell in MCG as defined in clause 8.2.3.2.4.

The starting time of interruption shall be within the delay as defined in clause 8.3.12.

### <End of Change 3>

### <Start of Change 4>

### 8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell

The requirements in this clause shall apply for the UE configured with one downlink SCell and when PUCCH is configured for the SCell being activated.

If the UE has a valid TA for transmitting on an SCell then the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tactivation_time+X+TCSI_ReportingNR slot length,

Where:

-	A TA is considered to be valid provided that the TimeAlignmentTimer [2] associated with the TAG containing the PUCCH SCell is running.

-	THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	Tactivation_time for FR1 is the SCell activation delay in millisecond as specified in section 8.3.2.

-	Tactivation_time for FR2 is the SCell activation delay in millisecond as specified in section 8.3.2 in which Tuncertainty_MAC is updated as below:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

-	TCSI_reporting is the delay (in ms) specified in clause 8.3.2

-	[X] sample measurement time is introduced when PL-RS of target PUCCH SCell is known. Longer activation time is expected if the pathloss reference signal is unknown.

-	FFS under what condition the [X] = 0 or [X] = 5

If the UE does not have a valid TA for transmitting on an SCell then the UE shall be capable to perform downlink actions related to the SCell activation command as specified in [7] for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tactivation_timeNR slot lengthTHARQ+Tactivation_time+TCSI_ReportingNR slot length, and shall be capable to perform uplink actions related to the SCell activation command as specified in [7] for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tdelay_PUCCH_SCellNR slot length n+THARQ+Tdelay_PUCCH_SCell+TCSI_Reporting_PUCCH_SCellNR slot lengthand shall transmit valid CSI report for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tdelay_PUCCH_SCellNR slot length, where:

Tdelay_PUCCH_SCell = Tactivation_time + [X] + max ((TFirst_available_CSI + TCSI_processing), (T1+T2+T3)) + TCSI_reporting_after

Where:

-	Tactivation_time for FR1is the SCell activation delay in millisecond as specified in section 8.3.2.

-	Tactivation_time for FR2 is the SCell activation delay in millisecond as specified in section 8.3.2 in which Tuncertainty_MAC is updated as below:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

-	[X] sample measurement time is introduced when PL-RS of target PUCCH SCell is known. Longer activation time is expected if the pathloss reference signal is unknown.

-	FFS under what condition the [X] = 0 or [X] = 5

-	TFirst_available_CSI: the delay uncertainty in acquiring the first available downlink CSI reference resource.

-	TCSI_processing: the UE processing time for CSI reporting.

-	TCSI_reporting_after: the delay uncertainty in acquiring the first available CSI reporting resource after T3

-	T1 is the delay uncertainty in acquiring the first available PDCCH triggered PRACH occasion in the PUCCH SCell after Tactivation_time.

-	T1 is up to the summation of a delay uncertainty for reception of PDCCH order, SSB to PRACH occasion association period and 10 ms, where SSB to PRACH occasion association period is defined in the table 8.1-1 of TS 38.213

-	T2 is the delay from slot n + (THARQ + Tactivation_time +T1)/NR slot length until UE has obtained a valid TA command for the target PUCCH Scell being activated. Slot n is the slot where the UE receives PUCCH SCell activation command.

-	T3 is the delay for applying the received TA for uplink transmission on target PUCCH SCell being activated, and greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213.

The pathloss reference signal is known for known PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L3 RSRP measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal:

The target pathloss reference signal determination is based on the latest L3 RSRP measurement reporting

The target pathloss reference signal remains detectable during the PUCCH SCell activation period

SNR of the target pathloss reference signal≥-3dB

The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

SNR of the associated SSB ≥-3dB

Otherwise, the pathloss reference signal is unknown.

The pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L1-RSRP measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

The target pathloss reference signal determination is based on the latest L1-RSRP measurement reporting

The target pathloss reference signal remains detectable during the PUCCH SCell activation period

SNR of the target pathloss reference signal≥-3dB

The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

SNR of the associated SSB ≥-3dB

Otherwise, the pathloss reference signal is unknown.

The above delay requirement shall apply provided that:

-	The RA on PUCCH SCell is not interrupted by the RA on PCell otherwise additional delay to activate the SCell is expected; and

-	No SRS carrier based switching occurs during the SCell activation procedure otherwise the PUCCH SCell activation delay can be extended. The starting point and the end-point of an interruption window on PCell or any activated SCell in MCG for NR standalone mode, or on PSCell or any activated SCell in SCG for EN-DC mode is the same as the interruption in single SCell activation requirement in clause 8.3.2.

-	For unknown PUCCH SCell activation, the requirements only apply when UE supports cross PUCCH group CSI reporting capability [TBD], and UE is configured with CSI reporting via SpCell. And the PDCCH order (when applicable) and the activation command for TCI, UL spatial relation, and PL-RS are based on latest valid L1-RSRP reporting via Primary PUCCH group.

### <End of Change 4>