3GPP TSG-RAN WG4 Meeting #103-e	R4-2209049

Electronic Meeting, 9th - 20th May, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RRM performance requirements for RedCap UEs

Agenda item:	9.19.4 RRM performance

Document for:	Discussion

Introduction

The RedCap work item, defined in [1], has the following objectives which impact RRM requirements:

- Reduced maximum UE bandwidth

- Reduced minimum number of Rx branches,

- Relaxed maximum modulation order,

- Support for HD-FDD, FD-FDD and TDD

- Support for extended DRX enhancements

- RRM measurement relaxations

Though some of the requirements are still open for discussion, as listed in the way forward in [5] and in [6], the majority has already been agreed.  In RAN4 #102, RAN4 has endorsed two CRs, one for TS 38.133 [3] and another to TS 36.133 [4], which contain the agreed RRM requirements for RedCap UEs.

In this contribution, we discuss the current RedCap RRM requirements and present an overview of the test cases and accuracy requirements that are necessary based on the latest RAN4 agreements.

Discussion

Accuracy requirements

During the RedCap WI, RAN4 has relied on link level simulations for assessing the performance degradation due to reducing the number of Rx branches. The simulation assumptions were agreed in: [2][7][8][9][10] and [11]. These simulations were used to decide whether to reuse the core requirements from NR Rel-15 or if it was necessary to increase the number of samples for 1 Rx RedCap UEs.

In general, for the definition of Core requirements, RAN4 used the performance of the worst channel among the ones defined in the simulation assumptions. However, the accuracy requirements currently defined in TS 38.133 and TS 36.133 assume that the Io EPRE is constant within the bandwidth, and the performance tests also assume AWGN channel.

Therefore, in our view, it is reasonable that the accuracy requirements are defined based on the performance degradation observed between the performance of 2 Rx RedCap UEs and 1 Rx RedCap UEs in AWGN channel.

Relaxation of the accuracy requirements for 1 Rx RedCap UEs is defined based on the accuracy degradation observed in AWGN channel.

Test Cases in TS 38.133

In the table below, we make an analysis of the test cases that will be needed, considering the requirements that were introduced in Rel-17 RedCap.

RAN4 to consider Table 1 in the discussion of the RRM requirements test cases for RedCap UEs, and on the discussion of the work plan.

Table 1 - List of proposed Test Cases for RedCap UEs

| Group of requirements | Test cases | Sub-test | Comments |
| --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | FR1-> FR1 | intra-frequency | Tests are needed |
|  |  | inter-frequency |  |
|  | FR2 -> FR2 | intra-frequency |  |
|  |  | inter-frequency |  |
| Neighbour cell measurement relaxations | Cell reselection for UE fulfilling stationary criterion, w/o eDRX | Intra-frequency FR1 / FR2 serving and target cell | Tests are needed |
|  |  | Inter-frequency FR1 / FR2 serving and target cell |  |
|  |  | Inter-RAT E-UTRAN target cell and FR1/FR2 serving cell |  |
|  | Cell reselection for UE fulfilling stationary criterion, w/ eDRX | Intra-frequency FR1 / FR2 serving and target cell | Tests are needed |
|  |  | Inter-frequency FR1 / FR2 serving and target cell |  |
|  |  | Inter-RAT E-UTRAN target cell and FR1/FR2 serving cell |  |
|  | Cell reselection for UE fulfilling 1) stationary and not-at -cell-edge criterion 2) stationary and low mobility criterion 3) stationary, not-at- cell-edge and low mobility criterion w/o eDRX | Intra-frequency FR1 / FR2 serving and target cell | Tests are needed |
|  |  | Inter-frequency FR1 / FR2 serving and target cell |  |
|  |  | Inter-RAT E-UTRAN target cell and FR1/FR2 serving cell |  |
|  | Cell reselection for UE fulfilling 1) stationary and not-at -cell-edge criterion 2) stationary and low mobility criterion 3) stationary, not-at- cell-edge and low mobility criterion w/ eDRX | Intra-frequency FR1 / FR2 serving and target cell | Tests are needed |
|  |  | Inter-frequency FR1 / FR2 serving and target cell |  |
|  |  | Inter-RAT E-UTRAN target cell and FR1/FR2 serving cell |  |
| HO (delay and interruptions) |  | intra-frequency, known target cell | Tests are needed |
|  | FR1-> FR1 and FR2 -> FR2 | intra-frequency, unknown target cell |  |
| RRC Re-establishment | FR1-> FR1 and FR2 -> FR2 | intra frequency | Tests are needed |
|  |  | inter frequency |  |
|  |  | intra frequency without serving cell timing |  |
| Random access | 4-step RACH | Contention based | Tests are needed |
|  |  | Non-contention based |  |
|  | 2-step RACH | Contention based |  |
|  |  | Non-contention based |  |
| RRC Connection Release with Redirection | FR1-> FR1 and FR2 -> FR2 |  | Tests are needed |
| Timing | UE Transmit Timing |  | Tests are needed |
|  | Timing advance adjustment accuracy |  | Tests are needed |
| Active BWP switch |  |  | Tests are needed |
| Radio link monitoring | PCell, SSB-based | Out-of-sync, in non-DRX | Tests are needed |
|  |  | In-sync, in non-DRX mode |  |
|  |  | Out-of-sync, in DRX mode |  |
|  |  | In-sync, in DRX mode |  |
|  | PCell, CSI-RS-based | Out-of-sync, in non-DRX | Tests are needed |
|  |  | In-sync, in non-DRX mode |  |
|  |  | Out-of-sync, in DRX mode |  |
|  |  | In-sync, in DRX mode |  |
|  | Scheduling restrictions |  | Tests are needed |
| Beam failure detection and link recovery | FR1 and FR2 PCell | SSB-based in non-DRX mode | Tests are needed |
|  |  | SSB-based in non-DRX mode |  |
|  |  | CSI-RS-based in non-DRX mode |  |
|  |  | CSI-RS -based in non-DRX mode |  |
|  | Scheduling restrictions | SSB-based in non-DRX mode | Tests are needed |
| Active TCI state switching delay | MAC-CE based | PCell in FR1 | Tests are needed |
|  | RRC based | PCell in FR1 |  |
| Intra-frequency measurement procedure | FR1 and FR2 PCell | Event triggered reporting under non DRX | Tests are needed |
|  |  | Event triggered reporting under DRX |  |
|  |  | Event triggered reporting under DRX with SSB index reading |  |
|  | L1-RSRP measurement | SSB based L1-RSRP when DRX is not used | Tests are needed |
|  |  | SSB based L1-RSRP when DRX is used |  |
|  |  | CSI-RS based L1-RSRP when DRX is not used |  |
|  |  | CSI-RS based L1-RSRP when DRX is used |  |
| Inter-frequency measurement procedure | FR1 PCell | Event triggered reporting when DRX is not used | Tests are needed |
|  |  | Event triggered reporting when DRX is used |  |
|  |  | Event triggered reporting when DRX is not used with SSB index reading |  |
| Accuracy for measurements | SS-RSRP | Intra-frequency FR1 serving and target cell | New tests are needed for 1 Rx RedCap |
|  |  | Inter frequency FR1 serving and target cell |  |
|  |  | Inter frequency FR2 serving and FR1 target cell |  |
|  | SS-RSRQ | Intra-frequency FR1 serving and target cell | New tests are needed for 1 Rx RedCap |
|  |  | Inter frequency FR1 serving and target cell |  |
|  | SS-SINR | Intra-frequency FR1 serving and target cell | New tests are needed for 1 Rx RedCap |
|  |  | Inter frequency FR1 serving and target cell |  |
|  | L1-RSRP | SSB based | New tests are needed for 1 Rx RedCap |
|  |  | CSI-RS based | New tests are needed for 1 Rx RedCap |
|  | CSI-RSRP | Intra-frequency FR1 serving and target cell | New tests are needed for 1 Rx RedCap |
|  |  | Inter frequency FR1 serving and target cell | New tests are needed for 1 Rx RedCap |
|  | CSI-RSRQ | Intra-frequency FR1 serving and target cell | New tests are needed for 1 Rx RedCap |
|  | CSI-SINR | Intra-frequency FR1 serving and target cell | New tests are needed for 1 Rx RedCap |
|  |  | Inter frequency FR1 serving and target cell | New tests are needed for 1 Rx RedCap |

Conclusions

In this paper we discuss Nokia’s views on RRM performance requirements and test cases for RedCap, the following proposals are discussed:

- Relaxation of the accuracy requirements for 1 Rx RedCap UEs is defined based on the accuracy degradation observed in AWGN channel.

- RAN4 to consider Table 1 in the discussion of the RRM requirements test cases for RedCap UEs, and on the discussion of the work plan.

References

- RP-211574, “Revised WID on support of reduced capability NR devices”, Ericsson

- R4-2115359, “Simulation assumptions for RedCap cell detection performance”, vivo

- R4-2205624, “Big CR for TS 38.133”, Ericsson

- R4-2205512, “Big CR for TS 36.133”, Ericsson

- R4-2206950, “WF on RedCap RRM requirements”, Ericsson (RAN4 #102)

- R4- 2207105, “WF on eDRX and RRM relaxations requirements for RedCap UE”, vivo (RAN4 #102)

- R4-2120386, “Simulation assumption on hypothetical PDCCH performance for RLM and BFD for RedCap UE”, vivo

- R4-2115360, “Simulation assumptions for RedCap SSB based RRM measurement performance”, Huawei, HiSilicon

- R4-2115361, “Simulation assumptions for RedCap RLM and BFD performance”, Ericsson

- R4-2115362, “Simulation assumptions for RedCap L1 RSRP measurement”, Ericsson

- R4-2115363, “Simulation assumptions for RedCap PBCH detection”, vivo