3GPP TSG-RAN WG4 Meeting #103-e	R4-2209065

Electronic Meeting, 9 May – 20 May 2022

Agenda Item:	9.6.4.1

Source:	Ericsson

Title:	UE demodulation requirements for DL 1024QAM

Document for:	Discussion

# 1	Introduction

RAN4#102-e agreed with the way forward on the UE demodulation and CQI reporting requirements for DL 1024QAM in FR1 [1]. This contribution provides the simulation results according to the agreed parameters.

# 2	Simulation results

Table 1	SNR to achieve 70% of the maximum throughput

|  |  | FDD 2Rx | FDD 4Rx | TDD 2Rx | TDD 4Rx |
| --- | --- | --- | --- | --- | --- |
| Tx EVM 2.5% | Alignment results | 25.9 | 22.7 | 26.1 | 22.9 |
|  | Impairment results | 27.9 | 24.7 | 28.1 | 24.9 |

# References

- R4-2207254, “Way forward for NR DL1024QAM demodulation and CQI reporting requirements”, Ericsson.

# Appendix: Simulation parameters

| Parameters | FDD | TDD |
| --- | --- | --- |
| SCS/CBW | 15kHz and 10MHz | 30kHz and 40MHz |
| TDD UL/DL configuration | N/A | 7D1S2U, with S=6:4:4 Schedule PDSCH in the special slots |
| MCS (MCS index table 4) | MCS23 |  |
| Rank | 1 |  |
| Antenna configuration | 2x2 with ULA Low 2x4 with ULA Low |  |
| Propagation channel | TDLD30-5 |  |
| PDSCH configuration | Type A mapping, Start symbol 2, Duration 12 |  |
| PDSCH DMRS configuration | Type 1, Single symbol, additional DMRS: pos1 |  |
| Tx EVM assumed for simulation | 2.5% |  |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |  |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 10 ms |  |
| Number of HARQ processes | 4 for FDD 15kHz, 8 for TDD 30kHz |  |
| Transform precoding | CP-OFDM |  |
| Allocated RBs | Full BWP |  |
| PRB bundling | 2 |  |
| Precoding model | Random Precoding, per slot and per RB bundling size granularity (codebook configuration: Single panel Type 1) |  |
| Receiver type | MMSE-IRC |  |
| Test metric | 70% of maximum throughput |  |