3GPP TSG-RAN WG4 Meeting # 103-e	R4-2209133

# Electronic Meeting, May 9 - May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2355 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on SRS antenna port switching requirements 38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - NR SRS antenna port switching requirements should be added in introduction part. - According to the agreement in R4-2115336, the prioritization between NR measurement and SRS antenna port switching should be within the same CG. - According to the agreement in R4-2206861, no requirement for AP L1-RSRP/L1-SINR measurements colliding with AP SRS. Current requirement states that no requirement when AP L1-RSRP/L1-SINR is configured regardless of whether colliding with AP SRS. - The interruption on E-UTRA victim cell is defined in TS 36.133 in number of subframes/symbols. It is confused to involve E-UTRA victim cell in TS 38.133 in number of slots. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add NR SRS antenna port switching requirements in introduction parts. - Clarify that UE is not required to perform SRS when colliding with NR measurement in the same CG. - Clarify that no requirement for AP L1-RSRP/L1-SINR measurements colliding with AP SRS - Remove the requirements for E-UTRA victim cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1, 8.2.1.2.18, 8.2.2.1, 8.2.2.2.16, 8.2.3.1, 8.2.3.2.16, 8.2.4.1 and 8.2.4.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

#### 8.2.1.1	Introduction

This clause contains the requirements related to the interruptions on PSCell, and SCell, when

E-UTRA PCell transitions between active and non-active during DRX, or

E-UTRA PCell transitions from non-DRX to DRX, or

E-UTRA SCell in MCG or SCell in SCG is added or released, or

E-UTRA SCell in MCG or SCell(s) in SCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either E-UTRA MCG or NR SCG, or

a supplementary UL carrier or an UL carrier is configured or de-configured, or

UL/DL active BWP is switched on PSCell or SCell in SCG, or

E-UTRA SCell in MCG or SCell(s) in SCG is directly activated and hibernated, or

E-UTRA SCell in MCG is hibernated, or

Multiple SCells in SCG are activated or deactivated, or

SCell dormancy switches, or

CQI/RRM measurement  happens during SCell dormancy, or

UE-specific CBW is changed on PSCell or SCell in SCG, or

CGI reading of an NR neighbour cell with autonomous gaps, or

CGI reading of an E-UTRA neighbour cell with autonomous gaps.

NR SRS carrier based switching, or

E-UTRA SRS carrier based switching, or

NR SRS antenna port switching, or

UE dynamic Tx switches between two uplink carriers, or

SCell in SCG is activated based on aperiodic CSI-RS.

The requirements shall apply for E-UTRA-NR DC with an E-UTRA PCell.

This clause contains interruptions where victim cell is PSCell or SCell belonging to SCG. Requirements for interruptions requirements when the victim cell is E-UTRA PCell or E-UTRA SCell belonging to MCG are specified in TS 36.133 [15].

For a UE which does not support per-FR measurement gaps, interruptions to the PSCell or activated SCG SCells may be caused by EUTRA PCell, EUTRA SCells or SCells on any frequency range. For UE which support per-FR gaps, interruptions to the PSCell or activated SCG SCells may be caused by EUTRA PCell, EUTRA SCells or SCells on the same frequency range as the victim cell.

### <End of Change 1>

### <Start of Change 2>

##### 8.2.1.2.18	 Interruptions at NR SRS antenna port switching

The requirements in this clause are applicable to SRS antenna port switching on FR1.

The UE shall perform SRS antenna port switching only if the below conditions are met.

-	 the SRS switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26] if the carrier on which the higher priority transmission is performed is one entry of txSwitchWithAnotherBand or is the same carrier on which SRS is transmitted.

-	the SRS switching is not colliding with any NR measurements (i.e., SSB/CSI-RS based L1/L3 measurements) and the measurements for RLM/BFD/CBD if the carrier on which the NR measurements and the measurements for RLM/BFD/CBD is performed is one entry of txSwitchImpactToRx or is the same carrier on which SRS is transmitted.

No requirements apply when aperiodic L1-RSRP/L1-SINR measurements collide with aperiodic SRS antenna port switching if the carrier on which the aperiodic L1-RSRP/L1-SINR configured is one entry of txSwitchImpactToRx or is the same carrier on which SRS antenna port switching is scheduled/configured.

When SRS antenna port switching is performed, interruption requirements does not depend on per-FR gap.

For interruption caused by SRS antenna port switching, the victim CC would be based on the band groups signaled in txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability. e.g., an UL interruption is allowed on any of the serving cells belonging to the same group to the SRS carrier as indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells belongs to the same group to the SRS carrier as indicated by txSwitchImpactToRx.

When 1 SRS symbol is configured in a slot for SRS antenna switching and the aggresor and victim carriers are synchronized, the interruption requirement in Table 8.2.1.2.18-1 applies. When 1 SRS symbol is configured in a slot for SRS antenna switching and the aggressor and victim carriers are asynchronized, the interruption requirement in Table 8.2.1.2.18-2 applies. For the rest of SRS configurations, the interruption requirement in Table 8.2.1.2.18-3 applies.

Table 8.2.1.2.18-1: Interruption length in symbols when 1 SRS symbol is configured and aggressor and victim CC are synchronized

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 | 3 | 2 | 2 |
| 30 | 4 | 3 | 3 |
| 60 | 8 | 6 | 5 |
| 120 | 14 | 10 | 8 |

Table 8.2.1.2.18-2: Interruption length in slots when 1 SRS symbol is configured and aggressor and victim CC are asynchronized

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 2 | 2 | 2 |
| 120 | 2 | 2 | 2 |

Table 8.2.1.2.18-3: Interruption length in slots for rest of the SRS configurations for syncronised and asynchronized scenarios

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

### <End of Change 2>

### <Start of Change 3>

#### 8.2.2.1	Introduction

This clause contains the requirements related to the interruptions on PCell and activated SCell if configured, when

up to 7 SCells are configured, de-configured, activated or deactivated, or

a supplementary UL carrier or an UL carrier is configured or de-configured, or

measurements on SCC with deactivated SCell in NR SCG, or

UL/DL BWP is switched on PCell or SCell, or

CGI reading of an NR neighbour cell with autonomous gaps, or

CGI reading of an E-UTRA neighbour cell with autonomous gaps.

UE-specific CBW is changed on PCell or SCell, or

NR SRS carrier based switching, or

NR SRS antenna port switching, or

UE dynamic Tx switches between two uplink carriers, or

SCell is activated based on aperiodic CSI-RS.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure according to TS38.331 [2] for SCell addition/release or MAC control signalling according to TS37.340 [17] for SCell activation/deactivation command.

This clause additionally contains requirements related to interruptions at inter-frequency SFTD between PCell in FR1 and neighbour cell in FR2.

For a UE which does not support per-FR measurement gap, interruptions to the PCell and activated SCell may be caused by SCells on any frequency range. For a UE which supports per-FR gaps, interruptions to PCell and activated SCell may be caused by SCells on the same frequency range as the victim cell.

In addition to standalone NR carrier aggregation when no CCA is configured, the requirements in clause 8.2.2. and all subclauses of 8.2.2 apply when the UE is configured with

-A PCell not using CCA in downlink and one or more SCells using CCA in downlink or

-A PCell and one or more SCells using CCA in downlink

### <End of Change 3>

### <Start of Change 4>

##### 8.2.2.2.16	 Interruptions at NR SRS antenna port switching

The requirements in this clause are applicable to SRS antenna port switching on FR1.

The interruption requirement in this clause is defined based on the band combination capability reported by UE, i.e., based on txSwitchImpactToRx or txSwitchWithAnotherBand.

The UE shall perform SRS antenna port switching only if the below conditions are met.

-	 the SRS switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26] if the carrier on which the higher priority transmission is performed is one entry of txSwitchWithAnotherBand or is the same carrier on which SRS is transmitted.

-	the SRS switching is not colliding with any NR measurements (i.e., SSB/CSI-RS based L1/L3 measurements) and the measurements for RLM/BFD/CBD if the carrier on which the NR measurements and the measurements for RLM/BFD/CBD is performed is one entry of txSwitchImpactToRx or is the same carrier on which SRS is transmitted.

-	 the SRS switching is not colliding with E-UTRA measurement if the carrier on which the E-UTRA measurement is performed is one entry of txSwitchImpactToRx or is the same carrier on which SRS is transmitted.

No requirements apply when aperiodic L1-RSRP/L1-SINR measurements collide with aperiodic SRS antenna port switching if the carrier on which the aperiodic L1-RSRP/L1-SINR configured is one entry of txSwitchImpactToRx or is the same carrier on which SRS antenna port switching is scheduled/configured.

When SRS antenna port switching is performed, interruption requirements does not depend on per-FR gap.

For interruption caused by SRS antenna port switching, the victim CC would be based on the band groups signaled in txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability. e.g., an UL interruption is allowed on any of the serving cells belongs to the same group to the SRS carrier as indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells belongs to the same group to the SRS carrier as indicated by txSwitchImpactToRx.

When 1 SRS symbol is transmitted and the aggresor and victim carriers are synchronized, the interruption requirement in Table 8.2.2.2.16-1 applies. When 1 SRS symbol is transmitted and the aggressor and victim carriers are asynchronized, the interruption requirement in Table 8.2.2.2.16-2 applies. For the rest of configurations, the interruption requirement in Table 8.2.2.2.16-3 applies.

Table 8.2.2.2.16-1: Interruption length in symbols when 1 SRS symbol is configured and aggressor and victim CC are synchronized

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 | 3 | 2 | 2 |
| 30 | 4 | 3 | 3 |
| 60 | 8 | 6 | 5 |
| 120 | 14 | 10 | 8 |

Table 8.2.2.2.16-2: Interruption length in slots when 1 SRS symbol is configured and aggressor and victim CC are asynchronized

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 2 | 2 | 2 |
| 120 | 2 | 2 | 2 |

Table 8.2.2.2.16-3: Interruption length in slots for rest of the SRS configurations for syncronised and asynchronized scenarios

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

### <End of Change 4>

### <Start of Change 5>

#### 8.2.3.1	Introduction

This clause contains the requirements related to the interruptions on PCell and SCell, when

E-UTRA PSCell transitions between active and non-active during DRX, or

E-UTRA PSCell transitions from non-DRX to DRX, or

E-UTRA PSCell/SCell in SCG or SCell in MCG is added or released, or

E-UTRA PSCell/SCell in SCG or SCell(s) in MCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either E-UTRA SCG or NR MCG or

PUSCH/PUCCH carrier configuration and deconfiguration in NR MCG, or

UL/DL BWP is switched on PCell or SCell in MCG, or

UE-specific CBW is changed on PCell or SCell in MCG, or

CGI reading of an NR neighbour cell with autonomous gaps, or

CGI reading of an E-UTRA neighbour cell with autonomous gaps.

NR SRS carrier based switching, or

E-UTRA SRS carrier based switching, or

NR SRS antenna port switching, or

SCell in NR MCG is activated based on aperiodic CSI-RS.

The requirements shall apply for NE-DC with an NR PCell.

This clause contains interruptions where victim cell is PCell or SCell belonging to MCG. Requirements for interruptions requirements when the victim cell is E-UTRA PSCell or E-UTRA SCell belonging to SCG are specified in TS 36.133 [15].

For a UE which does not support per-FR measurement gap, interruptions to the PCell, E-UTRA PSCell or activated MCG SCells may be caused by EUTRA PSCell, EUTRA SCells or SCells on any frequency range. For UE which support per-FR gap, interruptions to the PCell, E-UTRA PSCell or activated MCG SCells may be caused by EUTRA PSCell, EUTRA SCells or SCells on the same frequency range as the victim cell.

### <End of Change 5>

### <Start of Change 6>

##### 8.2.3.2.16	 Interruptions at NR SRS antenna port switching

The requirements in this clause are applicable to SRS antenna port switching on FR1.

The interruption requirement in this clause is defined based on the band combination capability reported by UE, i.e., based on txSwitchImpactToRx or txSwitchWithAnotherBand.

The UE shall perform SRS antenna port switching only if the below conditions are met.

-	 the SRS switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26] if the carrier on which the higher priority transmission is performed is one entry of txSwitchWithAnotherBand or is the same carrier on which SRS is transmitted.

-	the SRS switching is not colliding with any NR measurements (i.e., SSB/CSI-RS based L1/L3 measurements) and the measurements for RLM/BFD/CBD if the carrier on which the NR measurements and the measurements for RLM/BFD/CBD is performed is one entry of txSwitchImpactToRx or is the same carrier on which SRS is transmitted.

No requirements apply when aperiodic L1-RSRP/L1-SINR measurements collide with aperiodic SRS antenna port switching if the carrier on which the aperiodic L1-RSRP/L1-SINR configured is one entry of txSwitchImpactToRx or is the same carrier on which SRS antenna port switching is scheduled/configured.

When SRS antenna port switching is performed, interruption requirements does not depend on per-FR gap.

For interruption caused by SRS antenna port switching, the victim CC would be based on the band groups signaled in txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability. e.g., an UL interruption is allowed on any of the serving cells belongs to the same group to the SRS carrier as indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells belongs to the same group to the SRS carrier as indicated by txSwitchImpactToRx.

When 1 SRS symbol is transmitted and the aggresor and victim carriers are synchronized, the interruption requirement in Table 8.2.3.2.16-1 applies. When 1 SRS symbol is transmitted and the aggressor and victim carriers are asynchronized, the interruption requirement in Table 8.2.3.2.16-2 applies. For the rest of configurations, the interruption requirement in Table 8.2.3.2.16-3 applies.

Table 8.2.3.2.16-1: Interruption length in symbols when 1 SRS symbol is configured and aggressor and victim CC are synchronized

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 | 3 | 2 | 2 |
| 30 | 4 | 3 | 3 |
| 60 | 8 | 6 | 5 |
| 120 | 14 | 10 | 8 |

Table 8.2.3.2.16-2: Interruption length in slots when 1 SRS symbol is configured and aggressor and victim CC are asynchronized

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 2 | 2 | 2 |
| 120 | 2 | 2 | 2 |

Table 8.2.3.2.16-3: Interruption length in slots for rest of the SRS configurations for syncronised and asynchronized scenarios

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

### <End of Change 6>

### <Start of Change 7>

### 8.2.4	NR-DC: Interruptions

#### 8.2.4.1	Introduction

This clause contains the requirements related to the interruptions on PCell, PSCell and activated SCell if configured, when

up to 1 SCell in FR1 and up to 7 SCell(s) in FR2 are configured, deconfigured, activated or deactivated or,

a supplementary UL carrier or an UL carrier is configured or de-configured, or

measurements on SCC with deactivated SCell in NR SCG, or

measurements on the deactivated PSCell in NR SCG, or

UL/DL BWP is switched on PCell, PSCell or SCell,

UE-specific CBW is changed on PCell, PSCell or SCell, or

transitions between active and non-active during DRX, or

transitions from non-DRX to DRX, or

CGI reading of an NR neighbour cell with autonomous gaps, or

CGI reading of an E-UTRA neighbour cell with autonomous gaps.

NR SRS carrier based switching,

NR SRS antenna port switching, or

RLM/BFD Measurement on deactivatd NR PSCell, or

NR SCell is activated based on aperiodic CSI-RS.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure [2] for SCell addition/release or MAC control signalling [17] for SCell activation/deactivation command.

The requirements shall apply for NR-DC with an NR PCell, PSCell or SCell.

For a UE which does not support per-FR measurement gap, interruptions to the PCell and activated SCell may be caused by SCells on any frequency range. For a UE which supports per-FR gaps, interruptions to PCell, PSCell and activated SCell may be caused by SCells on the same frequency range as the victim cell.

### <End of Change 7>

### <Start of Change 8>

##### 8.2.4.2.14	Interruptions at NR SRS antenna port switching

The requirements in this clause are applicable to SRS antenna port switching on FR1.

The interruption requirement in this clause is defined based on the band combination capability reported by UE, i.e., based on txSwitchImpactToRx or txSwitchWithAnotherBand.

The UE shall perform SRS antenna port switching only if the below conditions are met.

-	the SRS switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26] if the carrier on which the higher priority transmission is performed is one entry of txSwitchWithAnotherBand or is the same carrier on which SRS is transmitted.

-	the SRS switching is not colliding with any NR measurements (i.e., SSB/CSI-RS based L1/L3 measurements) and the measurements for RLM/BFD/CBD in the same CG if the carrier on which the NR measurements and the measurements for RLM/BFD/CBD is performed is one entry of txSwitchImpactToRx or is the same carrier on which SRS is transmitted.

-	the SRS switching is not colliding with E-UTRA measurement if the carrier on which the E-UTRA measurement is performed is one entry of txSwitchImpactToRx or is the same carrier on which SRS is transmitted.

No requirements apply when aperiodic L1-RSRP/L1-SINR measurements collide with aperiodic SRS antenna port switching if the carrier on which the aperiodic L1-RSRP/L1-SINR configured is one entry of txSwitchImpactToRx or is the same carrier on which SRS antenna port switching is scheduled/configured.

When SRS antenna port switching is performed, interruption requirements does not depend on per-FR gap.

For interruption caused by SRS antenna port switching, the victim CC would be based on the band groups signaled in txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability. e.g., an UL interruption is allowed on any of the serving cells belongs to the same group to the SRS carrier as indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells belongs to the same group to the SRS carrier as indicated by txSwitchImpactToRx.

When 1 SRS symbol is transmitted and the aggresor and victim carriers are synchronized, the interruption requirement in Table 8.2.4.2.14-1 applies. When 1 SRS symbol is transmitted and the aggressor and victim carriers are asynchronized, the interruption requirement in Table 8.2.4.2.14-2 applies. For the rest of configurations, the interruption requirement in Table 8.2.4.2.14-3 applies.

Table 8.2.4.2.14-1: Interruption length in symbols when 1 SRS symbol is configured and aggressor and victim CC are synchronized

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 | 3 | 2 | 2 |
| 30 | 4 | 3 | 3 |
| 60 | 8 | 6 | 5 |
| 120 | 14 | 10 | 8 |

Table 8.2.4.2.14-2: Interruption length in slots when 1 SRS symbol is configured and aggressor and victim CC are asynchronized

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 2 | 2 | 2 |
| 120 | 2 | 2 | 2 |

Table 8.2.4.2.14-3: Interruption length in slots for rest of the SRS configurations for syncronised and asynchronized scenarios

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

### <End of Change 8>