3GPP TSG-RAN WG4 Meeting #102-e	R4-2209270

# Electronic meeting, May 09th – May 20th , 2022

Source:		UIC

Title:			TP 900MHz RMR band – conclusion

Agenda item:		8.2

Document for:	Approval

# Introduction

The provision of the RMR900 spectrum for 5G NR was started in the RAN#91 meeting through the corresponding WID approval.

## ECC RMR

# Discussion

In the meantime, the necessary subject areas 5G NR system, UE RF and BS RF have been developed in compliance with the regulatory requirements of ECC Decision (20)02 [1].

Observation 1: The necessary work for the provision of the RMR900 band using 5G NR can be completed after RAN4#102-e meeting.

# Conclusion

The following proposal is made :

Proposal 1: The conclusion clause provides the necessary outline of the work and the recommendation to transfer the findings to the corresponding normative technical specifications.

# References

- ECC Decision(20)02,  Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR), 20 November 2020

# Annex TP for conclusion clause

#### ***********************************************1st Change******************************************

# 10	Conclusion

The study dealt with how the 900MHz spectrum block (RMR900 874.4-880MHz/919.4-925MHz) can be used for rail communication using 5G NR applicable in Europe. The sub-areas system, BS RF and UE RF were examined in detail.

General:	The focus was on the use of RMR900 under uncoordinated conditions, assuming that RMR BS operates independently of other operators e.g. MFCN. Co-location with other operators and resulting coordination is subject to national regulation and will be not further detailed by 3GPP RAN4.

System Parameter: The RMR900 spectrum block can be used by 5MHz CBW reusing present sync raster design. The corresponding system parameters are to be transferred to 3GPP TS 38.101-1 [2] and 3GPP TS 38.104 [4].

BS RF:	The ECC Decision (20)02 [1], i.e. EIRP limits were analysed accordingly and the necessary conclusions for an uncoordinated operational approach were derived. These include the BS rated output power using corresponding assumptions, the resulting emission values of the transmitter and the specifications of the receiver. The agreed parameters are to be transferred to 3GPP TS 38.104 [4]. Furthermore, the coexistence with other 3GPP radio technologies has been included. These are to be captured in the corresponding normative 3GPP technical specifications.

UE RF:	The ECC Decision (20)02 [1], i.e. EIRP limits were analysed accordingly, and the necessary conclusions were derived for the use of Power Class 3 (23dBm). The agreed parameters in the present TR are to be transferred according to 3GPP TS 38.101-1 [2].

#### *********************************************End of Changes******************************************