3GPP TSG-RAN WG4 Meeting #103-e	R4-2209296

# Electronic Meeting, 9 - 20 May, 2022

Source:	Huawei, HiSilicon

Title:	Updated TP for 37.717-00-00 to add SUL configuration SUL_n79C-n80A for CA_n3A_SUL_n79C-n80A

Agenda item:	8.19.2

Document for:	Approval

1 Background

This contribution provides updated text proposal on the NR SUL band combination CA_n3A_SUL_n79C-n80A with SUL_n79C-n80A.

2 Text Proposal

---Start of changes---

## 5.21	CA_n3_SUL_n79-n80

### 5.21.1	Operating bands

Table 5.21.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n3_SUL_n79-n80 | n3, n79, n80 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.21.2	Channel bandwidths per operating band

Table 5.21.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A_SUL_n79A-n80A | SUL_n79A-n80A | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  | n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| CA_n3A_SUL_n79C-n80A | SUL_n79A-n80A SUL_n79C-n80A | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 5.21.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

### 5.21.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.21.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n3_SUL_n79-n80.

Table 5.21.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n3 | DL frequency range | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 |
| n79 | UL/DL frequency range | 4400 | 5000 | 8800 | 10000 | 15000 | 8070 | 17600 | 20000 |
| n80 | UL frequency range | 1710 | 1785 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 |

Referring to clause 7.3.2.2 from TR 38.862, we can get the IMD order for contiguous UL CA as below.

WGap = lowest band edge of n79 - highest DL band edge of n3 = 4400 – 1880 = 2520 MHz.

Min(maxUL aggregated BW, UL band bandwidth) = Min(200, 600) = 200 MHz.

IMD order = 2*ceil(WGap /Min(maxUL aggregated BW, UL band bandwidth) + 1 = 27.

Since IMD order is > 11, MSD can be ignored.

### 5.21.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.21.5-1: Supplementary uplink configuration for reference sensitivity

|  | NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n3 | n801 | 15 | 25 | 50 | 50 | 50 | 50 | 50 | 50 |  |  |  |  |  |  |
| n79 | n80 | 15 |  |  |  |  |  |  | 160 | 160 | 160 |  | 160 |  | 160 |
| NOTE 1: The Tx-Rx carrier center frequency separation between SUL band and DL band is the same as the Tx-Rx carrier center frequency separation of DL band specified in table 5.4.4-1 from TS 38.101-1. The channel bandwidth of SUL band is the same as DL band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.21.6	∆TIB and ∆RIB values

For CA_n3_SUL_n79-n80, the TIB,c and RIB,c values are given in the tables below.

Table 5.21.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3_SUL_n79-n80 | n3 | 0.3 |
|  | n79 | 0.8 |
|  | n80 | 0.3 |

Table 5.21.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3_SUL_n79-n80 | n79 | 0.5 |

---End of changes---

Reference

[1]	RP-220695, “Revised WID: Band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP)”, Huawei, HiSilicon