3GPP TSG-RAN WG4 Meeting #103-e		R4-2209431
Electronic Meeting, 09th May. – 20th May., 2022

Agenda item:	9.2.3.2

Source:	OPPO

Title:	Discussion on deriving performance requirement

Document for:	Approval

1	Introduction

There are two RAN4 meeting left for the performance part of FR1 TRP TRS WI, and the performance requirements are expected to be derived by the end of WI. There are still some open issues on specifying TRP and TRS requirement.

To make the progress smooth in the following two meeting, this paper discusses the open issues and presents our views.

2	Discussion

2.1 Calculation of TRP and TRS

In the last RAN4 #102-e meeting, the definition of TRP and TRS is improved and approved [1]. There are two forms of summation captured in the TR 38.834, i.e. based on the sin weights and based on the Clenshaw-Curtis quadrature integral approximation.

For the purpose of lab alignment and performance requirement derivation, it is recommended to specify one summation form to keep the process of TRP and TRS calculation consistent.

Proposal 1: It is recommended to specify one summation form to keep the process of TRP and TRS calculation consistent during the activity of lab alignment and performance requirement derivation.

2.2 Specifying TRP and TRS requirement

The value at which percentile of the CDF curve could be selected as the minimum requirement is TBD in the last meeting [2]. From the discussion, there are two potential options, i.e. 80% percentile of CDF curve and 95% percentile of CDF curve. The proposal of 80% percentile consists with other SDOs, while the proposal of 95% percentile considers about the device market access rate. Both of the proposals are reasonable.

From the past practice of 3GPP and other SDOs, there is no necessary correlation between X% percentile of CDF curve and device market access rate for long term exercise. Furthermore, 80% percentile of CDF curve keep the specification from 3GPP well aligned with other SDOs. Therefore, 80% percentile of CDF curve is proposed.

On the other hand, the device market access rate is indeed essential issue which should be carefully treaded, especially at the initial stage. From this perspective, it is proposed to consider properly additional relaxation based on the collected measurement data of commercial devices.

Proposal 2: Select the value of 80% percentile of CDF curve for deriving the minimum requirement, and consider properly additional relaxation based on collected measurement data of commercial devices.

3	Conclusion

Proposal 1: It is recommended to specify one summation form to keep the process of TRP and TRS calculation consistent during the activity of lab alignment and performance requirement derivation.

Proposal 2: Select the value of 80% percentile of CDF curve for deriving the minimum requirement, and consider properly additional relaxation based on collected measurement data of commercial devices.

References

[1] R4-2207321 Discussion and TP on performance metrics

[2] R4-2207425 Email discussion summary for [102-e][335] FR1_TRP_TRS_Part1