3GPP TSG-RAN WG4 Meeting #103-e	R4-2209440

Electronic Meeting, May 9- May 20, 2022

Agenda Item:	9.11.2.2

Source:	Ericsson

Title:	Simulation results on PDSCH performance for intra-cell inter-user interference

Document for:	Information

# 1	Introduction

In this contribution, the MMSE-IRC for suppressing inter-cells’ interference performance simulation results based will be shown. The detailed simulation assumptions are listed in table below.

Table 1. Common Parameters

| Parameters | Serving cell FDD |  | Serving cell TDD |
| --- | --- | --- | --- |
| Network scenario | Sync |  | Sync |
| Number of carriers | single carrier |  |  |
| Carrier frequency | [2 GHz] | [4GHz] |  |
| PDSCH allocation in frequency domain | Full PRB |  |  |
| Physical cell ID | 0 |  | 0 |
| Active DL BWP index | 1 |  | 1 |
| Channel bandwidth | 10MHz |  | 40MHz |
| TDD configuration | N/A |  | 7DS2U, S=6D+4G+4U |
| SCS | 15kHz |  | 30kHz |
| PDCCH allocation in time domain | symbols #0 and #1 of each slot for PDCCH |  |  |
| SSB position in burst | SSB index #0 |  |  |
| SSB periodicity | 20ms |  |  |
| TRS configuration | k0=0 for CSI-RS resource 1,2,3,4 l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 Density 3, 1 port Periodicity 20 slots Offset: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |  | k0=0 for CSI-RS resource 1,2,3,4 l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 Density 3, 1 port Periodicity 40 slots Offset: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| CSI-RS configuration | Not necessary of PDSCH Demod |  |  |
| Propagation condition | For rank 1+1: TDLC300-100 For rank 2+2: TDLA30-10 |  |  |
| Time Offset relative to serving cell | 0 us |  |  |
| Metric | SNR at 70% of max TP |  |  |
| Traffic model | Full buffer |  |  |
| Number of HARQ Processes | 4 | 8 |  |
| Maximum HARQ transmission | 4 |  |  |

Table 2. UE specific parameters

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Channel Bandwidth/SCS |  | MHz/KHz | 10/15 for FDD and 40/30 for TDD |  |
| Duplex mode |  |  | FDD and TDD |  |
| TDD pattern |  |  | 7D1S2U |  |
| MCS |  |  | 13 | 16 QAM random symbols |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 2 | 2 |
|  | DMRS port allocation |  | rank 1+1: Port 1000 rank 2+2: Port 1000 and 1001 | rank 1+1: Port 1001 rank 2+2: Port 1002 and 1003 |
| MIMO configuration |  |  | rank1+1: 2T2R and 2T4R, ULA Low rank 2+2: 4T4R ULA Low |  |
| Number of CDM groups without data |  |  | Rank 1+1: 1 Rank 2+2: 2 |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | - rank 1+1: Select the PMI matrix randomly for Target UE - rank 2+2: Select the precoding matrix to ensure orthogonality with Target UE |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| PDSCH Scrambling configuration |  |  | Same scrambling ID |  |
| Receiver type |  |  | MMSE-IRC | N/A |

# 2	Simulation Results

The simulation results are summarized as follow.

Table 3. simulation results for MU-MIMO FDD TDLA30-10

| Case Index | Rank combination | PMI | Antenna configuration | DMRS ports | CDM group | MCS | SNR [dB] @ 70% of max T-put |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | MRC | IRC | Gain |
| #1 | (2,2) | Orthogonal | 4*4 | (0,1; 2,3) | different | 13 | N/A | 11.56 | inf |

Table 4. simulation results for MU-MIMO FDD TDLC300-100

| Case Index | Rank combination | PMI | Antenna configuration | DMRS ports | CDM group | MCS | SNR [dB] @ 70% of max T-put |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | MRC | IRC | Gain |
| #2 | (1,1) | Random | 2*2 | (0; 1) | same | 13 | N/A | 13.57 | inf |

Table 5. simulation results for MU-MIMO TDD TDLA30-10

| Case Index | Rank combination | PMI | Antenna configuration | DMRS ports | CDM group | MCS | SNR [dB] @ 70% of max T-put |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | MRC | IRC | Gain |
| #3 | (2,2) | Orthogonal | 4*4 | (0,1; 2,3) | different | 13 | N/A | 12.40 | inf |

Table 6. simulation results for MU-MIMO TDD TDLC300-100

| Case Index | Rank combination | PMI | Antenna configuration | DMRS ports | CDM group | MCS | SNR [dB] @ 70% of max T-put |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | MRC | IRC | Gain |
| #4 | (1,1) | Random | 2*2 | (0; 1) | same | 13 | N/A | 14.53 | inf |

Observation 1: Compared with MRC, IRC receiver can have obvious gain in 70% of maximum throughput SNR point in all the configurations.

Detail simulation results are shown in the appendix.

# References

- R4-2207244, “Way Forward on MMSE-IRC for intra-cell inter-user interference”, Huawei, HiSilicon

# Appendix

FDD

- TDLA30-10

Figure 1. IRC and MRC performance for rank2+2 FDD (Orthogonal PMI)

- TDLA300-100

Figure 2. IRC and MRC performance for rank1+1 same CDM group FDD (Random PMI, 2Rx, 4Rx)

TDD

- TDLA30-10

Figure 5. IRC and MRC performance for rank2+2 TDD (Orthogonal PMI)

- TDLA300-100

Figure 6. IRC and MRC performance for rank1+1 same CDM group TDD(Random PMI, 2Rx, 4Rx)