3GPP TSG-RAN WG4 Meeting #103-e	R4-2209451

Electronic Meeting, May 9- May 20, 2022

Agenda Item:	9.10.2.2

Source:	Ericsson

Title:	Test case list for multiple concurrent MG patterns

Document for:	Discussion

# Introduction

In last RP meeting, the Core part of multiple concurrent and independent gap patterns was closed. In this contribution, we’ll discuss the test case design and list for concurrent gaps requirement.

# Test case design

The concurrent gaps can be applied for different usages, such as NR mobility, inter-RAT measurement and positioning. Especially, some special configuration rules are defined for positioning measurement. Thus, we prefer to define the test cases to at least include the following usages

- SSB-based L3 measurement + SSB-based L3 measurement

- SSB-based L3 measurement + LTE inter-RAT measurement

- SSB-based L3 measurement + NR positioning measurement

Proposal 1: Define concurrent gaps test cases to cover at least the following usage combinations:

- SSB-based L3 measurement + SSB-based L3 measurement

- SSB-based L3 measurement + LTE inter-RAT measurement

- SSB-based L3 measurement + NR positioning measurement

To simplify the test case setting, we prefer the following test configuration:

- 1 test for intra-frequency measurement and 1 test for inter-frequency measurement

- Only non-DRX

- without SSB time index detection

Proposal 2: Define the test cases with the following general configurations:

- Only inter-frequency measurement

- Only non-DRX

- without SSB time index detection

The supported concurrent gaps combinations are different for per-UE gap and per-FR gap. When UE doesn’t support per-FR gap, the configured two gaps should be per-UE gaps. However, when UE supports per-FR gap, the supported gap combinations are as follow.  Gap combination #3, #4, #5 will be only applied when per-UE gap is associated to positioning measurement.

Table 1. The Gap Combination Configurations for UE supporting per-FR gap

| Gap Combination Configuration Id | The number of simultaneous configured measurement gap patterns |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 measurement gap | Per-FR2 measurement gap | Per-UE measurement gap |
| 0 | 2 | 1 | 0 |
| 1 | 1 | 2 | 0 |
| 2 | 0 | 0 | 2 |
| 3Note 1 | 1 | 0 | 1 |
| 4Note 1 | 0 | 1 | 1 |
| 5Note 1 | 1 | 1 | 1 |
| 6 | 2 | 0 | 0 |
| 7 | 0 | 2 | 0 |
| Note 1: Gap Combination Configuration Id #3, #4, #5 will be only applied when the per-UE measurement gap is associated to measure PRS for any RSTD, PRS-RSRP, and UE Rx-Tx time difference measurement defined in TS 38.215 [4]. |  |  |  |

In legacy Rel-15, the serving cell and the target cell combinations for inter-frequency measurement are as follow.

- FR1 – FR1

- FR2 – FR2

- FR1 – FR2

To tradeoff the burden of test and the test coverage, we prefer to consider the following FR combinations with gap combination configuration.

| Test index | Cell layout | Gap Combination Configuration Id |
| --- | --- | --- |
| #0 | PCell FR1 – target cell 1 NR FR1, target cell 2 NR FR1 | #6 |
| #1 | PCell FR1 – target cell 1 NR FR1, target cell 2 LTE | #6 |
| #2 | PCell FR1 – target cell 1 NR FR1, target cell 2 NR FR1(positioning) | #3 |
| #3 | PCell FR2 – target cell 1 NR FR2, target cell 2 NR FR2 | #7 |
| #4 | PCell FR2 – target cell 1 NR FR2, target cell 2 NR FR2(positioning) | #4 |

Proposal 3: When UE also supports per-FR gap, define concurrent gaps test cases for the following cell configuration.

| Test index | Cell layout | Gap Combination Configuration Id |
| --- | --- | --- |
| #0 | PCell FR1 – target cell 1 NR FR1, target cell 2 NR FR1 | #6 |
| #1 | PCell FR1 – target cell 1 NR FR1, target cell 2 LTE | #6 |
| #2 | PCell FR1 – target cell 1 NR FR1, target cell 2 NR FR1(positioning) | #3 |
| #3 | PCell FR2 – target cell 1 NR FR2, target cell 2 NR FR2 | #7 |
| #4 | PCell FR2 – target cell 1 NR FR2, target cell 2 NR FR2(positioning) | #4 |

Another important aspect in concurrent gaps is the overlapping rule. We have the following suggestions for overlapping related configurations.

- Two measurement gaps have different MGRPs to avoid the fully overlapping scenario.

- To verify the dropping rule, RAN4 at least to define one test case with two measurement gaps’ offset less than 4ms.

- NW should configure different priorities for different MGPs

Proposal 4: To verify the overlapping rule, define the test cases with the following configurations:

- Two measurement gaps have different MGRPs to avoid the fully overlapping scenario.

- To verify the dropping rule, RAN4 at least to define one test case with two measurement gaps’ offset less than 4ms.

Based on the discussion before, we summarize the suggested test configuration as follow.

Proposal 5: The test case list for concurrent gaps is as follow.

| Test category | FR | Cell layout | Per-FR gap combination | DRX | SSB index reading | MGP1 | MGP1 association | MGP2 | MGP2 association |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-frequency measurement without SSB index reading | FR1 | PCell FR1 – target cell 1 NR FR1, target cell 2 NR FR1 | #6 | Non-DRX | no | MGP: #4 MG offset: 0 MG priority:1 | SSB | MGP: #0 MG offset: 10 MG priority:0 | SSB |
| Inter-frequency measurement without SSB index reading with inter-RAT measurement | FR1 | PCell FR1 – target cell 1 NR FR1, target cell 2 LTE | #6 | Non-DRX | no | MGP: #4 MG offset: 0 MG priority:1 | SSB | MGP: #0 MG offset: 4 MG priority:0 | Inter-RAT |
| Inter-frequency measurement without SSB index reading with NR positioning measurement | FR1 | PCell FR1 – target cell 1 NR FR1, target cell 2 NR FR1(positioning) | #3 | Non-DRX | no | MGP: #0 MG offset: 0 MG priority:1 | SSB | MGP: #24 MG offset: 5 MG priority:0 | positioning |
| Inter-frequency measurement without SSB index reading | FR2 | PCell FR2 – target cell 1 NR FR2, target cell 2 NR FR2 | #7 | Non-DRX | no | MGP: #4 MG offset: 0 MG priority:1 | SSB | MGP: #13 MG offset: 10 MG priority:0 | SSB |
| Inter-frequency measurement without SSB index reading with NR positioning measurement | FR2 | PCell FR2 – target cell 1 NR FR2, target cell 2 NR FR2(positioning) | #4 | Non-DRX | no | MGP: #13 MG offset: 0 MG priority:1 | SSB | MGP: #24 MG offset: 5 MG priority:0 | positioning |
| Note 1: MG priority indication #0 is higher than indication #1. |  |  |  |  |  |  |  |  |  |

# Conclusion

In the contribution, we discuss the open issues for concurrent gap. We have the following proposals:

Proposal 1: Define concurrent gaps test cases to cover at least the following usage combinations:

- SSB-based L3 measurement + SSB-based L3 measurement

- SSB-based L3 measurement + LTE inter-RAT measurement

- SSB-based L3 measurement + NR positioning measurement

Proposal 2: Define the test cases with the following general configurations:

- Only inter-frequency measurement

- Only non-DRX

- without SSB time index detection

Proposal 3: When UE also supports per-FR gap, define concurrent gaps test cases for the following cell configuration.

| Test index | Cell layout | Gap Combination Configuration Id |
| --- | --- | --- |
| #0 | PCell FR1 – target cell 1 NR FR1, target cell 2 NR FR1 | #6 |
| #1 | PCell FR1 – target cell 1 NR FR1, target cell 2 LTE | #6 |
| #2 | PCell FR1 – target cell 1 NR FR1, target cell 2 NR FR1(positioning) | #3 |
| #3 | PCell FR2 – target cell 1 NR FR2, target cell 2 NR FR2 | #7 |
| #4 | PCell FR2 – target cell 1 NR FR2, target cell 2 NR FR2(positioning) | #4 |

Proposal 4: To verify the overlapping rule, define the test cases with the following configurations:

- Two measurement gaps have different MGRPs to avoid the fully overlapping scenario.

- To verify the dropping rule, RAN4 at least to define one test case with two measurement gaps’ offset less than 4ms.

Proposal 5: The test case list for concurrent gaps is as follow.

| Test category | FR | Cell layout | Per-FR gap combination | DRX | SSB index reading | MGP1 | MGP1 association | MGP2 | MGP2 association |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-frequency measurement without SSB index reading | FR1 | PCell FR1 – target cell 1 NR FR1, target cell 2 NR FR1 | #6 | Non-DRX | no | MGP: #4 MG offset: 0 MG priority:1 | SSB | MGP: #0 MG offset: 10 MG priority:0 | SSB |
| Inter-frequency measurement without SSB index reading with inter-RAT measurement | FR1 | PCell FR1 – target cell 1 NR FR1, target cell 2 LTE | #6 | Non-DRX | no | MGP: #4 MG offset: 0 MG priority:1 | SSB | MGP: #0 MG offset: 4 MG priority:0 | Inter-RAT |
| Inter-frequency measurement without SSB index reading with NR positioning measurement | FR1 | PCell FR1 – target cell 1 NR FR1, target cell 2 NR FR1(positioning) | #3 | Non-DRX | no | MGP: #0 MG offset: 0 MG priority:1 | SSB | MGP: #24 MG offset: 5 MG priority:0 | positioning |
| Inter-frequency measurement without SSB index reading | FR2 | PCell FR2 – target cell 1 NR FR2, target cell 2 NR FR2 | #7 | Non-DRX | no | MGP: #4 MG offset: 0 MG priority:1 | SSB | MGP: #13 MG offset: 10 MG priority:0 | SSB |
| Inter-frequency measurement without SSB index reading with NR positioning measurement | FR2 | PCell FR2 – target cell 1 NR FR2, target cell 2 NR FR2(positioning) | #4 | Non-DRX | no | MGP: #13 MG offset: 0 MG priority:1 | SSB | MGP: #24 MG offset: 5 MG priority:0 | positioning |
| Note 1: MG priority indication #0 is higher than indication #1. |  |  |  |  |  |  |  |  |  |

# Reference

[1] R4-2202603, “WF on R17 NR MG enhancements - Multiple concurrent and independent MG patterns”, MediaTek Inc.

[2] R4-2202753, “Draft Big CR: RRM requirements Rel-17 NR MG enhancements”, Intel, MediaTek Inc.