3GPP TSG-RAN WG4 Meeting#103-e                	R4-2209526

Electronic Meeting, 9 – 20 May 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On RRM Tests for Rel-17 FR2 HST scenarios

Agenda item:	9.8.3

Document for:	Discussion

Introduction

In this contribution, we discuss the necessary test cases for RRM requirements which are enhanced for Rel-17 FR2 HST scenarios with a UE speed up to 350 km/h.

Discussion

In Table 1, we list relevant test cases along with configuration parameters.

Table 1: Proposed list of test cases and associated parameter settings

| Core requirement | Test case | Parameter |
| --- | --- | --- |
| Intrafrequency cell reselection in idle mode Pcell | Intrafrequency cell reselection | DRX cycle = 320 ms SMTC period = 20 ms Propagation condition: AWGN (serving cell) and AWGN with 9722 Hz frequency offset (neighbour cell) |
| Intrafrequency measurements in connected mode Pcell: - intrafrequency measurements without gaps - intrafrequency measurements with gaps | Intrafrequency event triggered reporting test without gap under DRX | DRX cycle = 40 ms SMTC period = 20 ms Propagation condition: AWGN (serving cell) and AWGN with 9722 Hz frequency offset (neighbour cell) |
| RRC re-establishment | Intrafrequency RRC re-establishment | SMTC period = 20 ms Propagation condition: AWGN |
| UE transmit timing: - Gradual timing adjustment - One-shot large UL timing adjustment for FR2 power class 6 UE | UE transmit timing tests verify the UE initial transmit timing accuracy, and the maximum amount of timing change in one adjustment according to the gradual timing adjustment and the one-shot large UL timing adjustment | DRX = off SMTC period = 20 ms Propagation condition: AWGN - DL timing adjustment value = 10*64*Tc - One-shot large UL timing adjustment Told-Tnew = 76*64*Tc |
| SSB-based radio link monitoring | - Radio link monitoring out-of-sync test for FR2 PCell configured with SSB-based RLM RS in DRX mode - Radio link monitoring in-sync test for FR2 PCell configured with SSB-based RLM RS in DRX mode | SMTC period = 20 ms DRX cycle = 40 ms Propagation condition: FFS |
| SSB-based beam failure detection | Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode | DRX cycle = 40 ms SMTC period = 20 ms Propagation condition: FFS |
| MAC-CE based TCI state switch delay | PCell active TCI state switch for a known TCI state | DRX cycle = 40 ms SMTC period = 20 ms Propagation condition: AWGN |
| L1-RSRP Reporting | SSB based L1-RSRP measurement when DRX is used | DRX cycle = 40 ms SMTC period = 20 ms Propagation condition: AWGN with 9722 Hz frequency offset |
| L1-SINR reporting | L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used | DRX cycle = 40 ms SMTC period = 20 ms Propagation condition: AWGN with 9722 Hz frequency offset |

Conclusion

In this contribution, we have discussed the necessary tests for RRM enhancements for FR2 HST scenarios and made the following proposal:

RAN4 discusses and agrees with the proposed list of RRM tests in Table 1 for the FR2 HST RRM enhanced requirements.

References

[1]    R4-2206848, WF on FR2 HST RRM (part 1), Nokia, Nokia Shanghai Bell