3GPP TSG-RAN WG4 Meeting#103-e                               R4-2209586

E-meeting, May 9 – May 20,2022

Agenda item:	9.15.4.1

Source: 	ZTE Corporation

Title: 	Further Discussion on BS Tx requirements for 52.6-71GHz

Document for:	Approval

Introduction

In the last RAN4 meeting, there were some progress on BS Tx requirements for 52.6-71GHz, however there are still some remaining issue left for further final confirmation even with extensive discussions before. In this contribution, we want to share some further inputs on this issue.

Discussion

## 2.1. TAE

For intra-band non-contiguous CA TAE requirements, it’s still open in BS RF session depending on the outcome of main session and RRM session agreed as following:

Intraband non-contiguous CA TAE: - Agree that 260ns requirement is applicable for intraband non-contiguous CA TAE. - FFS whether to specify such requirements to BS specification pending on the conclusion in main and RRM sessions for the supporting on non-contiguous CA in Rel-17.

Based on the agreement of RAN#95e meeting in [3], intra-band non-contiguous CA within FR2-2 is down-scoped for the sake for progress, therefore we think that corresponding TAE requirement should also been removed in Rel-17.

Final Proposal 3 (agreed): down-scope intra-band non-contiguous CA within FR2-2 from Rel-17.

Proposal 1: to remove the TAE requirements for intra-band non-contiguous CA TAE requirement in the endorsed draft CR R4-2207220.

## 2.2. EVM

In the last RAN4 meeting, there were some agreement [4] reached in main session for spectral utilization for FR2-2 as following. Based on that agreement, we could derive the corresponding FFT size as following:

- Put 50% EVM window length within [] until conformance test feasibility issues are resolved. - Note: The value on EVM window length can be further discussed and updated if needed. - FFT sizes to be also double checked by companies.

Agreement from R4-2206536

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 66 | 264 | N/A | N/A | N/A |
| 480 | N/A | 66 | 132 | 264 | N/A |
| 960 | N/A | 33 | 66 | 132 | [156] |

Table 3 : EVM window length for normal CP, FR2, 120 kHz SCS for FR2-2 [the same as table 1 for FR2-1]

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (Note) (%) | RB number | FFT utilization | SU |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 1024 | 72 | 36 | 50 | 66 | 0.77 | 0.9504 |
| 400 | 4096 | 288 | 144 | 50 | 264 | 0.77 | 0.9504 |

Table 4 : EVM window length for normal CP, FR2, 480 kHz SCS for FR2-2

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (Note) (%) | RB number | FFT utilization | SU |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 400 | 1024 | 72 | 36 | 50 | 66 | 0.77 | 0.9504 |
| 800 | 2048 | 144 | 72 | 50 | 132 | 0.77 | 0.9504 |
| 1600 | 4096 | 288 | 144 | 50 | 264 | 0.77 | 0.9504 |

Table 5 : EVM window length for normal CP, FR2, 960 kHz SCS for FR2-2

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (Note) (%) | RB number | FFT utilization | SU |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 400 | 512 | 36 | 18 | 50 | 33 | 0.77 | 0.9504 |
| 800 | 1024 | 72 | 36 | 50 | 66 | 0.77 | 0.9504 |
| 1600 | 2048 | 144 | 72 | 50 | 132 | 0.77 | 0.9504 |
| 2000 | 3072 | 216 | 108 | 50 | [156] | [0.61] | [0.89856] |

Conclusions

In this contribution, we want to further considerations on each BS Tx requirements for 60GHz and some proposals and observations are made as following:

Proposal 1: to remove the TAE requirements for intra-band non-contiguous CA TAE requirement in the endorsed draft CR R4-2207220.

Proposal 2: to propose define EVM window length as shown in Table 3, 4, 5.

References

- R4-2106113,WF on BS RF TX requirements for 52.6 – 71 GHz, approved.

- R4-2207216	WF on >52.6 GHz BS RF Tx Requirements, approved.

- RP-220951, Moderator's summary for discussion [95e-29-R17-52.6-71GHz-WI], Huawei, Noted.

[4] R4-2206536,WF on 60 GHz UE RF, approved.