3GPP TSG-RAN WG4 Meeting#103-e                               R4-2209602

E-meeting, May 9 – May 20,2022

Agenda item:	9.5.5.1.1

Source: 	ZTE Corporation

Title: 	Discussion on NR repeater Stimulus signals

Document for:	Approval

Introduction

Based on the approved Rel-17 RAN4 led repeater WID RP-212129, the conformance testing for both NR repeater should be completed in Sep, 2022. In this contribution, we want to provide some initial analysis how to configure the stimulus signals or test models for conformance testing of each repeater RF requirements.

Discussion

| FR1 conducted requirements | Stimulus signal/test model | Note |
| --- | --- | --- |
| Repeater output power | DL: NR-FR1-TM1.1 UL: IAB-MT-FR1-TM1.1 |  |
| Frequency stability Error Vector Magnitude | DL: NR-FR1-TM3.1 NR-FR1-TM3.1a NR-FR1-TM3.2 NR-FR1-TM3.3 UL: IAB-MT-FR1-TM3.1 IAB-MT-FR1-TM3.1a |  |
| Out of band gain | DL/UL: CW signal |  |
| Unwanted emissions, ACLR, spurious emission | DL: NR-FR1-TM1.1 UL: IAB-MT-FR1-TM1.1 |  |
| Input intermodulation | DL/UL: CW signal |  |
| Output intermodulation | DL: NR-FR1-TM 1.1 UL: IAB-MT-FR1-TM1.1 |  |
| Adjacent Channel Rejection Ratio (ACRR) | DL: NR-FR1-TM 1.1 UL: IAB-MT-FR1-TM1.1 | For the simplicity, E-UTRA and UTRA signal for conformance testing is not needed. |
| Transmit ON/OFF power | DL: NR-FR1-TM 1.1 UL: IAB-MT-FR1-TM1.1 |  |

| FR2 radiated requirements | Stimulus signal/test model | Note |
| --- | --- | --- |
| OTA Repeater output power | DL: NR-FR2-TM1.1 UL: IAB-MT-FR2-TM1.1 |  |
| OTA Frequency stability OTA Error Vector Magnitude | DL: NR-FR2-TM3.1 NR-FR2-TM3.1a UL: IAB-MT-FR2-TM3.1 IAB-MT-FR2-TM3.1a |  |
| OTA Out of band gain | DL/UL: CW signal |  |
| OTA Unwanted emissions, ACLR, spurious emission | DL: NR-FR2-TM1.1 UL: IAB-MT-FR2-TM1.1 |  |
| OTA Input intermodulation | DL/UL: CW signal |  |
| OTA Adjacent Channel Rejection Ratio (ACRR) | DL: NR-FR2-TM1.1 UL: IAB-MT-FR2-TM1.1 |  |
| OTA Transmit ON/OFF power | DL: NR-FR2-TM1.1 UL: IAB-MT-FR2-TM1.1 |  |

Conclusions

In this contribution, we provide some initial analysis how to configure the stimulus signals or test models for conformance testing of each repeater RF requirements.

References

- TS 38.106 17.0.0

- TS 36.143 15.0.0