3GPP TSG-RAN WG4 Meeting#103-e                               R4-2209606

E-meeting, May 9 – May 20,2022

Agenda item:	9.5.5.3

Source: 	ZTE Corporation

Title: 	Discussion on FR2 NR repeater test configuration

Document for:	Approval

Introduction

In last RAN4 meeting, Core requirement of NR repeater has been completed. Based on the approved Rel-17 RAN4 led repeater WID RP-212129, the conformance testing for both NR repeater should be completed in Sep, 2022. In this contribution, we provide some initial views on conformance testing requirement for FR2 NR repeater.

Discussion

- 2.1. Test setup for FR2 NR repeater

For the conformance testing for NR repeater, the 1st issue need to be addressed is how to establish the measurement setup for NR repeater conformance testing. Since for FR2 NR repeater type 1-C is similar as LTE repeater or UTRA repeater, therefore we propose the use the measurement setup in TS 38.141-2 as baseline with some updates for FR2 NR repeater as following:

### 2.1.1. Test setup for FR2 NR repeater

C.1	OTA output power EIRP, OTA Frequency stability, OTA Error Vector Magnitude and OTA Transmit ON/OFF power

Figure C.1-1: Measuring system set-up for maximum EIRP testing

Note 1:  The repeater is a bi-directional device. The signal generator may need protection.

Note 2:  The OTA chamber shown in figure C.1-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, Near field chamber, etc.)

C.2	Out of band gain

Figure C.2-1: Measuring system set-up for out of band gain

Note 1:  That repeater is a bi-directional device. The signal generator may need protection.

Note 2:  The OTA chamber shown in figure C.2-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, Near field chamber, etc.)

C.3	Unwanted emission: Operating band unwanted emission and ACLR

Figure C.3-1: Measuring system set-up for unwanted emission: ACLR and Operating band unwanted emission

Note 1:  That repeater is a bi-directional device. The signal generator may need protection.

Note 2:  The OTA chamber shown in figure C.3-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, Near field chamber, etc.)

C.4	Unwanted emission: Spurious emission

Figure C.4-1: Measuring system set-up for unwanted emission: Spurious emission.

Note 1:  That repeater is a bi-directional device. The signal generator may need protection.

Note 2:  The OTA chamber shown in figure C.4-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc.).

C.5	Input intermodulation

Figure C.5-1: Measuring system set-up for input intermodulation.

Note 1:  That repeater is a bi-directional device. The signal generator may need protection.

Note 2:  The OTA chamber shown in figure C.5-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, Near field chamber, etc.)

C.6	Adjacent Channel Rejection Ratio

Figure C.6-1: Measuring system set-up for Adjacent Channel Rejection Ratio

Note 1:  That repeater is a bi-directional device. The signal generator may need protection.

Note 2:  The OTA chamber shown in figure C.6-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, Near field chamber, etc.)

Proposal 1: propose to use the above measurement setup from C.1 to C.6 as baseline for FR2 NR repeater;

### 2.1.2. Potential testing concern

For FR2 repeater, since its need to receive the input signal firstly, then amplify and forward its received signal to the target node. There was some concerns raised in the past that input signal generator within the chamber would have some impacts on repeater output signal measurement. From our understanding, it is true that input signal for repeater placed within the chamber could also been detected by measurement equipment, however it should be known that input signal should be quite lower than compared with output power if repeater is set to the maximum gain mode. In addition, as described in the previous section, if input signal generator could form very narrow beam towards the repeater, then its transmission would also been blocked a lot without causing the impact on reception at the measurement equipment.

Observation: input signal generator for the repeater within chamber should have marginal impact on the reception of repeater output since its power should be much lower compared with repeater output power.

- 2.2. Measurement uncertainty

For the measurement uncertainty for FR2 repeater,we proposed to

| Subclause | Maximum Test System Uncertainty and Range over which Test System Uncertainty applies | Derivation of Test System Uncertainty |
| --- | --- | --- |
| OTA Output power | EIRP: Normal condition: ±1.7 dB (24.25 – 29.5 GHz) ±2.0 dB (37 – 43.5 GHz) Extreme condition: ±3.1 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) TRP: ±2.1 dB (24.25 – 29.5 GHz) ±2.4 dB (37 – 43.5 GHz) | Take FR2 BS Radiated transmit power and OTA base station output power MU as baseline |
| OTA Frequency stability | ±12 Hz | Take FR2 BS freq error MU as baseline |
| OTA Out of band gain | TBD Calibration of test set-up shall be made without DUT in order to achieve the accuracy | TBD |
| OTA Operating band unwanted emission | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) The interference from the signal generator ACLR shall be minimum 10 dB below that of a Base Station according toTS38.141-2 [xx] | Take FR2 BS UEM MU as baseline |
| OTA ACLR | Relative ACLR: ±2.3 dB (24.25 – 29.5 GHz) ±2.6 dB (37 – 43.5 GHz) Absolute ACLR: ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) The interference from the signal generator ACLR shall be minimum 10 dB below that of a Base Station according toTS38.141-2 [xx] | Take FR2 BS ACLR MU as baseline |
| OTA Spurious emissions | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz The interference from the signal generator ACLR shall be minimum 10 dB below that of a Base Station according to TS38.141-2 [xx] | Take FR2 BS ACLR MU as baseline |
| OTA Error vector magnitude | 1% signal analyser, 2% stimulus signal | Take FR1 repeater EVM MU as baseline |
| OTA Input intermodulation | ±3.9 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.9 dB, 37 GHz < f ≤ 43.5 GHz | Take FR2 BS Rx intermodulation MU as baseline |
| OTA Adjacent channel rejection ratio | 0,7 dB | Take FR1 repeater ACRR MU as baseline |
| OTA Transmit OFF power | ±2.9 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) | Take FR2 BS OFF MU as baseline |
| OTA Transmitter transient period | NA |  |

- 2.3. Test procedure

Regarding the test procedures for FR2 NR repeater, the existing test procedures for LTE repeater and FR2 NR BS conformance testing could be used as baseline. In addition, some additional requirements for FR2 NR based repeater should also been taken into account e.g. non-contiguous spectrum or multi-band operation, transmitter ON/OFF power etc.

Conclusions

In this contribution, we want to share some further considerations on those remaining open issues of conducted repeater and proposals are made as following:

Proposal 1: propose to use the above measurement setup from C.1 to C.6 as baseline for FR2 NR repeater;

Observation: input signal generator for the repeater within chamber should have marginal impact on the reception of repeater output since its power should be much lower compared with repeater output power.

Proposal 2: propose to use the measurement uncertainty in section 2.2 as baseline and encourage more inputs from TE vendors.

Proposal 3: propose to use the existing test procedure of LTE repeater as baseline and take the additional requirements defined for FR2 NR based repeater into account. e.g.non-contiguous spectrum, transmitter ON/OFF power etc.

References

- TS 38.106 17.0.0

- TS 36.143 17.0.0