3GPP TSG-RAN WG4 Meeting #103-e                                                                                                                                R4-2209633

Electronic Meeting, May 09 - May 20, 2022

Source:	ZTE Corporation

Title:	TP for TR 37.717-21-11: DC_1A-38A_n7A

Agenda Item:	8.12.2

Document for:	Approval

- Introduction

This contribution provides a text proposal on DC_1A-38A_n7A for TR 37.717-21-11[1].

- Reference

- TR 37.717-21-11 Rel-17 Dual Connectivity (DC) of 2 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) v0.8.0

- Text Proposal

----- Start of TP -----

5.x	DC_1-38_n7

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_1A-38A_n7A17,18 | N/A | 1A | N/A |
| NOTE 17: The combination is not used alone as fall back mode of other band combinations. NOTE 18: Power imbalance between downlink carriers on Band 7 and Band 38 or band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

### 5.x.3	Co-existence studies

No need to analysis the co-existence studies for this configuration.

### 5.x.3	∆TIB and ∆RIB values

For DC_1-38_n7, same TIB,c and RIB,c values of  DC_1-7_n38 are reused which are given in the tables as below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-38_n7 | 1 | 0.5 |
|  | 38 | 0 |
|  | n7 | 0 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-38_n7 | 1 | 0 |
|  | 38 | 0.2 |
|  | n7 | 0 |

### 5.x.4	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

----- End of TP -----