3GPP TSG-RAN WG4 Meeting #103                                                                   R4-2209644

E-meeting, 9th – 20th May, 2022

Agenda item:				8.21.2.2

Source:				CableLabs, Charter Communications

Title:			NR-U Triple Punctured Channel SEM for 100 MHz Bandwidth

Document for:			Approval

# Introduction

RAN4 #102-e meeting agreed on adding the 100 MHz bandwidth for NR-U in Rel-17, and agreed on the 100 MHz NR-U channel raster [1] and [2]. The new 100 MHz bandwidth introduces the interior triple punctured case. The corresponding triple punctured spectrum emission mask (SEM) will need to be defined. The candidate SEMs are -28 dBr at 10 MHz from the punctured channel edges [3], -25 dBr at 10 MHz from the puctured channel edges that follows the ETSI/BRAN decisions [4], and -25 dBr at 15 MHz from the edges [5] and [6].

# Proposed Triple Punctured SEM for 100 MHz Bandwidth

CableLabs conducted a comprehensive simulation analysis in [7] that compared three SEMs: (1) -31 dBr at 20 MHz from the edges; (2) -28 dBr at 10 MHz from the edges; and (3) -25 dBr at 10 MHz from the edges. SINR degradation of a NR-U network (victim) or Wi-Fi device using the punctured channels are estimated. The -25 dBr SEM of the aggressor network generates significant SINR and throughput degradation on the victim network. Note that the -25 dBr at 15 MHz SEM proposed in [4] is even worse than the SEM (3) we analyzed. The -28 dBr SEM is a good compromise for the aggressor RF design and coexistence. We would urge RAN4 to consider the -28 dBr SEM as described below:

Proposal 1: -28 dBr at 10 MHz from the edges. The SEM decreases from 0 dBr at the channel edges (-30 and 30 MHz) to -20 dBr at 1 MHz from the edges (-29 and 29 MHz), then decreases to -28 dBr at 10 MHz from the channel edges (-20 and 20 MHz), the SEM has a flat bottom of -28 dBr between -20 and 20 MHz (see Figure 1 and Table 1).

Figure 1. Definition of points A to F in SEM.

Table 1. NR-U Triple Punctured SEM for 100 MHz Bandwidth.

|  | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| ∆Frequency (MHz) | -30 | -29 | -20 | 20 | 29 | 30 |
| ∆Power (dBr) | 0 | -20 | -28 | -28 | -20 | 0 |

ETSI/BRAN adopted a SEM with -25 dBr floor in the punctured channels [4]. The SEM reduces to -25 dBr at half of the transmission channel bandwidth. In the triple punctured case, the channel bandwidth used for transmission is 20 MHz. The SEM reduces to -25 dBr at 10 MHz from the punctured channel edges.

Proposal 2: -25 dBr at 10 MHz from the edges. The SEM decreases from 0 dBr at the channel edges (-30 and 30 MHz) to -20 dBr at 1 MHz from the edges (-29 and 29 MHz), then decreases to -25 dBr at 10 MHz from the channel edges (-20 and 20 MHz), the SEM has a flat bottom of -25 dBr between -20 and 20 MHz (see Figure 1 and Table 2).

Table 1. NR-U Triple Punctured SEM for 100 MHz Bandwidth.

|  | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| ∆Frequency (MHz) | -30 | -29 | -20 | 20 | 29 | 30 |
| ∆Power (dBr) | 0 | -20 | -25 | -25 | -20 | 0 |

# Conclusion

There are two SEM options could be considered for the NR-U 100 MHz bandwidth channel with triple punctures. We prefer proposal 1.

Proposal 1: -28 dBr at 10 MHz from the edges.

Proposal 2: -25 dBr at 10 MHz from the edges.

# References

[1] R4-2206447, “WF on adding 100 MHz channel BW in NR-U bands n46 and n96,” Qualcomm, RAN4 #102-e, Mar. 2022.

[2] R4-220xxxx, “Email discussion summary for [102-e][113] NR_bands_R17_BWs,” Ericsson, Mar. 2022.

[3] R4-2119301, “NR-U Punctured Channel SEM for 100 MHz Bandwidth,” CableLabs and Charter Communications, RAN4 #101-e, Nov. 2021.

[4] “6 GHz RLAN; NR-U Harmonised Standard for access to radio spectrum”, ETSI, Draft ETSI EN 303 687 V0.0.13, June 2021.

[5] R4-2119197, “Further discussion on the introduction of 100MHz for NR-U,” ZTE, RAN4-#101-e, Nov. 2021.

[6] R4-2117879, “100 MHz channel bandwidth for NR-U,” Huawei and HiSilicon, RAN4-#101-e, Nov. 2021.

[7] R4-2112031, “NR-U Punctured Channel SEM for 100 MHz Bandwidth,” CableLabs and Charter Communications, RAN4 #100-e, Aug. 2021.