3GPP TSG-RAN WG4 Meeting #103-e	R4-2209691

Electronic Meeting, 9 May. – 20 May 2022

Agenda Item:	9.12.8.3.1

Source:	Ericsson

Title:	Discussion on the PDSCH requirement for NTN

Document for:	Discussion

# 1	Introduction

In the last RAN4 meeting, companies have detailed discussions on the non-terrestrial network demodulation requirements. A WF [1] was agreed to capture all agreements for UE PDSCH demodulation requirement issues.

Issue 3-2-3: Enhancement on HARQ - Proposals: Do you agree to just verify the functionality with disabled HARQ, e.g., schedule a low code rate - Option 1: Yes - Option 2: No (please specify the reasons if any) - Agreement: - Disable HARQ with number of re-Tx set to 1 to avoid defining a special test as the start point Issue 3-2-4: SCS/CBW set for PDSCH requirements - Proposals - Option 1: Only consider 15kHz SCS/10MHz - Option 2: In addition to 15kHz SCS/10MHz, need to further consider 30kHz SCS: 20MHz Moderator’s note: Do we need to align the SCS/CBW set for UL and DL? - Agreement: - Select 15kHz SCS/10MHz, further discuss whether to consider 30kHz SCS/ 20MHz Issue 3-2-5: Modulation order for PDSCH requirements - Proposals - Option 1: Only consider QPSK and 16QAM - Option 2: In addition to QPSK and 16QAM, need to further 64QAM - Agreement: - Consider QPSK and 16QAM, further discuss whether to consider 64QAM.

Meanwhile, it was also agreed on no PBCH/PDCCH/CSI reporting requirement for NTN UE demodulation.

Issue 3-3-1: Whether to define the PBCH requirements - Agreement: - Do not define PBCH requirements Issue 3-3-2: Whether to define the PDCCH requirements - Agreement: - Do not define PDCCH requirements Issue 3-4-1: CSI reporting requirements - Agreement: - Do not define CSI reporting requirements

In this contribution, we mainly discussed the left open issues for UE PDSCH requirement. All other general issues please refer to our correlated contribution [2].

# 2	Discussion

## 2.1 Scenarios

During the last RAN4 meeting’s discussion, companies had different views on whether to define PDSCH requirement for both GEO and LEO requirements. Followings are candidate options listed in the WF [1]:

- Proposals - Option 1: Only define requirements for LEO - Option 2: Define requirements for GEO and LEO separately - Option 3: Define one set requirements which are applicable for LEO and GEO (Moderator’s note: please explain how to define one requirement to apply for LEO and GEO if select this option)

From the table in TS 38.811, comparing GEO and LEO in S band, we observed that the altitude, delay, doppler shift are probably the most significant differences that are related to RAN4.

Table 5.3.5-1: Summary of Doppler shift, Doppler Shift variation and propagation delay for LEO at 600 km, GEO and HAPS

|  | Deployment-D1 | Deployment-D2 | Deployment-D3 | Deployment-D4 | Deployment-D5 | Cellular ( 10 km Radius) |
| --- | --- | --- | --- | --- | --- | --- |
| Platform orbit and altitude when relevant | GEO at 35 786 km | GEO at 35 786 km | Non-GEO down to 600 km | Non-GEO down to 600 km | Airborne vehicle up to 20 km |  |
| Frequency band | Ka band | S band | S band | Ka band | S band (Below 6 GHz) | S band |
| Max One way Propagation delay (ms) | Bentpipe: 272.37 ms gNB on board: 135.28 ms | 272.37 ms | 14.204 ms | 14.204 ms | 1.526 ms | 0.03333 ms |
| Max Differential delay (ms) | 16 (between Edge of satellite coverage and Nadir) | 16 (between Edge of satellite coverage and Nadir) | 4.44 (between Edge of satellite coverage and Nadir) | 4.44 (between Edge of satellite coverage and Nadir) | 0.697 (between Edge of satellite coverage and Nadir) | 0.00333(between cell centre and cell edge) equal to maximum delay |
| Max Doppler shift in kHz | For plane @ 20 GHz: +/- 18.51 kHz @30 GHz: +/- 27.7 kHz | For plane 1.851 kHz @ 20 GHz | +/- 48 kHz | @20 GHz : +/- 480 kHz @30 GHz : +/- 720 kHz | @ 2 GHz: +/- 100 Hz mainly due to platform motion | In case of UE on board a high speed train: +/- 925 Hz |
| % of the carrier frequency (Ratio of Doppler Shift over the central signal frequency | 10-4 % | 10-4 % | 0.0024% | 0.0024% |  |  |
| Max Doppler variation in Hz/s. | Negligible | Negligible | -544 Hz/s @ 2 GHz | -5.44 kHz/s @ 20Ghz (Downlink) -8.16 kHz/s @30 GHz (uplink) | Negligible | Negligible |

NOTE :	In some cases like UE on board an aircraft during taking off, the acceleration can add a supplementary Doppler variation in absolute value of respectively 262 Hz/s @ 20 Ghz, and 393 Hz/s @ 30 GHz.

While, given that the max one-way delay and max differential delay are not going to affect the demodulation performance, it is a little bit redundant to have two test cases just to verify different Doppler shift values. Considering that the UE will have rather the same behaviour on the demodulation processing, but worse performance due to high satellite motion velocity for LEO, we propose to have just one test case (one requirement) for LEO only.

However, the applicability rule is necessary as there are two different UE capabilities for GSO and NGSO. Here is the description from The TS38.306/331 V17.0.0:

| nonTerrestrialNetwork-r17 Indicates whether the UE supports NR NTN access. If the UE indicates this capability the UE shall support the following NTN essential features, i.e., timer extension in MAC/RLC/PDCP layers and RACH adaptation to handle long RTT, acquiring NTN specific SIB and more than one TAC per PLMN broadcast in one cell. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| ntn-ScenarioSupport-r17 Indicates whether the UE supports the NTN essential features in GSO scenario or NGSO scenario. If a UE does not include this field but includes nonTerrestrialNetwork-r17, the UE supports the NTN essential features for both GSO and NGSO scenarios, and also supports mobility between GSO and NGSO scenarios. Editor's Note: FFS on if ntn-ScenarioSupport-r17 also indicates all NTN optional features UE indicates have been supported in the corresponding scenario(s) | UE | No | No | No |

According to the description of UE capability ‘nonTerestrialNetwork-r17’ and ‘ntn-ScenarioSupport-r17’, if a UE supports ‘nonTerestrialNetwork-r17’ but has no field for ‘ntn-ScenarioSupport-r17’, then it indicates a support on both GSO and NGSO. If a UE indicates its support on ‘nonTerestrialNetwork-r17’ and ‘ntn-ScenarioSupport-r17’, then it is clear what exact scenario it can support. If a UE indicates its support on both ‘nonTerestrialNetwork-r17’ and ‘ntn-ScenarioSupport-r17’, then this UE can not support NTN.

Considering that the channel model for GEO is similar to that of TN scenario, UEs that pass the TN demodulation requirements are expected to pass the NTN GEO test (if any). But LEO is quite different from TN. In this case, we propose to have the following applicability rule along with only one requirement of LEO scenario for NTN PDSCH:

Table 2.1-1 Proposed applicability rule for one additional LEO test

| UE-NR-Capability-v1700 |  | Applicability |
| --- | --- | --- |
| nonTerrestrialNetwork-r17 | ntn-ScenarioSupport-r17 |  |
| Supported | GSO and NGSO | UE needs to pass the additional LEO test and TS38.101-4 requirements |
|  | GSO only | UE needs to pass TS38.101-4 requirements only |
|  | NGSO only | UE needs to pass the additional LEO test and TS38.101-4 requirements |
|  | N/A | UE needs to pass the additional LEO test and TS38.101-4 requirements |
| Not supported | N/A | UE needs to pass TS38.101-4 requirements only |

Therefore, we propose the following:

Proposal 1: Introduce one requirement for LEO only with the following applicability rules:

| UE-NR-Capability-v1700 |  | Applicability |
| --- | --- | --- |
| nonTerrestrialNetwork-r17 | ntn-ScenarioSupport-r17 |  |
| Supported | GSO and NGSO | UE needs to pass the additional LEO test and TS38.101-4 requirements |
|  | GSO only | UE needs to pass TS38.101-4 requirements only |
|  | NGSO only | UE needs to pass the additional LEO test and TS38.101-4 requirements |
|  | N/A | UE needs to pass the additional LEO test and TS38.101-4 requirements |
| Not supported | N/A | UE needs to pass TS38.101-4 requirements only |

# 3	Summary

To summarize, here is our proposal:

Proposal 1: Introduce one requirement for LEO only with the following applicability rules:

| UE-NR-Capability-v1700 |  | Applicability |
| --- | --- | --- |
| nonTerrestrialNetwork-r17 | ntn-ScenarioSupport-r17 |  |
| Supported | GSO and NGSO | UE needs to pass the additional LEO test and TS38.101-4 requirements |
|  | GSO only | UE needs to pass TS38.101-4 requirements only |
|  | NGSO only | UE needs to pass the additional LEO test and TS38.101-4 requirements |
|  | N/A | UE needs to pass the additional LEO test and TS38.101-4 requirements |
| Not supported | N/A | UE needs to pass TS38.101-4 requirements only |

# References

[1]	R4-2207464 WF on general and NTN UE demodulation requirements, Qualcomm Incorporated