3GPP TSG-RAN WG4 Meeting # 103-e	R4-2209778

# Electronic Meeting, May 09 - May 20, 2022

Agenda item:	9.15.10.2

Source:	Apple Inc.

Title:	Initial Simulation results for UE demod requirements for FR2-2

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#102-e demodulation requirements for 52.6 - 71 GHz was discussed and way forward [1] was agreed. In this contribution we present our initial simulation results UE demodulation requirements definition.

# 2. Simulation Results

Based on the agreed simulation assumptions in [1], we present initial simulation results for FR2-2 with 120KHz and 480 KHz SCS.

Study on phase noise impact for requirements derivation

Companies deliver two sets of ideal simulation results for requirement discussion. Result set#1 is without phase noise and set#2 is with phase noise.

- No Tx phase noise is modelled

- Rx Phase noise is modelled only to find feasible FRC configuration (i.e. achieve maximum throughput and loss in comparison to scenarios without Rx phase noise is less than 1 dB)

- 70 GHz carrier frequency is assumed

Phase noise model

Consider PN model set 1 and 2 in TR 38.808 for initial simulation results alignment.

PDSCH Simulation parameters:

Mapping type

Define PDSCH requirements with mapping type A

FFS define PDSCH test case to verify mapping type B processing.

MCS, modulation order for PDSCH requirements

Define PDSCH requirements with MCS 4, MCS 13 and MCS [22]

- Other MCS values are not precluded

Rank

Define PDSCH requirements with:

- Rank 1

  - FFS FD-OCC is not applied to all the antenna ports for DMRS

- FFS Rank 2

PTRS configuration

Consider PTRS configuration as K=2, L=1

Simulation Results with 120KHz SCS

TBA

Simulation Results with 480KHz SCS

TBA

# 3. Conclusion

In this paper, we provide our initial simulation results UE demodulation requirements definition.

# Reference

- R4-2207223, “WF on demodulation performance requirements definition for 52.6 - 71 GHz”, Intel.