3GPP TSG-RAN WG4 # 103-e	R4-2209798

Electronic Meeting, May 09 – May 20, 2022

Agenda item:	9.19.5.2.2

Source:	MediaTek Inc.

Title:	Simulation results and discussion on PDCCH requirements for RedCap

Document for:	Discussion

- Introduction

In RAN4#102-e, a way forward on RedCap was agreed [1]. In this contribution, we provide our view for the PDCCH requirements and present simulation results.

- Discussion

The simulation results in Table 2 and Table 4 are based on the test parameters in Table 1 and Table 3 respectively. For 20MHz TDD cases, it is also possible to reuse the CORESET duration 1 symbol as test number 2 in Table 5.3.2.2.1-1 and test number 1 in Table 5.3.2.2.2-1. Therefore, we suggest using CORESET duration as 1 to define test cases for FR1, 30kHz, TDD.

Observation 1: It is possible to configure CORESET 1 symbol duration for TDD in 20MHz channel bandwidth.

Proposal 1: Use 1 CORESET symbol to define test cases for TDD in 20MHz channel bandwidth.

Table 1. Test parameters for FDD

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| CCE to REG mapping type |  | nonInterleaved |  |
| REG bundle size |  | 6 |  |
| Shift index |  | 0 |  |

Table 2. Simulation results for FDD

| Rx number | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x1 | 1% | 5.3.2.1.1-1 Test 3 | 14.2 |
| 1Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x1 low | 1% | 5.3.2.1.2-1 Test 3 | 2.9 |

Table 3. Test parameters for TDD

| Parameter | Unit | 1 Tx Antenna |  |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR1.30-1 |  |  |  |
| CCE to REG mapping type |  | interleaved |  | interleaved |  |
| Interleaver size |  | 3 |  |  |  |
| REG bundle size |  | 2 | 2 |  |  |
| Shift Index |  | 0 |  |  |  |

Table 4. Simulation results for TDD

| Rx number | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 | 48 | 1 | 4 | TDLC300-100 | 1x1 | 1% | 5.3.2.2.1-1 Test 2 | 6.5 |
| 1Rx UE | 20 | 48 | 1 | 8 | TDLC300-100 | 2x1 low | 1% | 5.3.2.2.2-1 Test 1 | 0.91 |
| 2Rx UE | 20 | 48 | 1 | 4 | TDLC300-100 | 1x2 low | 1% | 5.3.2.2.1-1 Test 2 | 1.1 |
| 2Rx UE | 20 | 48 | 1 | 8 | TDLC300-100 | 2x2 low | 1% | 5.3.2.2.2-1 Test 1 | -2.9 |

- Conclusion

In this contribution, we provide simulation results for RedCap PDCCH requirements with the agreed simulation assumption in [1]. The proposals are summarized as below:

Observation 1: It is possible to configure CORESET 1 symbol duration for TDD in 20MHz channel bandwidth.

Proposal 1: Use 1 CORESET symbol to define test cases for TDD in 20MHz channel bandwidth.

- Reference

- R4-2203032, “WF on RedCap demodulation and CQI reporting requirements”, Ericsson