3GPP TSG-RAN WG4 # 103-e	R4-2209802

Electronic Meeting, May 09 – May 20, 2022

Agenda item:	9.6.4.3

Source:	MediaTek Inc.

Title:	Simulation results and discussion on the CQI requirements for 1024QAM

Document for:	Discussion

- Introduction

In RAN4#102-e, a way forward on DL1024QAM was agreed [1]. We provide our view for the remaining issues and present simulation results in this contribution.

- Discuss the final CQI definition requirements with CQI table 4 considering the impairment margin and SNR levels provided by TE with the assumption of Tx EVM 2.5%. - 2Rx UE: - This is the common understanding SNR corresponding to CQI index 14 for 1024QAM table is around 28-30dB for 2Rx UE without impairment margin. - Option 1: SNR = 28/29dB - Option 2: SNR = 29/30dB - 4Rx UE: - This is the common understanding SNR corresponding to CQI index 14 for 1024QAM table is around 26-28dB for 4Rx UE without impairment margin. - Option 1: SNR = 25/26dB - Option 2: SNR = 26/27dB

- Discussion

We provide our simulation results as below.

Table 1.	Summary for simulation results

|  | 2Rx |  |  |  |  | 4Rx |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR [dB] | Median CQI | Percentage of {median CQI-1, median CQI, median CQI+1} | BLER at Median CQI -1 for | BLER at median CQI | BLER at Median CQI +1 | Median CQI | Percentage of {median CQI-1, median CQI, median CQI+1} | BLER at Median CQI -1 for | BLER at median CQI | BLER at Median CQI +1 |
| 25 | 13 | 100% | 0% | 0% | 100% | 14 | 100% | 0% | 0% | 100% |
| 26 | 13 | 100% | 0% | 0% | 66.27% | 14 | 100% | 0% | 0% | 100% |
| 27 | 14 | 100% | 0% | 0% | 100% | 14 | 100% | 0% | 0% | 36.56% |
| 28 | 14 | 100% | 0% | 0% | 100% | 15 | 100% | 0% | 0% | - |
| 29 | 14 | 100% | 0% | 0% | 100% | 15 | 100% | 0% | 0% | - |
| 30 | 14 | 100% | 0% | 0% | 17.41% | 15 | 100% | 0% | 0% | - |

According to the simulation results, we propose to select 28/29dB for 2Rx and 25/26dB for 4Rx for 1024 QAM CQI reporting test case.

Proposal 1: Propose to select 28/29dB for 2Rx and 25/26dB for 4Rx for 1024 QAM CQI reporting test case.

- Conclusion

In this contribution, we provide simulation results for 1024QAM with the agreed simulation assumption in [1]. The proposals are summarized as below:

Proposal 1: Propose to select 28/29dB for 2Rx and 25/26dB for 4Rx for 1024 QAM CQI reporting test case.

- Reference

- R4-2207254, “Way forward for NR DL1024QAM demodulation and CQI reporting requirements”, Ericsson