3GPP TSG-RAN WG4 Meeting # 103-e	R4-2209833

# Electronic Meeting, May.9th –20th, 2022

Title: 	Simulation results for RedCap PDSCH requirements

Source: 	Huawei, HiSilicon

Agenda item:       9.19.5.2.1

Document for:	Discussion

Background

As per [1], RAN 4 agreed to define PDSCH and SDR requirements for RedCap UE. In this paper, we provide our simulation results for PDSCH requirements.

PDSCH test

The BLER-SNR curve are captured in Figure 2-1 and summary of simulation results are shown in Table 2-1 to 2-3.

Figure 2-1: BLER-SNR curve for PDSCH requirements

Table 2-1: Summary of simulation results for FDD 15kHz FR1

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Reference from TS38.101-4 Table | Ideal SNR(dB)@ 70% of max TP | Impairment SNR(dB)@ 70% of max TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x1 low | 5.2.2.1.1-3 Test 1-1 | 0.8 | 2.3 |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 5.2.2.1.1-3 Test 1-4 | 9.3 | 10.8 |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 5.2.2.1.1-4 Test 2-1 | 13.4 | 14.9 |
| 2Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x2 ULA low | 5.2.2.1.1-3 Test 1-1 | Reuse | Reuse |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 5.2.2.1.1-3 Test 1-4 | 5.3 | 6.8 |
| 2Rx UE | 10MHz / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 5.2.2.1.1-4 Test 2-1 | Reuse | Reuse |

Table 2-2: Summary of simulation results for TDD 30kHz FR1

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Reference from TS38.101-4 Table | Ideal SNR(dB)@70% of max TP | Impairment SNR(dB)@ 70% of max TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x1 low | 5.2.2.2.1-3 Test 1-1 | 1.2 | 2.7 |
| 1Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x1 low | 5.2.2.2.1-3 Test 1-4 | 9.3 | 10.8 |
| 1Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 5.2.2.2.1-4 Test 2-1 | 13.9 | 15.4 |
| 2Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x2 ULA low | 5.2.2.2.1-3 Test 1-1 | -2.5 | -1.0 |
| 2Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x2 ULA low | 5.2.2.2.1-3 Test 1-4 | 5.4 | 6.9 |
| 2Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 2 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 5.2.2.2.1-4 Test 2-1 | 16.2 | 17.7 |

Table 2-3: Summary of simulation results for TDD 120kHz FR2

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Reference from TS38.101-4 | Ideal SNR(dB)@70% of max TP | Impairment SNR(dB)@ 70% of max TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 100MHz / 120kHz | QPSK 1/3 Rank 1 | FR2.120-1A | TDLC60-300 | 2x2 ULA low | 7.2.2.2.1-3 Test 1-1 | Reuse | Reuse |
| 2Rx UE | 100MHz / 120kHz | 16QAM 0.48 Rank 2 | FR2.120-1 | TDLA30-300 | 2x2 ULA low | 7.2.2.2.1-4 Test 2-2 | Reuse | Reuse |
| 2Rx UE | 100MHz / 120kHz | 64QAM 0.46 Rank 2 | FR2.120-2 | TDLA30-75 | 2x2 ULA low | 7.2.2.2.1-4 Test 2-6 | Reuse | Reuse |

Conclusion

In this paper, we provide our simulation results for PDSCH demodulation requirements for alignment.

Reference

[1]   REV_ R4-2207206 WF RedCap UE demod. Ericsson