3GPP TSG-RAN WG4 Meeting # 103-e	R4-2209846

# Electronic Meeting, May.9th –20th, 2022

Title: 	Discussion on PUSCH requirements for FR2-2

Source: 	Huawei, HiSilicon

Agenda item:       9.15.10.3.1

Document for:	Discussion

Background

As per [1], RAN 4 has agreed to define PUSCH performance requirements for UE operating on FR2-2 band. There are some open issues left. In this paper, we will provide our initial simulation results and give analysis for these open issues.

Discussions

Issue 2-1-1: SCS for UL requirements definition

We agreed to define UL requirements for 120, 480 kHz SCS but FFS for 960kHz SCS. Based on our understanding, 960kHz need more capability and may be impractical in real world. Therefore, we propose to only consider 120kHz and 480kHz SCS.

Proposal 1: Only define UL requirements for 120 kHz and 480kHz SCS

CBW for UL requirements definition

CBW for UL requirements definition depends on max achievable SNR derived by link budget analysis and RAN4 agreed to study it in this meeting. Based on our understanding, it should belong to RF’s work and shouldn’t be discussed in demod part. But we can use the method of Rel-15 FR2-1 demodulation part as an initial analysis to exclude some bandwidth when the target SNR is obviously unachievable.

As per [1], we have following assumptions

- EIS_reference is 0dB higher than noise level (due to the SNR for the FRC being around 0dB).

- The AWGN power level for PUSCH is around 15dB higher than noise floor which is the same as EIS_reference level assuming SINR of around 0dB.

- The highest considered mean SINR threshold for uplink demodulation is around 25dB (for PUSCH MCS 16/17). Considering 10dB increment for fading channel variation, then we assume that the wanted signal should at least 50dB above EIS_reference for NR signal if all MCS are to be covered

- Take EIS_ RefSens_50M = -100dBm as a typical value. The wanted signal level at Wide area BS RIB is PRIB_demod_FR1 (dBm) = EIS_RefSens_50MHz + 3+50= -47dBm for 100MHz bandwidth PRIB_demod_FR1 (dBm) = EIS_RefSens_50MHz + 50= -41dBm for 400MHz bandwidth and PRIB_demod_FR1 (dBm) = EIS_RefSens_50MHz + 15+50= -35dBm for 1600MHz bandwidth.

- Signal generator output is -10dBm

- IAC:

Table 2.1 includes characters of instruments which we use in measurements for FR2-1 band derived from [1]. The total path loss between signal generator and coupler is also calculated. For 71GHz, some parameters can’t be achieved, we assume the total loss is same as 38GHz.

Table 2.1 characters of common instruments

| Frequency (GHz) | 3 | 5 | 27 | 38 | Note |
| --- | --- | --- | --- | --- | --- |
| Lcoupler (dB) | 3 | 3 | 3 | 3 | Coupler loss |
| GPA(dB) | 43 | 43 | 43 | 37 | Gain of power amplifier |
| L2dB_atten (dB) | 2 | 2 | 2 | 2 | 2dB attenuator loss |
| GPre(dB) | 29 | 29 | 28 | 25 | Gain of pre-amplifier |
| Lcable_1m (dB) | 1 | 1 | 1.5 | 2.5 | 1m cable loss |
| Lvar_atten (dB) | 2 | 2 | 2 | 2 | Variable attenuator loss |
| Lcombiner (dB) | 4 | 4 | 4 | 4 | Combiner loss |
| Lcable_5m (dB) | 3 | 3 | 7 | 10 | 5m cable loss |
| Ltotal (dB) | -60 | -60 | -54.5 | -41.5 | Ltotal=Lcable1m+Lcable5m+Lcombiner +Lvar_atten+L2dB_atten-GPA-GPre |

\

Figure 2.1 Setup for OTA demodulation

The results are captured in Table 2-2. Two metrics are used: PPA_out<PPA_out_max and PSG_est<-10dBm.

Table 2-2: OTA demodulation test link budget with IAC (wide range BS)

| Carrier Frequency (GHz) | 71 |  |  |  |  |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth | 100MHz |  | 400MHz |  | 1600MHz |  |  |
| Dimension (m) | 0.15 | 0.25 | 0.15 | 0.25 | 0.15 | 0.25 | 5 times of wave length and 10 times of wave length |
| dtest (m) | 11 | 15.8 | 11 | 15.8 | 11 | 15.8 | Far field distance |
| LFS (dB) | 90.3 | 93.4 | 90.3 | 93.4 | 90.3 | 93.4 | Free space loss |
| PPA_out dBm = PRIB_demod + LFS-Gtest_ant + Lcoupler | 23.3 | 26.5 | 29.3 | 32.5 | 35.3 | 38.5 | PRIB_demod =-47dBm 100MHz-41dBm 400MHz-35dBm 1600MHz Gtest_ant=23 dB. PPA_out_max=27dBm2~20GHz27dBm27~31GHz22dBm30~40GHz Lcoupler=3dB |
| PSG_est (dBm) =PPA_out+Ltotal | -18.2 | -15 | -12.2 | -9 | -6.2 | -3 | Estimated wanted signal level at SG Ltotal=-60dBFR1-54.5dB27GHz-41.5dB38GHz |

- CATR

The results are captured in Table 2-2. Two metrics are used: PPA_out<PPA_out_max and PSG_est<-10dBm.

We assume chamber area is  Areacatr=20m2. The coupling loss between test antenna and DUT is derived by power density

Lcoupling_catr=10log4π*Aeracatrλ2=10log80πλ2

We can estimate PPA_out and PSG_est in following table.

| Carrier frequency (GHz) | 71GHz |  |  | Note |
| --- | --- | --- | --- | --- |
| Bandwidth | 100 | 400 | 1600 |  |
| Lcoupling_catr (dB) | 71.5 | 71.5 | 71.5 | coupling loss |
| PPA_out dBm = PRIB_demod + Lcoupling_catr + Lcoupler | 27.5 | 33.5 | 39.5 | PRIB_demod =-47dBm 100MHz-41dBm 400MHz-35dBm 1600MHz PPA_out_max=27dBm2~20GHz27dBm27~31GHz22dBm30~40GHz Lcoupler=3dB |
| PSG_est (dBm) =PPA_out+Ltotal | -14 | -8 | -2 | Ltotal=-60dBFR1-54.5dB27GHz-41.5dB38GHz |

We can observe that all PA out of bandwidth are larger than PPA_out_max even PA_out_max of 40GHz is used. For 1600MHz, the expected PPA_out is far larger than PPA_out_max. Therefore, we propose to not define requirements for 1600MHz bandwidth.

We think more information about parameter on the test instrument are needed for 71GHz band if more accurate maximum achievable SNR calculation is performed.

Proposal 2: Not define requirements for 1600MHz bandwidth. Only consider 100MHz and 400MHz

Proposal 3: For 100MHz and 400MHz, more information about parameter on the test instrument are needed for 71GHz band if more accurate maximum achievable SNR calculation is performed.

MCS:

We provide our simulation for MCS 16 and MCS20 in Table 2-2:

Table 2-2: Simulation results for MCS16 and MCS20

| SCS/Bandwidth | MCS | Channel | Wo PN | PN Set2, CPE Comp | PN Set2, CPE and ICI Comp |
| --- | --- | --- | --- | --- | --- |
| 120/100 | 16 | TDL-A, 10ns | 8.1 | 8.2 | 10.2 |
|  | 20 | TDL-A, 10ns | 10.9 | 12.2 | 13.9 |
| 120/400 | 16 | TDL-A, 10ns | 8.3 | 8.5 | 9.7 |
|  | 20 | TDL-A, 10ns | 11.3 | 12.6 | 13.8 |
| 480/400 | 16 | TDL-A, 10ns | 8.4 | 9.6 | 10.2 |
|  | 20 | TDL-A, 10ns | 11.2 | 13.6 | 13.6 |

For MCS20, the performance loss of phase noise is larger than 1dB.

Observation 1: For MCS20, the performance loss of phase noise is larger than 1dB

Scope of PUSCH performance requirements

We have following options:

Define the following PUSCH performance requirements: - Performance requirements with transform precoding disabled with 70% throughput - FFS performance requirements with transform precoding enabled - FFS performance requirements for PUSCH repetition type A

Based on our understanding, considering this is first release for FR2-2, it’s better to focus on basic requirements. I.e. CP-OFDM PUSCH requirements.

Proposal 4: Not define the requirements with transform precoding enabled and not define requirements for PUSCH repetition type A

Conclusion

In this paper, we provide our views on PUSCH performance requirements definition. The proposals are:

Proposal 1: Only define UL requirements for 120 kHz and 480kHz SCS

Proposal 2: Not define requirements for 1600MHz bandwidth. Only consider 100MHz and 400MHz

Proposal 3: For 100MHz and 400MHz, more information about parameter on the test instrument are needed for 71GHz band if more accurate maximum achievable SNR calculation is performed.

Observation 1: For MCS20, the performance loss of phase noise is larger than 1dB

Proposal 4: Not define the requirements with transform precoding enabled and not define requirements for PUSCH repetition type A

Reference

[1]   R4-1901012	On link budget for OTA demodulation testing in FR1 and FR2. Ericsson