3GPP TSG-RAN WG4 Meeting #103-e	R4-2209860

# Electronic Meeting, 9th – 20th May, 2022

Title: 	Simulation results on PDSCH CA scenarios for NR UE HST FR1 performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	9.7.3

Document for:	Information

Background

During RAN4#102-e meeting, WF [1] on UE demodulation for FR1 HST was approved. In this contribution, we provide our updated simulation results for the demodulation requirements for NR UE HST FR1 CA scenarios.

Simulations

Here we provide updated simulation results for {35, 45} MHz for FDD 15kHz SCS for HST FR1 CA, as shown in Table 2-2, based on simulation assumption as shown in Table 2-1.

Table 2-1 Simulation assumption for HST FR1 HST CA

| Parameter | Value |
| --- | --- |
| Duplex mode | FDD |
| DMRS type | type 1 |
| Number of DMRS symbols | DMRS 1+1+1 |
| MCS | For HST-SFN JT: MCS 13 based on 64QAM table For DPS: MCS 17 based on 64QAM table |
| TRS periodicity | 10 ms, 2 slot pattern |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |
| Ds and Dmin | Ds = 700m, Dmin = 150m |
| Rank | Rank = 2 |
| Channel bandwidth | 35MHz/45MHz |
| Testing metric | SNR @ 70% of maximum throughput |

Table 2-2 Simulation results for HST FR1 HST CA

| Test number | Propagation condition | Number of Rx antennas | Channel bandwidth | Ideal SNR @ 70% maximum throughput | Impairment SNR @ 70% maximum throughput |
| --- | --- | --- | --- | --- | --- |
| 1 | HST-SFN | 2 | 35MHz | 11.30 | 13.80 |
| 2 | HST-SFN | 2 | 45MHz | 11.40 | 13.90 |
| 3 | HST-SFN | 4 | 35MHz | 8.48 | 10.98 |
| 4 | HST-SFN | 4 | 45MHz | 8.63 | 11.13 |
| 5 | HST-DPS | 2 | 35MHz | 9.52 | 12.02 |
| 6 | HST-DPS | 2 | 45MHz | 9.53 | 12.03 |
| 7 | HST-DPS | 4 | 35MHz | 6.64 | 9.14 |
| 8 | HST-DPS | 4 | 45MHz | 6.67 | 9.17 |

Conclusions

In this contribution, we provide our updated simulation results for demodulation performance for NR UE HST FR1 CA scenarios.

Reference

R4-2207194, WF on UE demodulation for FR1 HST, RAN4#102-e, CMCC