3GPP TSG-RAN WG4 Meeting #103-e	R4-2209865

# Electronic Meeting, 9th – 20th May, 2022

Title: 	Simulation results on PUSCH demodulation requirements for FR2 HST

Source: 	Huawei, HiSilicon

Agenda item:	9.8.4.2.1

Document for:	Information

Background

During RAN#102-e meeting, WF [2] on FR2 HST BS demodulation was approved. In this contribution, we provide our updated simulation results for the PUSCH demodulation requirements for FR2 HST.

Simulations

Here we provide updated simulation results for FR2 HST PUSCH for alignment, as shown in Table 2-2, based on simulation assumption as shown in Table 2-1. MCS 19 is selected based on the agreement achieved in last meeting.

Table 2-1 Simulation assumption for HST FR2 PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Channel Model |  | HST FR2 mode based on Bi-directional scenario B, the details can be referred as R4-2115726 |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos0, Pos1, Pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | 50MHz and 200MHz |
| resource | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2 |
| configuration | Time density (LPT-RS) | 1 |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns |  |  |

Table 2.2 Simulation results for FR2 HST PUSCH

| Case number | CBW [MHz] | DMRS | SNR@70%maximum throughput [dB] |  |
| --- | --- | --- | --- | --- |
|  |  |  | Ideal | Impairment |
| 1 | 50 | 1 | 10.29 | 12.79 |
| 2 | 50 | 1+1 | 9.65 | 12.15 |
| 3 | 50 | 1+1+1 | 9.41 | 11.91 |
| 4 | 200 | 1 | 10.25 | 12.75 |
| 5 | 200 | 1+1 | 9.41 | 11.91 |
| 6 | 200 | 1+1+1 | 9.51 | 12.01 |

Conclusions

In this contribution, we provide our updated simulation results for PUSCH performance requirements for NR HST FR2.

Reference

R4-2120703, WF on BS demodulation requirement for FR2 HST, RAN4#101-e, Samsung

R4-2207235, WF on BS demodulation requirement for FR2 HST, RAN4#102-e, Nokia, Nokia Shanghai Bell