3GPP TSG-RAN WG4 Meeting #103-e	R4-2209879

# Electronic Meeting, 9th – 20th May, 2022

Title: 	Discussion on satellite NTN demod PUCCH

Source: 	Huawei, HiSilicon

Agenda item:	9.12.8.2.2

Document for:	Discussion

Background

During RAN#102-e meeting, WF [1] on NTN satellite demodulation was approved. In this contribution, we share our views about NTN satellite PUCCH demodulation requirements.

Discussion

PUCCH

Scope of PUCCH requirements

Agreements - In addition to PUCCH format 0/1/2/3/4, RAN4 to define NTN multi-slot PUCCH demodulation requirements - Prioritize UCI with HARQ on PUCCH demodulation requirement

For PUCCH format 0, both 1 symbol and 2 symbol cases are defined for the legacy BS requirement. To reduce the test effort, we propose to only select one case for NTN performance requirements definition. 2 symbol case is preferred considering more sufficient link budget comparing to the 1 symbol case. In addition, ACK missed detection probability <1% can be reused as the test metric from the legacy BS requirements.

For PUCCH format 1, the test metric can be reused from the legacy BS requirements, i.e. NACK to ACK probability<0.1%, ACK missed detection probability <1%.

For PUCCH format 2, there are two cases for the testing with different UCI information bits for the legacy BS requirement. For the first case with test metric of ACK missed detection probability <1%, the test configuration can be reused directly. For the other case with the metric of UCI BLER<1%, as per agreement achieved in last meeting, only UCI with HARQ should be considered, so an addition note should be added that The UCI information does not contain CSI part 1 and CSI part 2. From our understanding, this test configuration is used for test only to ensure the test coverage.

For PUCCH format 3, the test metric can be reused from the legacy BS requirements, i.e. UCI BLER<1%. Addition note should be added that The UCI information does not contain CSI part 1 and CSI part 2. From our understanding, this test configuration is used for test only to ensure the test coverage. Also, there are two test cases with different time and frequency resource configuration (1 PRB with 14 symbol and 3PRB with 4 symbol). To reduce the test effort, we propose to only select one case for NTN performance requirements definition. 1 PRB with 14 symbol case is preferred with the additional DMRS configuration disabled.

For PUCCH format 4, the test metric can be reused from the legacy BS requirements, i.e. UCI BLER<1%. Addition note should be added that The UCI information does not contain CSI part 1 and CSI part 2. From our understanding, this test configuration is used for test only to ensure the test coverage. In addition, the additional DMRS configuration can be disabled to reduce the test effort.

For NTN PUCCH performance requirements definition, use the test configuration as shown in Table 3-1 to Table 3-7.

    - SCS/CBW set for PUCCH requirements

Candidate options - Option 1: 15kHz SCS: 5MHz/10MHz/20MHz, 30kHz SCS: 10MHz/20MHz - Option 2: a few of PRBs for all SCS

In last meeting, Option 2 is proposed by companies to reduce the test effort. We notice that for different bandwidth with other test configuration same, there is nearly same simulation results, so we think there is negligible influence on performance for different bandwidth. Therefore, only 5MHz requirements for 15kHz SCS and 10MHz requirements for 30kHz is enough.

Only define 5MHz requirements for 15kHz SCS and 10MHz requirements for 30kHz for NTN PUCCH performance requirements.

    - Antenna configuration for PUCCH requirements

Agreements - UE 1Tx - SAN 2Rx Candidate options - FFS: SAN 1Rx, SAN 4Rx and SAN 8Rx

For legacy BS performance requirements for PUCCH, except for multi-slot PUCCH, 2Rx/4RX/8Rx cases are defined with applicability rule that lowest and highest of supported Rx number shall be tested based on manufacture declaration. Similar method can be considered for NTN performance requirement definition to give manufacture more flexible to only test the configurations they support. Also considering that companies have concern about the test effort, we propose to apply the above applicability with the modification that only highest of supported Rx number shall be tested based on manufacture declaration rather than both lowest and highest.

Also consider SAN 4Rx and 8Rx for NTN PUCCH requirements definition except multi-slot PUCCH. Applicability rule can be defined so that only highest of supported Rx number shall be tested based on manufacture declaration.

Proposals

In this contribution, we discuss on satellite PUCCH demodulation requirements for NTN. Our observations and proposals are:

- For NTN PUCCH performance requirements definition, use the test configuration as shown in Table 3-1 to Table 3-7.

Table 3-1 PUCCH format 0

| Parameter | Test |
| --- | --- |
| PUCCH format | 0 |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 |
| Channel model | NTN-TDLA100-200 |
| Number of OFDM symbols | 2 |
| Test metric | ACK missed detection probability <1% |

Table 3-2 PUCCH format 1

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA100-200 |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

Table 3-3 PUCCH format 2

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA100-200 |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

Table 3-4 PUCCH format 2

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA100-200 |
| Test metric | UCI block error probability<1% |

Table 3-5 PUCCH format 3

| Parameter | Value |
| --- | --- |
| PUCCH format | 3 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | No additional DM-RS |
| Channel model | NTN-TDLA100-200 |
| Test metric | UCI block error probability<1% |

Table 3-6 PUCCH format 4

| Parameter | Value |
| --- | --- |
| PUCCH format | 4 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | No additional DMRS |
| Channel model | NTN-TDLA100-200 |
| Test metric | UCI block error probability<1% |

Table 3-7 PUCCH format 1 (multi-slot)

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 (multi-slot) |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA100-200 |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

Only define 5MHz requirements for 15kHz SCS and 10MHz requirements for 30kHz for NTN PUCCH performance requirements.

Also consider SAN 4Rx and 8Rx for NTN PUCCH requirements definition except multi-slot PUCCH. Applicability rule can be defined so that only highest of supported Rx number shall be tested based on manufacture declaration.

Reference

R4-2207198, WF on NTN SAN demodulation requirements, RAN4#102-e, Huawei, HiSilicon