# 3GPP TSG-RAN WG4 Meeting # 103-e	R4-2209897

# Electronic Meeting, 9 – 20 May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2388 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to relaxed RLM/BFD requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TBDs in the current requirements are removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change #1: TBDs are replaced by signaling parameters for RLM. Change #2: TBDs are replaced by signaling parameters for BFD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The relaxed RLM/BFD requirements in the specification will contain TBDs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.1, 8.5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### [Start of Change 1]

#### 8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria

For the UE supports SpCellConfig [2]and configured with explicit signaling [TBD]:

-	when the UE is not performing intra-band carrier aggregation configured with CSI-RS based RLM and CSI-RS based BFD on SCell, and

-	UE has fulfilled good serving cell quality criterion defined in goodServingCellEvaluationRLM [2] if the low mobility criteria is not configured, or

-	UE is also configured with low mobility criterion defined in lowMobilityEvaluationConnected [2] and UE has fulfilled both good serving cell quality criterion defined in goodServingCellEvaluationRLM [2] and low mobility criterion defined in lowMobilityEvaluationConnected [2].

-	when the UE is performing intra-band carrier aggregation configured with CSI-RS based RLM and CSI-RS based BFD on SCell, and

-	UE has fulfilled good serving cell quality criterion defined in goodServingCellEvaluationRLM[2] for CSI-RS based RLM on SpCell and for CSI-RS based BFD on serving cell if the low mobility criteria is not configured, or

-	the UE is configured with low mobility criterion defined in lowMobilityEvaluationConnected[2], and UE has fulfilled both good serving cell quality criterion defined in goodServingCellEvaluationRLM[2] and low mobility criterion defined in lowMobilityEvaluationConnected[2] for CSI-RS based RLM on SpCell and for CSI-RS based BFD on a serving cell.

The UE is allowed to apply the relaxed requirements defined in clause 8.1.2.[4] for SSB based radio link monitoring and the relaxed requirements defined in clause 8.1.3.[4] for CSI-RS based radio link monitoring; otherwise, UE shall apply the requirements defined in clause 8.1.2.2 for SSB based radio link monitoring and the requirements defined in clause 8.1.3.2 for CSI-RS based radio link monitoring.

The UE is not allowed to relax RLM measurements and apply the relaxed radio link monitoring provided that at least one of the following conditions is met:

-	The UE sends out-of sync indications to the higher layers,

-	The UE has triggered T310 timer

### [End of Change 1]

### [Start of Change 2]

#### 8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria

The relaxed beam failure detection procedures specified in this clause apply provided for the UE supports SpCellConfig or ScellConfig [2] and configured with dedicated signaling [TBD]:

-	when the UE is not performing intra-band carrier aggregation configured with CSI-RS based RLM on SpCell and CSI-RS based BFD on SCell provided that:

-	UE has fulfilled good serving cell quality criterion defined in goodServingCellEvaluationBFD [2] if the low mobility criteria is not configured, or

-	UE is also configured with low mobility criterion defined in lowMobilityEvaluationConnected [2] and UE has fulfilled both good serving cell quality criterion defined in goodServingCellEvaluationBFD [2] and low mobility criterion defined in lowMobilityEvaluationConnected [2].

-	when the UE is performing intra-band carrier aggregation configured with CSI-RS based RLM on SpCell and CSI-RS based BFD on SCell provided that:

-	UE has fulfilled good serving cell quality criterion defined in goodServingCellEvaluationBFD [2] for CSI-RS based RLM on SpCell and for CSI-RS based BFD on serving cell if the low mobility criteria is not configured, or

-	the UE is configured with low mobility criterion defined in lowMobilityEvaluationConnected [2], and UE has fulfilled both good serving cell quality criterion defined in goodServingCellEvaluationBFD [2] and low mobility criterion defined in lowMobilityEvaluationConnected [2] for CSI-RS based RLM on SpCell and for CSI-RS based BFD on a serving cell.

Note: 	the explicit signalling indicates that the low mobility state of the UE, if the low mobility criteria is not configured

The scenario and RS resource configurations in the set  defined in section 8.5.1 apply for this section.

The UE is allowed to apply the relaxed requirements defined in clause 8.5.2.4 for SSB based beam failure detection and the relaxed requirements defined in clause 8.5.3.4 for CSI-RS based beam failure detection; otherwise, UE shall apply the requirements defined in clause 8.5.5 for SSB based beam failure detection and the requirements defined in clause 8.5.3 for CSI-RS based beam failure detection.

The UE is not allowed to apply the relaxed link recovery procedures provided that at least one of the following conditions is met:

The UE sends a beam failure instance indication to the higher layers,

The UE has triggered T310 timer.

### [End of Change 2]