3GPP TSG-RAN WG4 Meeting # 103 -e	R4-2209962

Electronic Meeting, May 9th – 20th, 2022

Title:	TP to TR 37.717-21-11 Addition of DC_40A-42A_n77A

Source:	Nokia

Agenda item:	8.12.2

Document for:	Approval

# 1	Introduction

This contribution introduces DC_40A-42A_n77A.

******************************* Start of TP ***************************************

## 5.x	DC_40-42_n77

### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_40A-42A_n77A | DC_40A_n77A | CA_40A-42A | n77A |

### 5.x.2	 Co-existence studies

For UE coexistence study of Band 40 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.x.2-1.

Table 5.x.2-1: Harmonic and IMD analysis

|  | Fx low | Fx high | Fy low | Fy high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2300 | 2400 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 16500 | 21000 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 900 | 1900 | 5600 | 6600 |
| Two tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 400 | 703 | 4200 | 6100 |
| Two tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7900 | 9000 | 8900 | 10800 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2700 | 3900 | 7500 | 10300 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3800 | 1800 | 11200 | 13200 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10200 | 11400 | 12200 | 15000 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14500 | 10800 | 6300 | 5000 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8000 | 5100 | 600 | 1500 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15500 | 19200 | 12500 | 13800 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14500 | 17400 | 13500 | 15600 |

Additional UE coexistence study of Band 42 + Band n77, there is no need since it is not used for UL due to overlapped bands.

The Rx impacts can be identified as below.

- 5th Harmonic generated and 4th order IMD products generated by DC_40_n77 uplink fall into Rx of band 42.

### 5.x.3		∆TIB and ∆RIB values

For DC_40A-42A_n77A, the TIB,c and RIB,c values are reused from CA_40-42, DC_40_n77 and DC_42_n78 as given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_40-42_n77 | 40 | 0.45 |
|  | 42 | 0.55 |
|  | n77 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_40-42_n77 | 40 | 0.46 |
|  | 42 | 0.56 |
|  | n77 | 0.56 |
| NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 5.x.4	Reference sensitivity exceptions

MSD for Harmonic issue is already specified in the corresponding specs.

MSD for the IMD4 is derived from similar combination CA_40-42, CA_n40A-n77A and DC_1A_n40A-n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_40A-42A_n77A | 40 | 2350 | 5 | 25 | 2350 | N/A | N/A |
|  | 42 | 3500 | 5 | 25 | 3550 | 5 | IMD4 |
|  | n77 | 3500 | 5 | 25 | 3550 | 9.8 | IMD4 |
|  | 40 | 2350 | 5 | 25 | 2350 | 10.6 | IMD4 |
|  | 42 | 3525 | 5 | 25 | 3525 | N/A | N/A |
|  | n77 | 3500 | 5 | 25 | 3550 | N/A | N/A |

******************************* End of TP ***************************************