3GPP TSG-RAN WG4 Meeting # 103-e                                				R4-2209989

Electronic Meeting, 9– 20 May, 2022

Source: 	Huawei, HiSilicon

Title:	TP for TR 37.717-11-21: DC_n77A-n257A/G/H/I/J/K/L/M_1A-3A, DC_n77(2A)-n257A/G/H/I/J/K/L/M_1A-3A

Agenda Item:	8.15.3

Document for:	Approval

# Introduction

This contribution provides a text proposal on DC band combination of NR bands n77, n257 and LTE bands B1 and B3 for TR 37.717-11-21[1].

# Reference

- TR 37.717-11-21, 	Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL), v0.8.0

# Text Proposal

----- Start of TP -----

## 7.x	DC_n77-n257_1-3

### 7.x.1		Operating bands for DC

Table 6.x.1-1: DC band combination of NR 2DL/1UL + 2 LTE bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_n77-n257_1-3 | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |
|  | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |

### 7.x.2		Channel bandwidths per operating band for DC

Table 7.x.2-1: Supported bandwidths per DC inter-band NR 2DL/1UL + two LTE band

| NR DC and E-UTRA Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_n77A-n257A_1A-3A DC_n77A-n257G_1A-3A DC_n77A-n257H_1A-3A DC_n77A-n257I_1A-3A DC_n77A-n257J_1A-3A DC_n77A-n257K_1A-3A DC_n77A-n257L_1A-3A DC_n77A-n257M_1A-3A | DC_n77A_1A DC_n257A_1A DC_n77A_3A DC_n257A-3A | 3 | - | 550 250 350 450 550 650 750 850 |
|  |  | n77 | CA_n77A-n257A/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |
| DC_n77(2A)-n257A_1A-3A DC_n77(2A)-n257G_1A-3A DC_n77(2A)-n257H_1A-3A DC_n77(2A)-n257I_1A-3A DC_n77(2A)-n257J_1A-3A DC_n77 (2A) -n257K_1A-3A DC_n77(2A)-n257L_1A-3A DC_n77(2A)-n257M_1A-3A |  | 3 | - | 650 350 450 550 650 750 850 950 |
|  |  | n77 | CA_n77(2A)-n257A/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |

### 7.x.3		Co-existence studies

- For combination of DC_n77A_3A and DC_n77A_1A, the co-existence studies have been covered in TR 37.863-01-01 as DC_3A_n77A and DC_1A_n77A.

- For combination of DC_n257A_3A and DC_n257A_1A, there is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency separation.

### 7.x.4	∆TIB and ∆RIB values

Table 7.xx.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_n77-n257_1-3 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n77 | 0.8 |
|  | n258 | 0 |

Table 7.xx.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_n77-n257_1-3 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |
|  | n258 | 0 |

### 7.x.5	MSD

No additional MSD requirement is needed.

----- End of TP -----