3GPP TSG-RAN WG4 Meeting # 103-e												R4-2210096

Electronic Meeting, May 09 – 20, 2022

Agenda item:			9.20.1.6

Source:	Ericsson

Title:	Additional dropping rule for Cap 1B and Cap 2 UEs during gapless PRS measurement

Document for:	Discussion

# Introduction

RAN4#102-e agreed on scheduling availability of Cap 1A, Cap 1B, and Cap 2 UEs during gapless RSTD, UE RxTx, PRS-RSRP, and DL PRS-RSRPP measurements. In RAN1#108-e RAN1 made the following further agreement on UE scheduling availability during gapless PRS measurement [1]:

Agreement For capability 1A as per working assumption made in RAN1#106-e, the DL signalings/channels in a per UE fashion (i.e. both across NR & LTE) inside the PRS processing window are dropped if the DL PRS is determined to be higher priority. For capability 1B as per working assumption made in RAN1#106-e, only the DL signalings/channels from a certain band inside the PRS processing window are dropped if the DL PRS is determined to be higher priority. Working assumption: Subject to UE capability, support PRS measurement outside the MG, within a PRS processing window, and UE measurement inside the active DL BWP with PRS having the same numerology as the active DL BWP. - Inside the PRS processing window, subject to the UE determining that DL PRS to be higher priority, support the following UE capabilities: - Capability 1: PRS prioritization over all other DL signals/channels in all symbols inside the window. - Cap. 1A: The DL signals/channels from all DL CCs (per UE) are affected. - Cap. 1B: Only the DL signals/channels from a certain band/CC are affected. - FFS: band or CC - Capability 2: PRS prioritization over other DL signals/channels only in the PRS symbols inside the window - A UE shall be able to declare a PRS processing capability outside MG. - FFS: Details of capability signalling (e.g., per UE or per band, etc.). Agreement For capability 2 as per working assumption made in RAN1#106-e - For FR1, only the DL signals/channels from a certain CC inside the PRS processing window, which overlap with DL PRS symbols in time, are dropped if the DL PRS is determined to be higher priority - For FR2, only the DL signals/channels from a certain band inside the PRS processing window, which overlap with DL PRS symbols in time, are dropped if the DL PRS is determined to be higher priority For the DL signals/channels from a different FR2 band than the FR2 band of the DL PRS for capability 1B and 2, subject to dropping due to the same Rx beam across multiple FR2 bands if the DL PRS is determined to be higher priority, it is up to RAN4 to define. - Send an LS to RAN4

# Discussion

R1-2202842 requests RAN4 to define dropping rule for the DL signals/channels from a different FR2 band than the FR2 band of the DL PRS for capability 1B and 2 when UE has same Rx beam across multiple FR2 bands. In our understanding this type of scenario falls under common beam management (CBM) in DL inter-band CA framework in FR2. RAN4 has been developing requirements for CBM in DL inter-band CA framework in FR2 under Rel-17 WI, “Further enhancements of NR RF requirements for frequency range 2 (FR2)”. But due to lack of progress the CBM in DL inter-band CA in FR2 was removed at RAN#95e. Therefore, the revised WID does not contain the objective on CBM for DL inter-band CA in FR2 anymore [2].

Observation 1: Use of the same Rx beam across multiple FR2 bands where in one FR2 band it is receiving DL signals/channels and in the other FR2 band it is receiving DL PRS, falls under common beam management (CBM) in DL inter-band CA in FR2.

Observation 2: At RAN#95e, the objective on CBM in DL inter-band CA in FR2 was removed from Rel-17 WI, “Further enhancements of NR RF requirements for frequency range 2 (FR2)”.

Since CBM for DL inter-band CA in FR2 was removed from the WI scope in the last RAN plenary RAN#95e, RAN4 should not define the additional dropping rule for Cap 1B and Cap 2 UEs when UE has same Rx beam across multiple FR2 bands where in one FR2 band it is receiving DL signals/channels and in the other FR2 band it is receiving DL PRS.

Proposal 1: Do not define dropping rule for Cap 1B and Cap 2 UEs when UE has same Rx beam across multiple FR2 bands where in one FR2 band it is receiving DL signals/channels and in the other FR2 band it is receiving DL PRS.

RAN4 should therefore provide feedback to RAN1 by sending reply to LS to RAN1 on the dropping rule of DL signals/channels for capability 1B and 2. A draft of reply LS is provided at the annex of this contribution.

# Summary

In this paper issue related to dropping rule for Cap 1B and Cap 2 UEs when UE has same Rx beam across multiple FR2 bands where in one FR2 band it is receiving DL signals/channels and in the other FR2 band it is receiving DL PRS is addressed.

Observation 1: Use of the same Rx beam across multiple FR2 bands where in one FR2 band it is receiving DL signals/channels and in the other FR2 band it is receiving DL PRS, falls under common beam management (CBM) in DL inter-band CA in FR2.

Observation 2: At RAN#95e, the objective on CBM in DL inter-band CA in FR2 was removed from Rel-17 WI, “Further enhancements of NR RF requirements for frequency range 2 (FR2)”.

The following is proposed for agreement:

Proposal 1: Do not define dropping rule for Cap 1B and Cap 2 UEs when UE has same Rx beam across multiple FR2 bands where in one FR2 band it is receiving DL signals/channels and in the other FR2 band it is receiving DL PRS.

# References

[1] R1-2202842, “LS on the dropping rule of DL signals/channels for capability 1B and 2.”, Huawei.

[2] RP-220968, “Revised WID Further enhancements of NR RF requirements for frequency range 2 (FR2)”, Nokia, Nokia Shanghai Bell.

# Annex

# Draft LS to RAN1

Title:	Reply LS on the dropping rule of DL signals/channels for capability 1B and 2.

Response to:	R1-2202842

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	TSG RAN WG4

To:	TSG RAN WG1

Cc:

Contact Person:

Name:			Deep Shrestha

E-mail Address:			deep.shrestha@ericsson.com

1. Overall Description:

RAN4#102-e has already made agreements on scheduling availability of UE during gapless PRS measurement. RAN4 also discussed the additional dropping rule for Cap 1B and Cap 2 UEs when UE has same Rx beam across multiple FR2 bands where in one FR2 band it is receiving DL signals/channels and in the other FR2 band it is receiving DL PRS. In RAN4 view this issue is related to CBM. But at RAN#95e, the objective on CBM in DL inter-band CA in FR2 was removed from Rel-17 WID, “Further enhancements of NR RF requirements for frequency range 2 (FR2)” in RP-220968. Since CBM is out of the above Rel-17 WI scope, therefore RAN4 made the following agreement:

Agreement:

Do not define dropping rule for Cap 1B and Cap 2 UEs when UE has same Rx beam across multiple FR2 bands where in one FR2 band it is receiving DL signals/channels and in the other FR2 band it is receiving DL PRS.

2. Actions:

To RAN1

ACTION: 	RAN4 respectfully requests RAN1 to take above agreement into account.

3. Date of Next TSG-RAN WG4 Meetings:

| TSG-RAN WG4 Meeting #104 | 22 August - 26 August 2022 | Toulouse, France |
| --- | --- | --- |
| TSG-RAN WG4 Meeting #104-bis-e | 10 October - 19 October 2022 | Electronic Meeting |