3GPP TSG-RAN WG4 Meeting#103-e	R4-2210149

Online, 9th May  – 20th May, 2022

Source: 	Qualcomm Incorporated

Title: 	Views on m-TRP CSI Performance Requirements

Agenda item:	9.18.4.3.1

Document for:	Discussion

- Introduction

To evaluate the performance of m-TRP CSI in the context of FeMIMO, we would need to determine CSI quantities to be tested along with other test parameters that might affect the CSI reporting. In this paper, we provide our views on the requirements for the m-TRP CSI tests.

- Requirements for m-TRP CSI Tests

Issue 3-1-1: Test cases for CSI reporting enhancement for m-TRP transmission

The performance of the PMI reporting is captured via the precoding gain, which can be expressed as the relative improvement in throughput when the network follows the UE reported PMI compared to the case when it follows the random precoding. Since only one CQI will need to be reported and rank reporting may not be different, i.e., considering single TRP (e.g., rank 4) and m-TRP with 2+2 case, we propose to define performance requirement only for PMI reporting.

Tentative agreements:

- Define PMI reporting requirement for single-DCI based Multi-TRP scheme with full overlapped resource allocation (SDM) only in FR1

Candidate Options:

- FFS on additional CSI reporting requirement for single/multi-DCI based Multi-TRP scheme

  - Option 1

    - Option 1a: Define new CSI reporting requirement for CQI reporting for Multi-DCI based Multi TRP scheme

    - Option 1b: Define RI, CQI reporting requirement for single-DCI based Multi-TRP, and define CQI reporting requirement for multi-DCI

  - Option 2: Not define RI, CQI reporting requirement for single-DCI, Not define CQI reporting requirement for multi-DCI

From CQI/RI processing point of view, the multi-DCI processing is not different compared to that of single DCI transmission. Hence, we don’t see a need to define such requirement. Also, our understanding is that multi-DCI is not part of the FeMIMO WID for “Enhancement on CSI measurement and reporting”.

Proposal 1: Option 2 (Not define RI, CQI reporting requirement for single-DCI, Not define CQI reporting requirement for multi-DCI)

- Conclusions

This paper provides our views on the remaining open issues in defining the requirements for the m-TRP CSI tests. The following has been proposed:

Proposal 1: Option 2 (Not define RI, CQI reporting requirement for single-DCI, Not define CQI reporting requirement for multi-DCI)

# References

[1] R4-2207209, “WF on CSI requirement for Rel-17 FeMIMO,” 3GPP TSG RAN WG4 Meeting #102-e, Online, Feb 21 – Mar 3, 2022.