3GPP TSG-RAN WG4 Meeting # 103-e 											       R4-2210255

Electronic Meeting, 9th – 20th May, 2022

Agenda item:			8.22

Source:	Moderator (Huawei)

Title:	Email discussion summary for [103-e][120] NR_BCS4-Core

Document for:	Information

Introduction

This email discussion handles the contributions submitted to agenda item 8.22 for NR_BCS4 and MSD_Inter_Band_ENDC. The scope of this email discussion covers the maximum aggregated bandwidth for CA with BCS4/BCS5, Improvements to MSD table, and some CRs. There are three topics listed as below in this email discussion and multiple sub-topics within each of them.

#1 The maximum aggregated bandwidth for CA with BCS4/BCS5

#2 Improvements to MSD table

#3 Discussion on CRs

Topic #1: The maximum aggregated bandwidth for CA with BCS4/BCS5

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2208676 | Qualcomm Incorporated | Observation 1: Multiple feature sets approach will introduce a huge amount of signalling overhead. Proposal 1: Introduce a new IE to indicate the maximum aggregated bandwidth for intra-band CA per band combination in Rel-17. Proposal 2: The new IE of maximum aggregated bandwidth can be applied for BCS5 with early implementation from Rel-15. FFS on whether and how to apply the new IE for BCS4. Observation 2: The maximum aggregated CBW that supports by UE is up to 600MHz as of now and it might be even more in future. But UE might not be able to support such a high aggregated CBW due to the limitation. Proposal 3: Introduce a new IE to indicate the maximum aggregated bandwidth for inter-band CA per band combination in Rel-17. Proposal 4: To approve the draft LS to RAN2 provided in the appendix. |
| R4-2208858 | Xiaomi | Proposal 1: Introduce a new UE capability indicating the max aggregated CBW for CA in Rel-17 that the UE actually supports. Proposal 2: BCS 5 with the new signalling of max aggregated CBW could be allowed for early implementation from Rel-15. Proposal 3: Send LS to RAN2 to ask introduce the new signalling of max aggregated CBW as the annex. Proposal 4: How BCS4 uses the new signalling of max aggregated CBW need further discuss. |

Open issues summary

Sub-topic 1-1

Sub-topic description:

The maximum aggregated bandwidth for intra-band CA with BCS4/5 was discussed in last meeting. Based on the email discussion summary R4-2206414, there is no conclusion on whether the maximum aggregated bandwidth can be changed or not for intra-band contiguous CA with BCS4/5 in the later release to avoid creating new issues, e.g. potential problematic intra-band combination CA_n7B.

In this meeting, moderator proposes to discuss companies’ proposals about the maximum aggregated bandwidth for CA with BCS4/BCS5 in the first round. Once RAN4 can agree these proposals, the LS can be discussed in second round.

Issue 1-1-1: Please discuss whether the following proposals for intra-band CA can be agreeable

Proposals in Tdoc R4-2208676 (Qualcomm):

Proposal 1: Introduce a new IE to indicate the maximum aggregated bandwidth for intra-band CA per band combination in Rel-17.

Proposal 2: The new IE of maximum aggregated bandwidth can be applied for BCS5 with early implementation from Rel-15. FFS on whether and how to apply the new IE for BCS4.

Proposals in Tdoc R4-2208858 (Xiaomi):

Proposal 1: Introduce a new UE capability indicating the max aggregated CBW for CA in Rel-17 that the UE actually supports.

Proposal 2: BCS 5 with the new signalling of max aggregated CBW could be allowed for early implementation from Rel-15.

Proposal 4: How BCS4 uses the new signalling of max aggregated CBW need further discuss.

Proposals

  - Option 1: Yes.

  - Option 2: No.

  - Option 3: Others.

- Recommended WF

  - TBA

Issue 1-1-2: Please discuss whether the following proposals for inter-band CA can be agreeable:

Proposals in Tdoc R4-2208676 (Qualcomm):

Proposal 3: Introduce a new IE to indicate the maximum aggregated bandwidth for inter-band CA per band combination in Rel-17.

Proposals

  - Option 1: Yes.

  - Option 2: No.

  - Option 3: Other options.

- Recommended WF

TBA

Companies views’ collection for 1st round

Open issues

Sub-topic 1-1

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Issue 1-1-1: Option 2. As we discussed in last meeting, the maximum aggregated bandwidth can’t be changed for intra-band contiguous CA with BCS4/5 in the later release to avoid creating new issues if we can specify the theoretically possible max aggregated CBW for intra-band contiguous CA with BCS4/5 as we did for CA_n41C_BCS4. Issue 1-1-2: Option 2: No. Currently, we have capabilities supportedBandwidthDL and supportedBandwidthUL to Indicates maximum UL channel bandwidth supported for a given SCS that UE supports within a single CC. It’s enough and there is no need to introduce this redundant capability. |
| Qualcomm | Issue 1-1-1: Not sure if the three options are applicable for all the five proposals from two companies? Shouldn’t we response case by case? In general, we support to introduce a new IE for max aggregated CBW. At least for BCS5, introducing a new IE will not have impact on the release independent since we already agreed to introduce the min CBW for BCS5 in Rel-17. The proposals of introducing a new signalling for intra-band CA is to solve the issue on how to specify the maximum aggregated CBW for BCS4/5 for some of intra-band CA band combos which could not specify as theoretically max aggregated CBW. If this issue could not be solved, how the BCS4/5 can be applied for those band combos? Should RAN4 restrict the BCS4 not to apply for the intra-band CA? This should be solve in the last meeting for BCS4 WI. From our point of view, introducing a new IE is the way to solve this issue. If the aggregated CBW has to be indicated by multiple feature set, additional processing is needed both on UE side and network side to process potential large number of feature set combinations. Network side has to process all different possible BW combinations and pick one that is most suitable for UE. In this case, network will have additional processing. Also, additionally, UE vendors will also have to define these individual band combinations with different bandwidth combinations by multiple feature set. Therefore, with new signaling, everyone in ecosystem gets the benefit. To Huawei: As we mentioned in the above, the problem is how to specify the band combos in which the theoretically max aggregated CBW could not be used, e.g., CA_n7B. For “the maximum aggregated bandwidth can’t be changed for intra-band contiguous CA with BCS4/5 in the later release to avoid creating new issues…”, could you remind what’s the new issue here? Issue 1-1-2: We are the proponent for this proposal. The signalling of supportedBandwidthDL and supportedBandwidthUL are used for single carrier per feature set. But UE could report multiple feature set with different possible BW combinations. In this case, network side will have to process the feature sets which will increase the implantation complexity. As we mentioned in Issue 1-1-1, with new signaling, everyone in ecosystem gets the benefit. |

| Xiaomi | The proposals coming from two companies are consistent considering both issue 1-1-1 and 1-1-2, we support Introduce a new IE to indicate the maximum aggregated bandwidth for intra-band CA and inter-band CA per band combination in Rel-17. The new IE of maximum aggregated bandwidth can be applied for BCS5 with early implementation from Rel-15. - FFS on whether and how to apply the new IE for BCS4. |
| --- | --- |

| ZTE | Seems the two companies’ proposals are similar. The common issue from the two companies’ proposals are the new IE for BCS4. We have discussed this issue in last meeting, if new IE is introduced for BCS4, then it cannot release independent from Rel-15, also there seems no need to have BCS4 and BCS5 together. So we think new IE could not be introduced to BCS4, otherwise, the previous agreements would be overturned. BTW, there seems no agreements on ‘the maximum aggregated bandwidth can’t be changed for intra-band contiguous CA with BCS4/5 in the later release, also we share the similar comments with QC, what’s the new issue here? |
| --- | --- |
| T-Mobile USA | Issue 1-1-1: We support adding a new UE capability for maximum aggregated BW for intra-band CA. However, One thing we haven’t seen discussed is that there could be more than one intra-band CA combination in a given NR CA combination. For instance, CA_n7B-41C-n78(2A). In this case, there might need to be a maximum aggregated bandwidth per intra-band combination, or at least a way to indicate which intra-band combination the max applies to. To avoid problems with adding new signalling to BCS4, we would be OK if the new maximum aggregated intra-band bandwidth per band IE applies to BCS5 but not BCS4. Issue 1-1-2: We support adding a new UE capability for maximum total aggregated bandwidth, but this should include both inter-band and intra-band for combinations with intra-band plus inter-band CA or DC. We have heard from multiple UE/chip vendors of such maximum aggregated BW limitations, and it is very inefficient to have to use multiple features sets to indicate such limitations. |
| CHTTL | Issue 1-1-1 & Issue 1-1-2: One clarification is that this new signaling IE is applied to BCS5 only, is it correct understanding? |
| Nokia | First of all, the new capability shall not be used together with BCS4 and if it’s introduced, it shall be from Rel-17. Importance of the matter of the introduction of a capability for aggregated channel bandwidth may not be the same as that of supportedMinBandwidthDL/ supportedMinBandwidthUL. Regarding the latter, without supportedMinBandwidthDL/ supportedMinBandwidthUL, in order to minimize the supported channel bandwidth combinations, a UE needs to drop supporting a channel bandwidth(s) per band as single band operation while some of the UE channel bandwidths are defined as mandatory support in the specifications so that they were introduced. Thanks to that capability, it just reduces the supported channel bandwidth combinations during CA if necessary and no impact on supported channel bandwidths for single band operation. While regarding maximum aggregated bandwidth, we have an alternative, i.e., multiple features sets. Lastly, we don’t agree with a comment from Huawei that max aggregated channel bandwidth cannot be changed. And there is no such an agreement. If it’s changed, the UE shall report newly introduced wider aggregated channel bandwidth explicitly. The measures to do so is not have to use a new IE. Currently signaling makes it possible but we need to make sure that UEs supporting newly introduced wider aggregated channel bandwidths shall be explicitly reported. Otherwise, network has no idea on which UE can handle which of the wider aggregated channel bandwidths. |
| Huawei | After reading the comments from QC and T-Mobile USA, I tried to understand the limitations for UE/chip vendors. - Are the multiple features sets issue only for BCS4/5? Or other traditional BCS have the same problems especially for inter-band CA? - Can the maximum total aggregated bandwidth represent the baseband capability limitation? If the reason for introduction of the maximum total aggregated bandwidth is due to the baseband capability limitation, I’d like to share my understanding as below. RAN2 used to try to introduce a total baseband capability for per CA band combination / UE in Rel-15. But it ended up with failure. Why it’s failed is that channel bandwidth / aggregated bandwidth only represent one parameter of the baseband. We still have different parameters: such as SCS, DL MIMO layer and so on. If SCS, DL MIMO or other parameters are changed, network will not know how to configure the channel bandwidth for each carrier. For example: If UE indicate the 100MHz maximum total aggregated bandwidth for CA_n25-n41 with, I’d like to ask companies whether network can the following cases represent the same baseband capabilities with same 100MHz maximum total aggregated bandwidth. CASE 1: (n25, 40MHz, 15kHz SCS, 2DL MIMO…) + (n41, 60MHz, 60kHz SCS, 2DL MIMO…) 100MHz total aggregated bandwidth CASE 2: (n25, 10MHz, 15kHz SCS, 2DL MIMO…) + (n41, 90MHz, 30kHz SCS, 4DL MIMO…) 100MHz total aggregated bandwidth These two cases have the same 100MHz total aggregated bandwidth, but the baseband capabilities are different. From network perspective, 30MHz baseband capability for band n25 is not same as to band n41. How can network deploy and configure that based on the new IE maximum total aggregated bandwidth? In my understanding, baseband capability limitation = f 1(maxBW, SCS, DL MIMO…) + f 2(maxBW, SCS, DL MIMO…) + …. It isn’t related to maximum total aggregated bandwidth. If so, I think the new IE maximum total aggregated bandwidth can’t solve the multiple features sets issue. To QC question, CA_n7B_BCS0 with 50MHz maximum total aggregated bandwidth has been specified in current spec. In the future, if operators have demands to specify BCS4 with larger maximum total aggregated bandwidth, it seems that theoretical max aggregated CBW 70MHz is the only choice to stop introducing new BCS. To Nokia, Since it’s approaching the end of Rel-17, in order to avoid some discussions for this controversial issue, maximum aggregated bandwidth can’t be changed for all the intra-band contiguous/non-contiguous CA with BCS4/5 specified in Rel-17. Since we didn’t see any BCS4/5 intra-band contiguous/non-contiguous CA cases which need to maximum aggregated bandwidth. We can further discuss whether maximum aggregated bandwidth can be changed or not for all the intra-band contiguous/non-contiguous CA with BCS4/5. By the way, To T-Mobile USA, band n7 and n41 can’t be deployed together in one UE or a certain region. |
| Ericsson | Issue 1-1-1: Option 2. No. We don’t support adding a new UE capability for maximum aggregated BW for intra-band CA. Issue 1-1-2: Option 2: No. Currently, we have capabilities supportedBandwidthDL and supportedBandwidthUL to Indicates maximum UL channel bandwidth supported for a given SCS that UE supports within a single CC. It’s enough and there is no need to introduce this capability. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Issue 1-1-1 and Issue 1-1-2: Qualcomm, Xiaomi, T-Mobile and ZTE(?) support option 1. Huawei and Ericsson support option 2. CHTTL want to clarify whether this new signaling IE is applied to BCS5 only. Huawei tried to understand the limitations for UE/chip vendors and ask the following questions. 1) Are the multiple features sets issue only for BCS4/5? Or other traditional BCS have the same problems especially for inter-band CA? 2) Can the maximum total aggregated bandwidth represent the baseband capability limitation? Ericsson think we have capabilities supportedBandwidthDL and supportedBandwidthUL to Indicates maximum UL channel bandwidth supported for a given SCS that UE supports within a single CC. Given companies still have concerns on these proposals and views are diversity, proponents can try to address companies’ concerns and find a potential compromise in the 2nd round. From moderator’s perspective, this is last meeting for this Rel-17 WI, if we can’t reach a consensus in the end, we have to drop this proposal. The new IE maximum total aggregated bandwidth was also proposed in FR2 email thread. Based on the discussion, the limitations for UE/chip vendors is not only for BCS4/5, but also other traditional BCS. Maybe a wide discussion is needed. Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss Issue 1-1-1 and Issue 1-1-2 in the 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Topic #2: Improvements to MSD table

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2208454 | CHTTL | Proposal 1: When considering the MSD table reduction for the direct hit harmonic and the cross-band isolation for FR1, at least the following two cases are considered to be specified. - The MSD requirement for the minimum victim downlink channel bandwidth. - The MSD requirement for the maximum victim downlink channel bandwidth of 100MHz if the victim band supports 100MHz channel bandwidth. Proposal 2: When considering the MSD table reduction for the direct hit harmonic and the cross-band isolation for FR1, for the case that the maximum supported channel BW of the victim band is smaller than 100MHz, the following options can be considered. - - Option 2-1: The 2nd MSD requirement is based on the maximum victim downlink channel bandwidth supported in the latest spec, if larger victim downlink channel bandwidth is introduced in the future whether additional requirement is needed can be further discussed. - - Option 2-2: Whether the 2nd MSD requirement based on the maximum victim downlink channel bandwidth can be requested by operators if needed. Proposal 3: When considering transferring the current table to the new format, the value of the MSD and the uplink configurations should remain unchanged. |
| R4-2208681 | ZTE Corporation | Proposal 1: Except the minimum victim downlink channel bandwidth, another test point is proposed as maximum victim downlink channel bandwidth. Proposal 2: On top of the MSD test points proposed above, additional test point(s) would be needed in case of minimum victim downlink channel bandwidth or maximum victim downlink channel bandwidth changed in future. |
| R4-2209358 | Huawei, HiSilicon | Proposal 1: To update the REFSENS exceptions due to cross band isolation for SUL as below. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; X band interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n80; n41; 1780; 10; 15; 50 (RBstart=0); 2505; 10; 4.3; >ACLR2. n80; n41; 1780; 10; 15; 50 (RBstart=0); 2550; 100; 3.0; >ACLR2. n95; n41; 2017.5; 15; 15; 75 (RBstart=0); 2505; 10; 6.1; >ACLR2. n95; n41; 2017.5; 15; 15; 75 (RBstart=0); 2550; 100; 6.1; >ACLR2. n97; n41; 2360; 80; 30; 216 (RBstart=0); 2505; 10; 20.7; ACLR2. n97; n41; 2360; 80; 30; 216 (RBstart=0); 2550; 100; 10.6; ACLR2. Proposal 2: To update the REFSENS exceptions due to harmonic interference for SUL as below. UL band; DL band; UL BW; SCS of UL band; UL RB Allocation; DL BW; MSD; UL/DL fc condition; UL/DL harmonic order. (MHz); (kHz); LCRB; (MHz); (dB). n80; n77; 5; 15; 25 (RBstart=0); 10; 23.9; NOTE 2; UL2/DL1 direct-hit. n80; n77; 10; 15; 50 (RBstart=0); 100; 13.8; NOTE 2; UL2/DL1 direct-hit. n80; n77; 5; 15; 25 (RBstart=0); 10; 1.1; NOTE 6; UL2/DL1 near-miss. n80; n78; 5; 15; 25 (RBstart=0); 10; 23.9; NOTE 2; UL2/DL1 direct-hit. n80; n78; 10; 15; 50 (RBstart=0); 100; 13.8; NOTE 2; UL2/DL1 direct-hit. n80; n78; 5; 15; 25 (RBstart=0); 10; 1.1; NOTE 6; UL2/DL1 near-miss. n81; n41; 5; 15; 16 (RBstart=4); 10; 13; NOTE 3; UL3/DL1 direct-hit. n81; n41; 5; 15; 25 (RBstart=0); 100; 3.5; NOTE 3; UL3/DL1 direct-hit. n81; n78; 5; 15; 16 (RBstart=4); 10; 10.8; NOTE 4; UL4/DL1 direct-hit. n81; n78; 5; 15; 25 (RBstart=0); 100; 1.4; NOTE 4; UL4/DL1 direct-hit. n81; n79; 5; 15; 25 (RBstart=0); 40; 6.8; NOTE 5; UL5/DL1 direct-hit. n81; n79; 5; 15; 25 (RBstart=0); 100; 4.4; NOTE 5; UL5/DL1 direct-hit. n82; n78; 5; 15; 16 (RBstart=4); 10; 10.8; NOTE 4; UL4/DL1 direct-hit. n82; n78; 5; 15; 20 (RBstart=2); 100; 1.0; NOTE 4; UL4/DL1 direct-hit. n83; n78; 5; 15; 10 (RBstart=8); 10; 10.4; NOTE 5; UL5/DL1 direct-hit. n83; n78; 5; 15; 25 (RBstart=0); 100; 0.7; NOTE 5; UL5/DL1 direct-hit. n84; n77; 5; 15; 25 (RBstart=0); 10; 23.9; NOTE 2; UL2/DL1 direct-hit. n84; n77; 10; 15; 100 (RBstart=0); 100; 13.8; NOTE 2; UL2/DL1 direct-hit. n84; n77; 5; 15; 25 (RBstart=0); 10; 1.1; NOTE 6; UL2/DL1 near-miss. n86; n78; 5; 15; 25 (RBstart=0); 10; 23.9; NOTE 2; UL2/DL1 direct-hit. n86; n78; 10; 15; 100 (RBstart=0); 100; 13.8; NOTE 2; UL2/DL1 direct-hit. n86; n78; 5; 15; 25 (RBstart=0); 10; 1.1; NOTE 6; UL2/DL1 near-miss. n97; n79; 5; 15; 100 (RBstart=0); 40; 29.4; NOTE 2; UL2/DL1 direct-hit. n97; n79; 5; 15; 270 (RBstart=0); 100; 25.3; NOTE 2; UL2/DL1 direct-hit. n99; n77; 5; 15; 25 (RBstart=0); 10; 23.9; NOTE 2; UL2/DL1 direct-hit. n99; n77; 10; 15; 100 (RBstart=0); 100; 13.8; NOTE 2; UL2/DL1 direct-hit. n99; n77; 5; 15; 25 (RBstart=0); 10; 1.1; NOTE 6; UL2/DL1 near-miss. NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 4: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively.. Proposal 3: It can be clarified in the spec that “for different combinations of (UL configurations, DL channel bandwidths), the sensitivity degradation exception due to harmonic interference / cross band isolation are allowed for band combinations. The limited test configurations are specified to verify MSD requirements” when MSD table simplification is implemented. |

Open issues summary

Sub-topic 2-1

Sub-topic description: Based on the approved WF R4-2206450 as below, companies provided their views on the open issues in this meeting. We’d like to discuss them and make some progress in this meeting.

o	Option 4: When considering MSD table improvement, for a given band combination:

-	two MSD test points for the case of direct-hit harmonic interference can be considered,

-	two MSD test points for cross-band isolation interference can be considered,

-	one MSD test point for the case of near-miss harmonic interference can be considered.

Note: For the case when 2 MSD test points can be considered at least, one test point is specified for the minimum victim downlink channel bandwidth.

Companies are encouraged to bring proposals on criteria

1)	 how to select the second MSD test point (is it based on maximum DL CBW? based on MSD level? other?); and,

2)	 how to capture the test point in the new table format in case new “lowest” or new “highest” CBW are introduced in the future.

3)	Other options are not precluded, including if 2 test points are needed for near miss harmonic MSD.

Open issues and candidate options before e-meeting:

Issue 2-1-1: If the victim band supports 100MHz channel bandwidth, how to select the second MSD test point for the direct hit harmonic and the cross-band isolation for FR1 in current release?

- Proposals

  - Option 1: The MSD requirement for the maximum victim downlink channel bandwidth of 100MHz.

  - Option 2: Others

- Recommended WF

  - Option 1

Issue 2-1-2: If the maximum supported channel BW of the victim band is smaller than 100MHz, how to select the second MSD test point for the direct hit harmonic and the cross-band isolation for FR1 in current release?

- Proposals

  - Option 1: The 2nd MSD requirement is based on the maximum victim downlink channel bandwidth supported in the latest spec.

  - Option 2: Whether the 2nd MSD requirement based on the maximum victim downlink channel bandwidth can be requested by operators if needed.

  - Option 3: Others

- Recommended WF

  - TBA

Issue 2-1-3: how to capture the test point in the new table format in case new “lowest” or new “highest” CBW are introduced in the future release?

- Proposals

  - Option 1: Additional test point(s) would be needed in case of minimum victim downlink channel bandwidth or maximum victim downlink channel bandwidth changed in future release.

  - Option 2: if larger victim downlink channel bandwidth is introduced in the future whether additional requirement is needed can be further discussed.

  - Option 3: Others.

- Recommended WF

  - TBA

Issue 2-1-4: Please discuss whether Proposal 1 and 2 in Tdoc R4-2209358 for SUL band combinations can be acceptable?

- Proposals

  - Option 1: Yes, it’s acceptable.

  - Option 2: No, it can be modified.

- Recommended WF

  - TBA

Sub-topic 2-2

Sub-topic description: Other proposals which are related to MSD improvement can be discussed in this sub-topic:

Issue 2-2-1: Please discuss whether Proposal 3 in Tdoc R4-2208454 can be acceptable?

Proposal 3: When considering transferring the current table to the new format, the value of the MSD and the uplink configurations should remain unchanged.

- Proposals

  - Option 1: Yes, it’s acceptable.

  - Option 2: No, it can be modified.

  - Option 3: Others.

- Recommended WF

  - TBA

Issue 2-2-2: Please discuss whether Proposal 3 in Tdoc R4-2209358 for SUL band combinations can be acceptable?

Proposal 3: It can be clarified in the spec that “for different combinations of (UL configurations, DL channel bandwidths), the sensitivity degradation exception due to harmonic interference / cross band isolation are allowed for band combinations. The limited test configurations are specified to verify MSD requirements” when MSD table simplification is implemented.

- Proposals

  - Option 1: Yes, it’s acceptable.

  - Option 2: No, it can be modified.

  - Option 3: Others.

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Sub topic 2-1

| Company | Comments |
| --- | --- |
| MediaTek | Issue 2-1-1: We are fine with recommended WF Issue 2-1-2: Option 1 Issue 2-1-3: Option 3. No need to add/modify existing test points if same impairment mechanism keeps. New test points to be added/modified only when worse mechanism happens (ex: aggressor IMD5 falling into victim BW aggressor IMD3 falling into victim BW) Issue 2-1-4: Option 1 |
| Huawei | Issue 2-1-1: We are fine with recommended WF Issue 2-1-2: Both Option 1 and option 2 are OK. Issue 2-1-3: We tend to agree with MTK’s view. It can be further discussed case by case. Issue 2-1-4: Option 1 |
| Meta | Issue 2-1-1: When 100MHz CBW is supported, then the 2nd MSD test point is up to operator request. Issue 2-1-2: Option 2 Issue 2-1-3: RAN4 will further discuss based on the operator request for the 2nd MSD test point |
| ZTE | Issue 2-1-1: Option 1 Issue 2-1-2: Option 2 Issue 2-1-3: Option 1 and option 2 are not conflict. For option 2, except larger victim downlink channel BW in introduce, smaller victim downlink channel BW should also be considered. The comments from MTK somehow imply new test point would be needed. At least, new test point would not be excluded if in case new “lowest” or new “highest” CBW are introduced in the future release? |
| CHTTL | Issue 2-1-1: Option 1 as the proponent. Issue 2-1-2: Option 1 and option 2 are OK, in some case there might be some on-going larger BW, so such operator might be able to provide better expectation on the larger BW status. Issue 2-1-3: Option 1 and option 2 are ok. Probably in this meeting we can only agree that further discuss is needed. |
| Skyworks | We apologise for missing the deadline for uploading our contribution. Issue 2-1-1: Option 2. In our view, the MSD test point for 100MHz or for the highest supported DL CBW is an optional test point: the choice of DL CBW should remain open to proponents to accommodate concerns such as regional spectrum range, network deployment considerations, etc... For MSD due to cross band isolation, the agreed guidelines R4-2202275 WF on LB-LB MSDs should be used to define the UL/DL configurations of the first MSD test point, denoted “TP#1”. It is key to specify MSD for the smallest DL CBW. As a reminder, these agreements are: Guidelines for UL band configuration: - the UL carrier is configured with the highest UL CBW. This ensures the lowest UL IMD order has maximum reach towards the affected DL band, this is key when the frequency separation is "small" relative to the UL CBW (eg. CA_n5-n28, CA_n1-n3..) - UL RB blocks shall be located as close as possible to the affected downlink band, - UL Lcrb is that specified in Table 7.3.2-3 - UL configuration for UL Band REFSENS for the corresponding UL CBW. This solves the numerous inconsistencies that are present in Table 7.3A.6.2 today. - The UL carrier centre-frequency shall be configured as close as possible to the affected DL band. Guidelines for DL affected band configuration: - Smallest DL CBW, a 2nd test point targeting the highest DL CBW is not precluded, - The affected DL carrier centre-frequency shall be configured as close as possible to the UL band. Issue 2-1-2: Option 2 Issue 2-1-3: Option 3. Same view as Mediatek and Huawei: - new “highest” CBW may impact MSD if this new CBW is that of the UL band. - new “lowest” CBW may impact MSD if this new CBW is that of the affected DL band. So, the impact should be evaluated on a case by case, test points may not always need to be changed. Issue 2-1-4: Option 2: for consistency, we would prefer to align the UL configuration of the first test point with TP#1 guidelines for MSD due to crossband isolation – see blue highlights. For CA_n97-n41, the first test point should become BCS4 compliant and therefore MSD should be re-evaluated for UL-CBW=100MHz, Lcrb=270 (RBstart=3). UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; X band interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n80; n41; 1760; 50; 15; 50 (RBstart=220); 2505; 10; 4.3; >ACLR2. n80; n41; 1780; 10; 15; 50 (RBstart=0); 2550; 100; 3.0; >ACLR2. n95; n41; 2017.5; 15; 15; 75 (RBstart=4); 2505; 10; 6.1; >ACLR2. n95; n41; 2017.5; 15; 15; 75 (RBstart=0); 2550; 100; 6.1; >ACLR2. n97; n41; 2360; 80; 30; 216 (RBstart=1); 2505; 10; 20.7; ACLR2. n97; n41; 2360; 80; 30; 216 (RBstart=0); 2550; 100; 10.6; ACLR2. |

Sub topic 2-2

| Company |  | Comments |
| --- | --- | --- |
| MediaTek |  | Issue 2-2-1: Option 1. The later spec format change shall not impact/change previous agreed specs values. Issue 2-2-2: Option 2. We don’t need to say “Limited test configurations”. We can say “only the listed test points are needed to be tested” |
| Huawei |  | Issue 2-2-1: OK with Option 1. But if special cases are identified, it can be further discussed case by case. Issue 2-2-2: OK with MTK’s revision. |
| Meta |  | Issue 2-2-1: Option 1 is our preference. But we can further discuss if necessary to change the configurations. Issue 2-2-2: Generally, we are fine with the proposal |
| ZTE |  | Issue 2-2-1: Option 1 is our preference. |
| CHTTL |  | Issue 2-2-1: Option 1 unless there exist errors. Issue 2-2-2: though it mentions that the issue is for SUL band combinations, but I guess it is for all the combinations if I understand the contribution correctly. We think such clarification is ok, regarding the wording “… allowed for band combinations” probably we can say “… allowed for the listed band combinations”. Maybe we don’t need to say only the listed test points are needed to be tested, since if the requirement is not listed, of course it cannot be tested? |
| Skyworks |  | Issue 2-2-1: Option 1 is fine with us for this meeting. Question to Huawei: Shall we migrate the test points for MSD due to cross-band isolation into the new table format? For MSD due to cross-band isolation: we are Ok with option 1 at this meeting. We have posteda draft CR for review based on off-line discussions (link: Draft CR for R17 38.101-1 new table format for MSD due to cross-band isolation). We would like to thank the proponents for their inputs, and we welcome further comments. For the next meeting, some MSDs need to be re-evaluated due to TP#1 UL/DL guidelines. We invite interested companies to review the list below and come back at next meeting with MSD proposal. We believe the majority of MSDs can be retrieved from the legacy levels because in some cases TP1 has little impact on Tx noise level falling in Rx DL band. For example, the band n41 IM order falling in band n48 5MHz lowest channel changes from order >> 17 to order=17 for TP#1 below. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n1; n3; 1945; 50; 15; 128 (RBstart=0); 1877.5; 5; FFS; ACLR1. n1; n40; 1955; 50; 15; 128 (RBstart=142); 2302.5; 5; FFS; >ACLR2. n3; n74; 1735; 50; 15; 50 (RBstart=0); 1515.5; 5; FFS; >ACLR2. n34; n3; 2017.5; 15; 15; 75 (RBstart=0); 1877.5; 5; FFS; >ACLR2. n38; n1; 2590; 40; 15; 216 (RBstart=0); 2167.5; 5; FFS; >ACLR2. n38; n25; 2590; 40; 15; 216 (RBstart=0); 1992.5; 5; FFS; >ACLR2. n38; n78; 2600; 40; 15; 216 (RBstart=0); 3305; 10; FFS; >ACLR2. n40; n1; 2350; 100; 30; 270 (RBstart=0); 2167.5; 5; FFS; ACLR2. n41; n3; 2546; 100; 30; 270 (RBstart=0); 1877.5; 5; FFS; >ACLR2. n41; n25; 2546; 100; 30; 270 (RBstart=0); 1992.5; 5; FFS; >ACLR2. n41; n66; 2546; 100; 30; 270 (RBstart=0); 2167.5; 5; FFS; >ACLR2. n41; n48; 2640; 100; 30; 270 (RBstart=3); 3552.5; 5; FFS; >ACLR2. n41; n70; 2546; 100; 30; 270 (RBstart=0); 2017.5; 5; FFS; >ACLR2. n41; n77; 2640; 100; 30; 270 (RBstart=3); 3305; 10; FFS; >ACLR2. n41; n78; 2640; 100; 30; 270 (RBstart=3); 3305; 10; FFS; >ACLR2. n77; n2; 3350; 100; 30; 270 (RBstart=0); 1932.5; 5; FFS; >ACLR2. Issue 2-2-2: Option 2 same view as Mediatek. |
| Skyworks | With the new table format, we simplify considerably the number of test points, but we have one vulnerability because the test points rely on the “lowest” DL CBW (first test point) and the “highest” DL CBW (2nd test point). We would like to suggest that for round 2, a WF is drafted to define guidelines so that proponents who will request new “lowest” or new “highest” CBW for a frequency band are aware of the impact on MSD tables, for example if MSD needs to be evaluated or not - Refer to Issue 2-1-3 and thread [119]. WF may also capture guidelines to proponents who will request BCS4 in case BCS4 means a new “highest” or new “lowest” CBW needs to be supported, and whether MSD needs re-evaluation. For the case of intra-band non-contiguous, the WF may also clarify guidelines on what is the maximum aggregated CBW or the minimum gap between CCs, and if MSD needs to be re-evaluated etc,. - refer to thread [116] BCS4 requests. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Issue 2-1-1: If the victim band supports 100MHz channel bandwidth, how to select the second MSD test point for the direct hit harmonic and the cross-band isolation for FR1 in current release? MTK, Huawei, ZTE, CHTTL are OK with option 1. Meta express that When 100MHz CBW is supported, then the 2nd MSD test point is up to operator request. Skyworks provide another option as below. The MSD test point for 100MHz or for the highest supported DL CBW is an optional test point: the choice of DL CBW should remain open to proponents to accommodate concerns such as regional spectrum range, network deployment considerations, etc. From moderator’s perspective, we can have a way forward to further discuss the specific principle in the 2nd round and address all the companies’ concerns. However, the principle shouldn’t have an impact on the conversion progress of MSD table format. Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss the specific principle based on WF in the 2nd round. Issue 2-1-2: If the maximum supported channel BW of the victim band is smaller than 100MHz, how to select the second MSD test point for the direct hit harmonic and the cross-band isolation for FR1 in current release? Option 1: MTK, Huawei, CHTTL Option 2: Huawei, Meta, ZTE, CHTTL, Skyworks. Based on the discussion, it seems that the views companies are not very diversity. It’s encouraged to reach a compromise in 2nd round. Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss the potential compromise based on WF in the 2nd round. Issue 2-1-3: how to capture the test point in the new table format in case new “lowest” or new “highest” CBW are introduced in the future release? MTK, Skyworks and Huawei prefer option 3: further discuss case by case. Meta express that RAN4 will further discuss based on the operator request for the 2nd MSD test point. ZTE and CHTTL are OK with both option 1 and option 2. From moderator perspective, if we can’t reach an agreement on this issue in this meeting, the baseline is to further discuss case by case. But let’s have a try to find a compromise in the 2nd round. Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss the potential compromise based on WF in the 2nd round. Issue 2-1-4: Please discuss whether Proposal 1 and 2 in Tdoc R4-2209358 for SUL band combinations can be acceptable? Option 1 can be acceptable with some modifications. Tentative agreements: Option 1 can be acceptable with some modifications. Candidate options: Recommendations for 2nd round: Option 1 can be acceptable with some modifications. |
| Sub-topic#2-2 | Issue 2-2-1: Please discuss whether Proposal 3 in Tdoc R4-2208454 can be acceptable? All the companies are OK with option 1 for this meeting. Tentative agreements: All the companies are OK with option 1 for this meeting. Candidate options: Recommendations for 2nd round: The draft CR with the conversion of MSD table format can be implemented based on this agreement. Issue 2-2-2: Please discuss whether Proposal 3 in Tdoc R4-2209358 for SUL band combinations can be acceptable? The intention of this proposal can be accepted, but the specific wording should be further improved. Tentative agreements: Candidate options: Recommendations for 2nd round: The draft CR with the conversion of MSD table format can be implemented based on this proposal. But the detailed wording can be further improved, |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #3: Discussion on CRs

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2209420 | Huawei, HiSilicon | Reason of change: It isn’t clarified that BCS4 and BCS5 can't be reported together in TS 38.101-3. Summary of change: The following clarification is added in clause 5.5A.1. “BCS5 shall not be indicated together with BCS4 for a CA configuration.” |
| R4-2209421 | Huawei, HiSilicon | Reason of change: Based on the current spec, some clarification specified in clause 5.5A.0 is missing in SUL configuration. Summary of change: The following clarification is added in clause 5.5C. “BCS5 shall not be indicated together with BCS4 for a SUL configuration. For SUL band combinations including FR1 intra-band CA and with BCS4 or BCS5, the Bandwidth Combination Sets for the FR1 intra-band CA are BCS4 or BCS5.” |

Companies views’ collection for 1st round

CRs/TPs comments collection

Companies can comment the CR directly.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2209420 | ZTE: we think the text ‘ and BCS5 shall not be indicated together with BCS4 for a CA configuration’ is not the same meaning of ‘ BCS4 and BCS5 can't be reported together’, as stated in the CR cover. In addition, we think there is no need to add the text in RAN4 spec, some rules were already captured in the TR38.862, which BCS is reported pending on UE capability. |
|  | T-Mobile USA: We support the CR. We think the wording is consistent with the wording in 38.101-1 5.5A.0: “BCS5 shall not be indicated together with BCS4 for a CA configuration.” |
|  | To ZTE, this CR has been endorsed in RAN4#101bis. It’s resubmission. If you want to reword the text based on the agreement, we can further discuss it. |
| R4-2209421 | ZTE: Same as above. |
|  | T-Mobile USA: We support the CR. We think the wording is consistent with the wording in 38.101-1 5.5A.0: “BCS5 shall not be indicated together with BCS4 for a CA configuration.” |
|  | To ZTE, If you want to reword the text based on the agreement, we can further discuss it. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Referring to the agreed CR R4-2206451 in last meeting, the same clarification is added for CA. Thus, the same clarification is needed for SUL and FR1+FR2 CA. After offline discussion with ZTE, moderator think both R4-2209420 and R4-2209421 can be agreed. Tentative agreements: Both R4-2209420 and R4-2209421 are agreeable. Candidate options: Recommendations for 2nd round: Both R4-2209420 and R4-2209421 are agreeable. No further discuss in the 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on … | YYY |  |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
|  | WF on criteria on Rel-17 enhanced MSD table format | Skyworks Solutions Inc. |  |
|  | Draft CR for R17 38.101-1 to introduce new table format for MSD due to cross-band isolation | Skyworks Solutions Inc. | During the post email discussion, basket rapporteurs (ZTE 2band CA_PC3, China Telecom 2band CA_PC2/PC1.5 ) can merge these draft CRs into basket big CRs respectively in order to solve the CR conflicts, if it’s agreeable. |
|  | Draft CR for 38.101-1 to introduce new table format for MSD due to harmonic interference and for SUL | Huawei, HiSilicon | During the post email discussion, basket rapporteurs (ZTE 2band CA_PC3, Huawei SUL, China Telecom 2band CA_PC2/PC1.5 ) can merge these draft CRs into basket big CRs respectively in order to solve the CR conflicts, if it’s agreeable. |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2208454 |  | Discussion on the reduced MSD configurations for harmonic and cross-band isolation for NR CA | CHTTL | Noted |  |
| R4-2208676 |  | Discussion on maximum aggregated channel bandwidth | Qualcomm Incorporated | Noted |  |
| R4-2208681 |  | Discussion on improvement on MSD for harmonic and cross-band isolation for NR CA | ZTE Corporation | Noted |  |
| R4-2208858 |  | Discussion on the max aggregated CBW of intra-band CA for BCS4/5 | Xiaomi | Noted |  |
| R4-2209358 |  | Discussion on simplifying extended MSD table for SUL | Huawei, HiSilicon | Noted |  |
| R4-2209420 |  | CR for 38.101-3 to clarify that BCS4 and BCS5 can't be reported together | Huawei, HiSilicon | Agreeable |  |
| R4-2209421 |  | CR for 38.101-1 to introduce the missing requirements for BCS4 | Huawei, HiSilicon | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Huawei | Peng (Henry) Zhang | zhangpeng169@Huawei.com |
| Skyworks Solutions, Inc. | Laurent Noel | laurent.noel@skyworksinc.com |

| MediaTek | Huanren Fu | huanren.fu@mediatek.com |
| --- | --- | --- |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)